3GPP TSG-RAN WG4 Meeting #96e	R4-2010610

Electronic Metting, 17 – 28 August 2020

Source:	Ericsson

Title:	Scenario Z NR PUSCH UL timing adjustment simulation results

Agenda item:	7.15.3.2.3

Document for:	Information

# Introduction

In this contribution, we presented our simulation results for scenario Z NR HST PUSCH UL timing adjustment.

# Simulation assumptions and simulation results

In this section, our presented our simulation assumptions based on [1] in Table 1 for UL timing adjustment. The corresponding ideal simulation results and results with impairments are given in Table 2 and Table 3 respectively.

| Parameters | Values |
| --- | --- |
|  | FR1 – Scenario Y |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| Number of Tx and Rx | 1Tx 2Rx |
| Number of DMRS symbols | (1+1+1) DM-RS symbols with l0 = 2 for PUSCH mapping type A, and l0 = 0 for PUSCH mapping type B |
| (A, Δω) | SCS = 15 kHz: (10 µs, 0.18 s-1) SCS = 30 kHz: (5 µs, 0.36 s-1) |
| System bandwidth | SCS = 15 kHz: 10MHz SCS = 30 kHz: 40 MHz |
| Frequency domain resource | SCS = 15 kHz: 25 PRBs for each UE SCS = 30 kHz: 50 PRBs for each UE |
| MCS index | 16 |
| SRS (optional) | SCS = 15 kHz: single region with length of 40 PRBs SCS = 30 kHz: single region with length of 80 PRBs |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for UL timing adjustment at 500 km/h, scenario Z [1].

Table 5. Ideal simulation results for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Z | 15 kHz / 10 MHz | A | 6 |
|  |  | B | 6 |
|  | 30 kHz / 40 MHz | A | 6 |
|  |  | B | 6 |

Table 6. Results with impairments for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ % of maximum throughput |
| --- | --- | --- | --- |
| Scenario Z | 15 kHz / 10 MHz | A | 8.2 |
|  |  | B | 8.4 |
|  | 30 kHz / 40 MHz | A | 8.5 |
|  |  | B | 8.5 |

# Conclusion

In this contribution, we summarized our 70% of maximum throughput simulation results for scenario Z NR UL timing adjustment.

# References

R4-2005536, WF on Rel-16 NR HST UL TA BS demodulation requirements, ZTE.