3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010716

# Electronic Meeting, 17 – 28 Aug., 2020

Source:	OPPO

Title:	On UE capability signaling for CSI-RS L3 measurement

Agenda Item:	7.14.1.5

Document for:	Discussion

Introduction

In last RAN 88e meeting, a Rel-16 Work Item Exception for CSI-RS based L3 measurement[1] was approved including UE capability requirement

- UE capability

  - New UE capability on the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell

  - New UE capability for minimum separation between two slots

    - Note: can be handled with low priority

In this contribution, we further discuss UE capability signaling for CSI-RS L3 measurement.

Discussion

In last RAN4#95e-bis meeting, a WF on CSI-RS based L3 measurement capability and requirements [2] was approved.

UE capability of simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell

  - FFS: Introduce new UE capability to indicate the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell

In last meeting, we also discussed whether to introduce the UE capability to indicate the simultaneous reception of CSI-RS of neighbor cell and SSB of serving cell. In our view, as the existing capability SimultaneousRxDataSSB-DiffNumerology in Rel-15 is for simultaneous reception of SSB of neighbor cell and data of serving cell, it definitely cannot be reused for CSI-RS L3 measurement.

# Table 1. UE feature list for Rel-15

| Index | Feature group | Components | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | simultaneousRxDataSSB-DiffNumerology | MeasAndMobParametersFRX-Diff MeasAndMobParametersMRDC-FRX-Diff | No | Yes | Optional with capability signalling |

However, since CSI-RS and data can share the same SCS due to single FFT, it is similar to introduce a capability for UE supporting different SCS in source and target cells for measurement, which is also under discussion for NR mobility enhancement in [3]. So we prefer a new UE capability defined for NR mobility enhancement can be reused here as well. Otherwise, introduce a dedicated new capability of simultaneous reception of CSI-RS of neighbor cell and SSB of serving cell for CSI-RS L3 measurement.

# Table 2. UE feature list for Rel-16 NR mobility enhancement

| 5. Mobility Enhancement | 5-5 | Support of different SCS-s in source and target cells for combination | Support of different SCS-s in source and target cells for intra-frequency case Support of different SCS-s in source and target cells for inter-frequency case | The network cannot configure the different SCS-s in source and target cells | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- |

Proposal 1: If a new capability for UE supporting different SCS in source and target cells is defined in Rel-16 NR mobility measurement, reuse it for CSI-RS L3 measurement.

Otherwise, introduce a dedicated new capability of simultaneous reception of CSI-RS of neighbor cell and SSB of serving cell for CSI-RS L3 measurement.

Besides, for new UE capability for minimum separation between two slots, it has been agreed to be handled with low priority. In our view, no more discussion is needed in Rel-16 due to limited time and diverse views through several meetings. It can be left to be discussed in Rel-17 if possible.

Proposal 2: Do not introduce UE capability for minimum separation between two slots in Rel-16.

Conclusion

In this contribution, we propose our views as follows.

Proposal 1: If a new capability for UE supporting different SCS in source and target cells is defined in Rel-16 NR mobility measurement, reuse it for CSI-RS L3 measurement.

Otherwise, introduce a dedicated new capability of simultaneous reception of CSI-RS of neighbor cell and SSB of serving cell for CSI-RS L3 measurement.

Proposal 2: Do not introduce UE capability for minimum separation between two slots in Rel-16.

References

- RP-201340, Rel-16 WI Exception for Core part: RRM requirement for CSI-RS based L3 measurement in NR, CATT

- R4-2005357, WF on CSI-RS based L3 measurement capability and requirements, OPPO