3GPP TSG-RAN WG4 #96-e	R4- 2010720

Electronic Meeting, August 17th – August 28th, 2020

Agenda item:	7.8.1.2.1

Source:	MediaTek Inc.

Title:	Discussion on eMBB UE performance requirement with pre-emption

Document for:	Discussion

- Introduction

In RAN4#95-e, simulation assumptions for eMBB UE pre-empted by URLLC UE were captured in the agreed way forward [1]. In this contribution, we discuss about performance requirement for eMBB UE with pre-emption.

- Simulation results

In Fig. 1 and Fig. 2, we provide the simulation results for MCS4 and MCS 13 based on the agreed simulation assumptions. Besides, Table 1 show the SNR values to achieve the 70% of maximum throughput.

Figure 1. PDSCH performance for pre-emption with MCS4

Figure 2. PDSCH performance for pre-emption with MCS13

Table 1. SNR values at 70% throughput for FDD

| MCS | Pre-emption probability | SNR @ 70% throughput [dB] |  |
| --- | --- | --- | --- |
|  |  | Without HARQ buffer Flushing | With HARQ buffer Flushing |
| 4 | 10% | -2.09 | -2.09 |
|  | 20% | -1.36 | -1.38 |
| 13 | 10% | 6.19 | 6.15 |
|  | 20% | 8.86 | 8.31 |

- Discussion

Based on our simulation results, there are small SNR loss at 70% throughput between the case of with HARQ buffer flushing and without HARQ buffer flushing in both MCS 4 and MCS 13, which are similar to the results provided in [2]. However, considering MCS 13 and pre-emption probability 20%, our simulation results are quite different from that shown in [3], which provides 2.6 dB gain with the case of HARQ buffer flushing compared with the case of without HARQ buffer flushing. Besides, for eMBB UE, when PDSCH is pre-empted and it cannot decode the PDSCH correctly, it will flush the HARQ buffer and wait for the re-transmission. Hence, we think it is difficult to achieve almost the same throughput as the case of no pre-emption even when considering MCS 4 and pre-emption probability 10% [3]. In conclusion, we propose to discuss more about the HARQ buffer flushing behavior of eMBB UE when pre-empted.

Observation 1: Small SNR loss at 70% throughput between the case without HARQ flushing and with HARQ flushing in both MCS 4 and MCS 13.

Proposal 1: Discuss more about the HARQ buffer flushing behavior of eMBB UE when pre-empted.

- Summary

The observations and proposals are summarized as below:

Observation 1: In FDD, small SNR loss at 70% throughput between the case without HARQ flushing and with HARQ flushing in both MCS 4 and MCS 13.

Proposal 1: Discuss more about the HARQ buffer flushing behavior of eMBB UE when pre-empted.

- Reference

- R4-2008807, “Way forward for NR UE URLLC performance requirements”, Intel

- R4-2007929, “Discussion on UE performance requirements for Pre-emption”, Ericsson

- R4-2006527, “Discussion on UE performance requirements for URLLC”, Intel