3GPP TSG-RAN4 Meeting #96-e	R4-2010721

E-meeting, 17 – 28 August, 2020

Source: 	Ligado Networks

Title: 	System Parameters for n24  and discussion related to regulatory emission limits for UE

Agenda item:	10.25.1

Document for: 	Discussion and Approval

# Introduction

In RAN#88e, a new work item (WI) to introduce a new NR band n24 was approved [1].  The following objectives related to UE aspects were specified:

- Specify a new NR FDD operating band, n24, to include

  - UE transmit frequency range: 1626.5 MHz – 1660.5 MHz, BS transmit frequency range: 1525 MHz – 1559 MHz.

  - ARFCN

  - Channel bandwidth of 5 MHz with supported subcarrier spacing of 15 kHz

  - Channel bandwidth of 10 MHz with supported subcarrier spacing of 15 kHz, 30 kHz and 60 kHz

  - Updated regulatory limits on unwanted emissions

- Specify relevant requirements to align with the modified FCC rules

  - Address implications of tighter out of band emission limits specified in the FCC Order recently published.

  - Specify two Tx/Rx spacing allowing use of either UL channel (1627.5 – 1637.5 MHz or 1646.5 – 1656.5 MHz) with the DL channel, 1526 – 1536 MHz.

  - Identify impact on requirements for the uplink power level for 1627.5 – 1632.5MHz, if needed, specify requirements.

This document proposes system parameters for n24 as well as addresses i) two Tx/Rx spacings, ii) assesses the regulatory aspects related to the uplink power ramp in the 1627.5 – 1632.5 MHz frequency range and iii) considerations for the new OOBE limits.

# Band 24 Review

Figure 1 depicts the current 3GPP Band 24 specification spanning 1525 – 1559 MHz on the DL and 1626.5 – 1660.5 MHz on the UL.  As per the regulatory update, DL transmission is only authorized in 1526 – 1536 MHz.

Figure 1: 3GPP Band 24

## UE Related Changes

As discussed in [3], there are two regulatory updats related to the Band 24 UE operations:

- First one is related to regulatory updates to the OOBE emission limits as listed below and shown in Figure 2.

- Second, for period ending Dec 16, 2020 ([3], [4]), the maximum EIRP for the lowest five megahertz of this spectrum—that is, 1627.5-1632.5 MHz—will ramp up from -31 dBW at 1627.5 MHz to -7 dBW at 1632.5 MHz before becoming subject to the -7 dBW limit for the entire segment.

Additionally, [3] also discussed the need to specify an additional Tx-Rx frequency spacing to allow flexibility of pairing the 10 MHz DL channel with either the lower or upper 10 MHz UL channels.

Updated UE Emission Limits from [2]:

- The maximum EIRP limit for UL channels: -7 dBW

- The wideband OOBE emissions limits:

  - -105 dBW/MHz or less between 1541 – 1608 MHz

  - A linear ramp from -105 dBW/MHz to -100 dBW/MHz between 1608 – 1610 MHz

  - A linear ramp from -100 dBW/MHz at 1610 MHz to -67 dBW/4kHz at 1627.5 MHz

  - -58 dBW/4 kHz at a 1 MHz offset from edge of 1646.5-1656.5 MHz band

  - -67 dBW/4kHz at 1627.5 MHz

- The narrowband emission limits:

  - A linear ramp from -44 dBW/700 Hz at 1625 MHz to – 110 dBW/700 Hz at 1610 MHz between 1625 – 1610 MHz

  - A linear ramp from -110 dBW/700 Hz at 1610 MHz to -115 dBW/700 Hz at 1608 MHz between 1610 – 1608 MHz

  - -115 dBW/700 Hz between 1608-1559 MHz

  - -132 dBW/2 kHz between 1559 – 1541 MHz

Figure 2 compare the new DL and UL emission limits specified in [2] to the old limits.

Figure 2: Old and New wideband emission limits for Band 24 User Equipment

## Proposed System Parameters

The objective of the work item is to specify a new NR FDD operating band n24 to include the core requirements with supported subcarrier spacing of 15 kHz for 5 MHz channel bandwidth, and subcarrier spacing of 15 kHz, 30 kHz and 60 kHz for 10 MHz channel bandwidth. Following are the proposed systems parameters for n24:

- operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- | --- |
| n24 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |

- channel bandwidth

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n24 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |

- Channel raster

| NR Operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |

- Synchronization raster

| NR Operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n24 | 15 kHz | Case A | 3818 – <1> – 3892 |
| n24 | 30 kHz | Case B | 3824 – <1> – 3886 |

The 30 kHz synchronization raster is specified to allow DSS configuration without the use of MBSFN based blanking for LTE.

Proposal 1: Approve the proposed system parameters for operating band, channel bandwidth, NR-ARFCN ranges and the synchronization raster ranges.

## Dual Tx-Rx Spacing

One concern for two duplex spacing is the potential of self interference if the Tx/Rx spacing is small.  Given that the new Tx/Rx spacing is going to be larger than the existing one, no self interference is expected for the -120.5 MHz spacing.  To confirm this, a measurement of PA noise was made using a commercially available Band 4 PA operating at the lower edge (fc = 1715 MHz).  Band 4 UL is close to the  Band 24 UL and has a large duplex spacing of 180 MHz.  With 23dBm output power (after accounting for post-PA losses), the Band 4 PA noise density reaches its floor at and is flat within the receiver band. Therefore, the self-interference for the -120.5 MHz is going to be comparable to that of -101.5 MHz spacing for Band 24.

Proposal 2: Agree to two Tx-Rx spacings of -101.5 and -120.5 MHz.

## Treatment for transmit power ramp-down from 1632.5 – 1627.5 MHz

Besides the changes to the power and emission limits enumerated above, the FCC’s Order and Authorization describes a linear ramp in maximum EIRP for the 1627.5-1632.5 MHz sub-band from -7 dBW at 1632.5 MHz  to -31 dBW at 1627.5 MHz “for a period of five years.”  This condition also states that these limits apply “[c]onsistent with Ligado’s agreements with the GPS companies.”  The relevant GPS company agreement—Ligado’s December 16, 2015 settlement agreement with Garmin—expressly states that this linear ramp shall run “for five years from the Effective Date,” which places the end of the ramp-down period on December 16, 2020.

The band modification is expected to complete in March, 2021 and commercial availability of UEs is expected shortly thereafter.  Thus, specifications for the ramp-down will never be required to be implemented in the deployed network.

Proposal 3: Given the completion timeline of the WI, agree that further evaluation and specification development related to linear ramp between 1627.5 – 1632.5 MHz is unnecessary and not required.

## Addressing Regulatory Updates related to OOB Emissions

The FCC has tightened the OOBE mask for Band 24’s uplinks below 1627.5 MHz, relative to its previous requirements, shown in Figure 2 above.  This creates challenges for both UE power amplifier (PA) linearity and post-PA filtering, such as provided by a duplexer.  For such stringent emission requirements, the evaluation should be based on measured data of PAs with high 1 dB compression point and filters with high selectivity and not generic component models. Ligado is currently surveying such PAs and duplexers to determine the feasibility of meeting the new requirements. We encourage 3GPP members to present filter and PA measurements for such components at the next meeting to be used in the evaluation.

Proposal 4: Companies are encouraged to bring measurements for PAs with high 1 dB compression point and duplexers with high selectivity to develop input parameters for evaluating additional requirements on UE maximum output power to RAN4#97e.

# Proposal

Proposal 1: Approve the proposed system parameters for operating band, channel bandwidth, NR-ARFCN ranges and the synchronization raster ranges.

Proposal 2: Agree to two Tx-Rx spacings of -101.5 and -120.5 MHz.

Proposal 3: Given the completion timeline of the WI, agree that further evaluation and specification development related to linear ramp between 1627.5 – 1632.5 MHz is unnecessary and not required.

Proposal 4: Companies are encouraged to bring measurements for PAs with high 1 dB compression point and duplexers with high selectivity to develop input parameters for evaluating additional requirements on UE maximum output power to RAN4#97e.

# References

[1] RP-201356, New WID proposal for Introduction of NR band n24

[2] FCC Order DA 20-48A1 https://ecfsapi.fcc.gov/file/04222926711874/FCC-20-48A1.pdf

[3] R4-2006090, Regulatory updates for 3GPP Band 24