3GPP TSG-RAN WG4 Meeting # 96-e 															R4-2010931

Electronic Meeting, 17 August – 28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	General discussion about Rel-17 band combinations for Uu and V2X con-current operation

Agenda Item:	10.19.1

Document for:	Approval

Introduction

In RAN#88e meeting, a new WI [1] was approved that Band combinations for Uu and V2X con-current operation will be introduced in Rel-17. The main scenarios with max two simultaneous transmission bands within FR1 are listed below.

- Concurrent operation between NR Uu band and NR PC5 band.

- Concurrent operation between LTE Uu band and NR PC5 band.

- Concurrent operation between NR Uu band and LTE PC5 band.

In this paper, we’d like to generally discuss this topic.

Discussion

Band combinations with band n47/47

It can be foreseen that the ITS frequency band is very important when deriving the RF requirements for band combinations with Uu and V2X con-current operation. Thus, it’s necessary to consider the band n47/47 filter performance. Based on the investigation, the the band n47/47 filter performance are shown below.

Table 2.1-1 Filter performance for band n47/47

| Filter | IL [dB] |  | Min Attenuation [dB] @ |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Nominal | Worst Case | 693-2690 MHz | 3300-4200 MHz | 4400-5000 MHz |
| n47/47 (5850-5925 MHz) | 1.2 | 1.5 | > 37 | > 32 | > 30 |

When RAN4 specify the REFSENS requirements for specific band combinations, these parameters can be considered as a reference.

Referring to TR 36.787 [2], a separated antenna architecture as example are shown in Figure 2.1-1. Generally, RF model of band n47/47 is independent of Uu RF model. At least, 10 dB antenna isolation can be achieved for separated antenna architecture. However, as it goes on, higher frequency band may be used in licensed band. Thus, it can’t be excluded that a shared antenna architecture between licensed band and ITS band is implemented in one RF model as shown in Figure 2.1-2. ΔTIB,c and ΔRIB,c for band n47/47 need to be further evaluated. At least, we can reuse the value of band n79 (ΔTIB,c = 0.8, ΔRIB,c = 0.5)

Proposal 1: Filter performance for band n47/47 in Table 2.1-1 can be used to evaluate the MSD exception for V2X band combinations.

Proposal 2: RAN4 need to evaluate ΔTIB,c and ΔRIB,c for band n47/47 and UE architecture.

Figure 2.1-1: Example RF architecture for V2X_20A-47A con-current operation with harmonic trap filter

Figure 2.1-2: Example RF architecture with shared antenna with licensed band

Referring to TR 36.787 [2], the harmonics/IMD impact between licensed band and ITS frequency band are shown below.

Table 2.1-2: Harmonics /IMDs impacts on own Rx bands

| Licensed Band |  |  | B47 Range (MHz) | Comments for harmonics/IMD problems |
| --- | --- | --- | --- | --- |
|  |  |  | 5855 – 5925 |  |
| E-UTRA band | UL Range (MHz) | Harm. Order | Harmonic Range (MHz) |  |
| B3 | 1710 – 1785 | 3x | 5130 – 5355 | No Harmonics/IMDs |
| B7 | 2500 - 2570 | 2x | 5000 - 5140 | No Harmonics/IMDs |
| B8 | 880 -915 | 6x | 5280 – 5490 | No Harmonics/IMDs |
| B39 | 1880 - 1920 | 3x | 5640 – 5760 | No Harmonics/IMDs |
| B41 | 2496 - 2690 | 2x | 4992 – 5380 | No Harmonics/IMDs |
| B1 | 1920 - 1980 | 3x | 5760 – 5940 | 1) Harmonic into B47 2) 5th IMD into B47 |
| B5 | 824 - 849 | 7x | 5768 – 5943 | 1) Harmonic into B47, No IMD problems |
| B19 | 830 - 845 | 7x | 5810 – 5915 | 1) Harmonic into B47 No IMD problems |
| B20 | 832 - 862 | 7x | 5824 – 6034 | 1) Harmonic into B47, No IMD problems |
| B21 | 1448 - 1463 | 4x | 5792 – 5852 | 1) Side-lobe into B47 2) 4th IMD into B21 |
| B26 | 814 - 849 | 7x | 5698 – 5943 | 1) Harmonic into B47 No IMD problems |
| B28 | 703 - 748 | 8x | 5624 – 5984 | 1) Harmonic into B47 No IMD problems |
| B31 | 452.5 – 457.5 | 13x | 5882.5 – 5947.5 | 1) Harmonic into B47 but 13th harmonic impact can ignore. No IMD problems |
| B34 | 2010 - 2025 | 3x | 6030 - 6075 | No Harmonics/IMDs |
| B65 | 1920 - 2010 | 3x | 5760 – 6030 | 1) Harmonic into B47 2) 5th IMD into B47 and B65 |
| B71 | 663 - 698 | 8x | 5304 - 5584 | No Harmonics/IMDs |
| The other Licensed bands | - | Xx | - | No Harmonic impact. But need to IMD analysis into Band 47 and own licensed operating bands |

Based on the analysis Table 5.1-1, we summary the coexistence problem to own V2X Rx bands as below

-	Issue 1: Harmonics problem into Band 47

-	Issue 2: IMD problem into Band 47  Not occurred issues

-	Issue 3: IMD problem into licensed band (Band 21, Band 65)

For issue 2, there is no need to consider IMD problem which may fall into band 47 Rx band due to the half-duplex mode. For the harmonics problem, a summary which capture all the possible licensed NR band are shown in Table 2.1-3. A few bands may have a harmonic issues or harmonic mixing issues. IMD problem which may fall into licensed band can be solved one by one.

Table 2.1-3: NR Bands with harmonic interference or affected by harmonic mixing

| harmonic interference | 5855 | 5925 | Bands with harmonic interference | Bands affected by harmonic mixing |
| --- | --- | --- | --- | --- |
| 2nd harmonic | 2927.5 | 2962.5 | No licensed band | No licensed band |
| 3rd harmonic | 1951.667 | 1975 | n1 n65 | n2 n25 |
| 4th harmonic | 1463.75 | 1481.25 | n50 n74 | n50 n74 |
| 5th harmonic | 1171 | 1185 | No licensed band | No licensed band |
| 6th harmonic | 975.8333 | 987.5 | No licensed band | No licensed band |
| 7th harmonic | 836.4286 | 846.4286 | n5 n20 n26 | No licensed band |

Besides, cross band isolation problem between C-bands and ITS band may need to be further studied.

Observation 1: There is no need to consider IMD problem which may fall into band 47 Rx band due to the half-duplex mode.

Observation 2: Harmonic interference, harmonic mixing or IMD problem which may fall into licensed band can be solved one by one.

Observation 3: Cross band isolation problem between C-bands and ITS band may need to be further studied.

Band combinations with licensed band used for NR V2X

Firstly, RAN4 need to study whether the original RF chain or antenna of NR Uu licensed band can be reused for NR V2X. If the original RF chain of NR Uu licensed band can be reused, the corresponding ΔTIB,c and ΔRIB,c need to be specified. We can analysis REFSENS requirements or exception based on shared RF chain or antenna architecture. Or a separated antenna architecture should be considered.

For some cases, we may multiplex the Uu’s requirements or exception in the condition that the RF requirements are same between Uu and PC5 for the same licensed band. Or we need to identify the difference between Uu and PC5 for the same licensed band, such as bandwidth, spurious emission for UE co-existence and REFSENS.

Observation 4: When we specify the band combinations with licensed band used for NR V2X, we can study the UE RF architecture firstly. And we can only multiplex the Uu’s requirements or exception in the condition that the RF requirements are same between Uu and PC5 for the same licensed band.

The affected clauses for NR V2X band combinations with con-current operation

For the concurrent operation between NR Uu band and NR PC5 band, the affected clauses in the TS 38.101-1 are listed below.

Table 2.3-1: The affected clauses in TS 38.101-1

| 5.2E.2 | V2X operating bands for concurrent operation | Operating bands |
| --- | --- | --- |
| 5.3E.2 | Channel bandwidth for V2X concurrent operation | Bandwidth combination set |
| 6.2E.4.2 | Configured transmitted power for V2X con-current operation | ΔTIB,c |
| 6.5E.3.3 | Spurious emissions for UE co-existence for V2X con-current operation | Spurious emissions table |
| 7.3E.3 | Reference sensitivity power level for V2X con-current operation | REFSENS, MSD exception and ΔRIB,c |

For the concurrent operation between LTE Uu band and NR PC5 band / NR Uu band and LTE PC5 band, the affected clauses in the TS 38.101-3 are listed below.

Table 2.3-2: The affected clauses in TS 38.101-3

| 5.2C.2 | Inter-band V2X bands | Operating bands |
| --- | --- | --- |
| 5.5E.4.1 | Inter-band V2X configurations within FR1 (two bands) | Configurations |
| 6.2C.4.2 | UE configured output power for Inter-band V2X | ΔTIB,c |
| 6.5C.3.2.2 | Spurious emission band UE co-existence | Spurious emissions table |
| 7.3C.2.3 | Inter-band V2X con-current operation | REFSENS, MSD exception and ΔRIB,c |

Summary

Based on the discussion, all the proposals are listed below:

Proposal 1: Filter performance for band n47/47 in Table 2.1-1 can be used to evaluate the MSD exception for V2X band combinations.

Proposal 2: RAN4 need to evaluate ΔTIB,c and ΔRIB,c for band n47/47 and UE architecture.

Observation 1: There is no need to consider IMD problem which may fall into band 47 Rx band due to the half-duplex mode.

Observation 2: Harmonic interference, harmonic mixing or IMD problem which may fall into licensed band can be solved one by one.

Observation 3: Cross band isolation problem between C-bands and ITS band may need to be further studied.

Observation 4: When we specify the band combinations with licensed band used for NR V2X, we can study the UE RF architecture firstly. And we can only multiplex the Uu’s requirements or exception in the condition that the RF requirements are same between Uu and PC5 for the same licensed band.

- References

- RP-200924, New basket WID: Band combinations for Uu and V2X con-current operation, CATT

- TR 36.787, Vehicle-to-Everything (V2X) new band combinations