3GPP TSG-RAN WG4 Meeting #96-e	R4-2010962

# E-meeting, 25th May - 6th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4908 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Band n46 in 36.104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Band n46 in 36.104 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Band n46 in 36.104 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band n46 is not protected by other E-UTRA bands in the same geographical area |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4.3, 6.6.4.4, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

6	Transmitter characteristics

6.1	General

Unless otherwise stated, the requirements in clause 6 are expressed for a single transmitter antenna connector. In case of multi-carrier transmission with one or multiple transmitter antenna connectors, transmit diversity or MIMO transmission, the requirements apply for each transmitter antenna connector.

Unless otherwise stated, the transmitter characteristics are specified at the BS antenna connector (test port A) with a full complement of transceivers for the configuration in normal operating conditions. If any external apparatus such as a TX amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B).

Unless otherwise stated the requirements in clause 6 applies at all times, i.e. during the Transmitter ON period, the Transmitter OFF period and the Transmitter transient period.

Unless otherwise stated the requirements for NB-IoT in clause 6 applies for all operation modes (In-band operation, Guard-band operation and Stand-alone operation).

Figure 6.1-1: Transmitter test ports

6.6	Unwanted emissions

Unwanted emissions consist of out-of-band emissions and spurious emissions [2]. Out of band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The out-of-band emissions requirement for the BS transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and Operating band unwanted emissions. The Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges 10 MHz above and 10 MHz below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

For a BS supporting multi-carrier or intra-band contiguous CA, the unwanted emissions requirements apply to channel bandwidths of the outermost carrier larger than or equal to 5 MHz.

There is in addition a requirement for occupied bandwidth.

6.6.4	Transmitter spurious emissions

The transmitter spurious emission limits apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band. For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, this exclusion applies for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the single-band requirements apply and the multi-band exclusions and provisions are not applicable. Exceptions are the requirements in Table 6.6.4.3.1-2, Table 6.6.4.3.1-3, and specifically stated exceptions in Table 6.6.4.3.1-1 that apply also closer than 10 MHz from the downlink operating band and Table 6.6.4.3.1-1a that applies inside the downlink operating band. For some operating bands the upper frequency limit is higher than 12.75 GHz.

The requirements shall apply to BS that supports E-UTRA or  E-UTRA with NB-IoT in-band/guard band operation or NB-IoT standalone operation.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification. Unless otherwise stated, all requirements are measured as mean power (RMS).

6.6.4.3	Additional spurious emissions requirements

These requirements may be applied for the protection of system operating in frequency ranges other than the E-UTRA BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the E-UTRA BS, or they may be set by local or regional regulation as a mandatory requirement for an E-UTRA operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.3.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.) as listed below.

6.6.4.3.1	Minimum Requirement

The power of any spurious emission shall not exceed the limits of Table 6.6.4.3.1-1 for a BS where requirements for co-existence with the system listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.6.4.3.1-1 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.6.4.3.1-1 apply for the operating band supported at that antenna connector.

Table 6.6.4.3.1-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 8 |
|  | 876 - 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.4.2. |
| DCS1800 | 1805 - 1880 MHz | -47 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 3. |
|  | 1710 - 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 3, since it is already covered by the requirement in clause 6.6.4.2. |
| PCS1900 | 1930 - 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 2, band 25, band 36 or band 70. |
|  | 1850 - 1910 MHz | -61 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 2 or 25, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in band 35. |
| GSM850 or CDMA850 | 869 - 894 MHz | -57 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -61 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 1 or 65 |
|  | 1920 - 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70. |
|  | 1850 - 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 2 or 25, since it is already covered by the requirement in clause 6.6.4.2 |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1805 - 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 3 or 9. |
|  | 1710 - 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 3, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.6.4.2. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66 |
|  | 1710 - 1755 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 869 - 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 860 - 890 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 6, 18, 19. |
|  | 815 - 830 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 18, since it is already covered by the requirement in clause 6.6.4.2. |
|  | 830 - 845 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 6, 19, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2620 - 2690 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 7. |
|  | 2500 - 2570 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 7, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 - 960 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 8. |
|  | 880 - 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 3 or 9. |
|  | 1749.9 - 1784.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 3 or 9, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66 |
|  | 1710 - 1770 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 10 or 66, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in clause 6.6.4.2. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 11, 21, 32, 50, 74 or 75. |
|  | 1427.9 - 1447.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 11 or 74, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to BS operating in band 32, 50, 51, 75 or 76. |
|  | 1447.9 - 1462.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 21 or 74, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to BS operating in band 32, 50 or 75. |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12 or 85. |
|  | 699 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12 or 85, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6). |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 13. |
|  | 777 - 787 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 13, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 758 - 768 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 14. |
|  | 788 - 798 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 14, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 17 | 734 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 17. |
|  | 704 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 17, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6). |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 791 - 821 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20 or 28. |
|  | 832 - 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 22, 42, 48 or 49. |
|  | 3410 – 3490 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 22, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in Band 42 |
| E-UTRA Band 24 | 1525 – 1559 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 24. |
|  | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 24, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1930 – 1995 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70. |
|  | 1850 – 1915 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 25, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.6.4.2 |
| TRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 859 – 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |
|  | 814 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 26, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.4.2. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| E-UTRA Band 27 | 852 – 869 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 5, 26 or 27. |
|  | 807 – 824 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 27, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in clause 6.6.4.2. This requirement also applies to E-UTRA BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (Note 5). |
| E-UTRA Band 28 or NR Band n28 | 758 - 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20, 28, 44, 67 or 68. |
|  | 703 - 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 28, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in Band 44. For E-UTRA BS operating in Band 67, it applies for 703 MHz to 736 MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |
| E-UTRA Band 29 or NR Band n29 | 717 – 728 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 29 or 85. |
| E-UTRA Band 30 or NR Band n30 | 2350 – 2360 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 30 or 40. |
|  | 2305 – 2315 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 30, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in Band 40. |
| E-UTRA Band 31 | 462.5 -467.5 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31, 72 or 73. |
|  | 452.5 -457.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in band 72 or 73. |
| UTRA FDD band XXXII or E-UTRA band 32 | 1452 – 1496 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 11, 21, 32, 50, 74 or 75. |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 33. |
| UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 34. |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 - 1910 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 35. |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 2 and 36. |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 - 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880 - 1920MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 39. |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 - 2400MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 30 or 40. |
| E-UTRA Band 41 or NR Band n41 | 2496 - 2690 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 41 or 53. |
| E-UTRA Band 42 | 3400 - 3600 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48, 49 or 52. |
| E-UTRA Band 43 | 3600 - 3800 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 42, 43, 48 or 49. |
| E-UTRA Band 44 | 703 - 803 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 28 or 44. |
| E-UTRA Band 45 | 1447 - 1467 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 45. |
| E-UTRA Band 46 or NR Band n46 | 5150 - 5925 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 46. |
| E-UTRA Band 47 | 5855 - 5925 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 48 or NR Band n48 | 3550 - 3700 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 49. |
| E-UTRA Band 49 | 3550 - 3700 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 49. |
| E-UTRA Band 50 | 1432 - 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |
| E-UTRA Band 51 or NR Band n51 | 1427 - 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 50, 51, 75 or 76. |
| E-UTRA Band 52 | 3300 - 3400 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 42 or 52. |
| E-UTRA Band 53 or NR Band n53 | 2483.5 - 2495 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 41 or 53. |
| E-UTRA Band 65 or NR Band n65 | 2110 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 1 or 65, |
|  | 1920 - 2010 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 65, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 66 or NR Band n66 | 2110 - 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 4, 10, 23 or 66. |
|  | 1710 - 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 66, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.6.4.2. For E-UTRA BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 67 | 738 – 758 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 28 or 67. |
| E-UTRA Band 68 | 753 -783 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 28, or 68. |
|  | 698-728 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 68, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 69 | 2570 - 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |
| E-UTRA Band 70 or NR Band n70 | 1995 - 2020 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70 |
|  | 1695 – 1710 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 70, since it is already covered by the requirement in clause 6.6.4.2 |
| E-UTRA Band 71 or NR Band n71 | 617 - 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 71 |
|  | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 71, since it is already covered by the requirement in clause 6.6.4.2 |
| E-UTRA Band 72 | 461 - 466 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31, 72 or 73. |
|  | 451 - 456 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 72, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in band 73. |
| E-UTRA Band 73 | 460 - 465 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 31, 72 or 73. |
|  | 450 - 455 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 73, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 74 | 1475 – 1518 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 11, 21, 32, 50, 74 or 75. |
|  | 1427 – 1470 MHz | -49 dBm | 1MHz | This requirement does not apply to E-UTRA BS operating in Band 74, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to BS operating in band 32, 45, 50, 51, 75 or 76. |
| E-UTRA Band 75 or NR Band n75 | 1432 - 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |
| E-UTRA Band 76 or NR Band n76 | 1427 - 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 50, 51, 75 or 76. |
| NR Band n77 | 3.3 – 4.2 GHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48, 49 or 52. |
| NR Band n78 | 3.3 – 3.8 GHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48, 49 or 52. |
| NR Band n79 | 4.4 – 5.0 GHz | -52 dBm | 1 MHz |  |
| NR Band n80 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 3, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.6.4.2. |
| NR Band n81 | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.4.2. |
| NR Band n82 | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20, since it is already covered by the requirement in clause 6.6.4.2. |
| NR Band n83 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 28, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in Band 44. For E-UTRA BS operating in Band 67, it applies for 703 MHz to 736 MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |
| NR Band n84 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 85 | 728 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12, 29 or 85. |
|  | 698 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 85, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6). |
| NR Band n86 | 1710 - 1780 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 66, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.6.4.2. For E-UTRA BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 87 | 420 - 425 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 87 or 88. |
|  | 410 – 415 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 87, since it is already covered by the requirement in clause 6.6.4.2 |
| E-UTRA Band 88 | 422 - 427 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 87 or 88. |
|  | 412 - 417 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 88, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in band 87. |
| NR Band n89 | 824 - 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| NR Band n91 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 50, 51, 75 or 76. |
|  | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20, since it is already covered by the requirement in clause 6.6.4.2. |
| NR Band n92 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |
|  | 832 – 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 20, since it is already covered by the requirement in clause 6.6.4.2. |
| NR Band n93 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 50, 51, 75 or 76. |
|  | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.4.2. |
| NR Band n94 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |
|  | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in clause 6.6.4.2. |
| NR Band n95 | 2010 - 2025 MHz | -52 dBm | 1 MHz |  |
| NOTE 4: Void |  |  |  |  |

Additional co-existence requirements in Table 6.6.4.3.1-1a may apply for some regions.

Table 6.6.4.3.1-1a: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in Band 46

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 46a | 5150 - 5250 MHz | -40 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46c or 46d. |
| E-UTRA Band 46b | 5250 - 5350 MHz | -40 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46c or 46d. |
| E-UTRA Band 46c | 5470 - 5725 MHz | -40 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46a or 46b. |
| E-UTRA Band 46d | 5725 - 5925 MHz | -40 dBm | 1 MHz | This is only applicable to E-UTRA BS operating in Band 46a or 46b. |
| NOTE 1: This requirement may apply to E-UTRA BS operating in certain regions. |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in Table 6.6.4.3.1-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see Table 5.5-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.4.3.1-1 assumes that two operating bands, where the frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 5:	For E-UTRA Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for E-UTRA BS for co-existence with E-UTRA Band 27 UL operating band.

NOTE 6:	For E-UTRA Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for E-UTRA BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band.

The power of any spurious emission shall not exceed the limits of Table 6.6.4.3.1-1A for a Home BS where requirements for co-existence with a Home BS type listed in the first column apply.

Table 6.6.4.3.1-1A: Home BS Spurious emissions limits for co-existence with Home BS operating in other frequency bands

| Type of coexistence BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band I or E-UTRA Band 1 | 1920 - 1980 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band II or E-UTRA Band 2 | 1850 - 1910 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 2 or 25, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band III or E-UTRA Band 3 | 1710 - 1785 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 3, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.6.4.2. |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 4, 10 or 66, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band V or E-UTRA Band 5 | 824 - 849 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 815 - 830 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 18, since it is already covered by the requirement in clause 6.6.4.2. |
|  | 830 - 845 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 6, 19, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band VII or E-UTRA Band 7 | 2500 - 2570 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 7, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band VIII or E-UTRA Band 8 | 880 - 915 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 8, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 3 or 9, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 10 or 66, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in clause 6.6.4.2. |
| UTRA FDD Band XI, XXI or E-UTRA Band 11, 21 | 1427.9 - 1447.9 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 11 or 74, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to Home BS operating in band 32, 50, 51, 75 or 76. |
|  | 1447.9 - 1462.9 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 21 or 74, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to Home BS operating in band 32, 50 or 75. |
| UTRA FDD Band XII or E-UTRA Band 12 | 699 - 716 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 12 or 85, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 5) |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 13, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 14, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 17 | 704 - 716 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 17, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 5) |
| UTRA FDD Band XX or E-UTRA Band 20 | 832 - 862 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 20, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 - 3490 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 22, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to Home BS operating in Band 42 |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 24, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1850 - 1915 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 25, since it is already covered by the requirement in clause 6.6.4.2 |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 26, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.4.2. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| E-UTRA Band 27 | 807 - 824 MHz | -71 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in Band 27, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in clause 6.6.4.2. This requirement also applies to E-UTRA BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (Note 4). |
| E-UTRA Band 28 | 703 – 748 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 28, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to Home BS operating in Band 44. For E-UTRA BS operating in Band 67, it applies for 703 MHz to 736 MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |
| E-UTRA Band 30 | 2305 – 2315 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 30, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to Home BS operating in Band 40. |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 38. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 39 |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 - 2400MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 30 or 40 |
| E-UTRA Band 41 | 2496 – 2690 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 41 |
| E-UTRA Band 42 | 3400 - 3600 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 22, 42, 43, 48 or 52. |
| E-UTRA Band 43 | 3600 - 3800 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 42, 43 or 48. |
| E-UTRA Band 44 | 703 - 803 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 28 or 44 |
| E-UTRA Band 48 | 3550 - 3700 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 22, 42, 43 or 48. |
| E-UTRA Band 50 | 1432 - 1517 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 11, 21, 32, 50, 51, 74, 75 or 76. |
| E-UTRA Band 51 | 1427 - 1432 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 50, 51, 75 or 76. |
| E-UTRA Band 52 | 3300 - 3400 MHz | -71 dBm | 100 kHz | This is not applicable to Home BS operating in Band 42 or 52. |
| E-UTRA Band 65 | 1920 - 2010 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 65, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 66 | 1710 - 1780 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 66, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.6.4.2. For Home BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 68 | 698-728 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 68, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.4.2. |
| E-UTRA Band 70 | 1695 - 1710 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 70, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 71 | 663 – 698 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 71, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 74 | 1427 – 1470 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in Band 74, since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to BS operating in band 32, 50, 51, 75 or 76. |
| E-UTRA Band 85 | 698 - 716 MHz | -71 dBm | 100 kHz | This requirement does not apply to Home BS operating in band 85, since it is already covered by the requirement in clause 6.6.4.2. For Home BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 5). |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band 27, Band 28 or Band 29, the coexistence requirements in Table 6.6.4.3.1-1A do not apply for the 10 MHz frequency range immediately outside the Home BS transmit frequency range of a downlink operating band (see Table 5.5-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.4.3.1-1A assumes that two operating bands, where the frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	For E-UTRA Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for E-UTRA BS for co-existence with E-UTRA Band 27 UL operating band.

NOTE 5:	For E-UTRA Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for E-UTRA BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the downlink operating band and 10 MHz above the highest BS transmitter frequency of the downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.6.4.3.1-2: E-UTRA BS Spurious emissions limits for BS for co-existence with PHS

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -41 dBm | 300 kHz | Applicable when co-existence with PHS system operating in 1884.5-1915.7MHz |

The following requirement shall be applied to BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.6.4.3.1-3: BS Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -46 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -46 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -46 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -46 dBm | 6.25 kHz |  |

Table 6.6.4.3.1-4: Void

The following requirement shall be applied to BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.6.4.3.1-5: BS Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -13 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

The following requirement may apply to E-UTRA BS operating in Band 41 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.6.4.3.1-6: Additional E-UTRA BS Spurious emissions limits for Band 41

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 2505MHz – 2535MHz | -42dBm | 1 MHz |  |
| 2535MHz – 2655MHz | -22dBm | 1 MHz | Applicable at offsets ≥ 250% of channel bandwidth from carrier frequency. |
| NOTE: This requirement applies for 10 or 20 MHz E-UTRA carriers allocated within 2545-2575MHz or 2595-2645MHz. |  |  |  |

The following requirement may apply to E-UTRA BS operating in Band 30 in certain regions. This requirement is also applicable at the frequency range from 10 MHz below the lowest frequency of the BS downlink operating band up to 10 MHz above the highest frequency of the BS downlink operating band.

The power of any spurious emission shall not exceed:

Table 6.6.4.3.1-7: Additional E-UTRA BS Spurious emissions limits for Band 30

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 2200MHz – 2345MHz | -45dBm | 1 MHz |  |
| 2362.5MHz – 2365MHz | -25dBm | 1 MHz |  |
| 2365MHz – 2367.5MHz | -40dBm | 1 MHz |  |
| 2367.5MHz – 2370MHz | -42dBm | 1 MHz |  |
| 2370MHz – 2395MHz | -45dBm | 1 MHz |  |

In addition for Band 46 operation, the BS may have to comply with the applicable spurious emission limits established regionally, when deployed in regions where those limits apply and under the conditions declared by the manufacturer. The regional requirements may be in the form of conducted power, power spectral density, EIRP and other types of limits. In case of regulatory limits based on EIRP, assessment of the EIRP level is described in Annex H.

The following requirement may apply to E-UTRA BS operating in Band 48 and Band 49 in certain regions. The power of any spurious emission shall not exceed:

Table 6.6.4.3.1-8: Additional E-UTRA BS Spurious emissions limits for Band 48 and Band 49

| Frequency range | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 3530MHz – 3720MHz | -25dBm | 1 MHz | Applicable 10MHz from the assigned channel edge |
| 3100MHz – 3530MHz 3720MHz – 4200MHz | -40dBm | 1 MHz |  |

6.6.4.4	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with an E-UTRA BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

6.6.4.4.1	Minimum Requirement

The power of any spurious emission shall not exceed the limits of Table 6.6.4.4.1-1 for a Wide Area BS where requirements for co-location with a BS type listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.6.4.4.1-1 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.6.4.4.1-1 apply for the operating band supported at that antenna connector.

Table 6.6.4.4.1-1: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| Macro GSM900 | 876-915 MHz | -98 dBm | 100 kHz |  |
| Macro DCS1800 | 1710 - 1785 MHz | -98 dBm | 100 kHz |  |
| Macro PCS1900 | 1850 - 1910 MHz | -98 dBm | 100 kHz |  |
| Macro GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | 100 kHz |  |
| WA UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 - 1910 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 - 1785 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 - 849 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 - 2570 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 - 915 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 –1447.9 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50 or 75 |
| WA UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 - 716 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 788 - 798 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 17 | 704 - 716 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 18 | 815 - 830 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 - 862 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 32, 50 or 75 |
| WA UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42 |
| WA E-UTRA Band 23 | 2000 - 2020 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1850 – 1915 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 27 | 807 - 824 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 44 |
| WA E-UTRA Band 30 or NR Band n30 | 2305 – 2315 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 40 |
| WA E-UTRA Band 31 | 452.5 -457.5 MHz | -96 dBm | 100 kHz |  |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 33 |
| WA UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 34 |
| WA UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 35 |
| WA UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 2 and 36 |
| WA UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| WA UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 38. |
| WA UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880 – 1920MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 33 and 39 |
| WA UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 30 or 40 |
| WA E-UTRA Band 41 or NR Band n41 | 2496 – 2690 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 41 |
| WA E-UTRA Band 42 | 3400 – 3600 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 52 |
| WA E-UTRA Band 43 | 3600 – 3800 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43 or 48 |
| WA E-UTRA Band 44 | 703 – 803 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 28 or 44 |
| WA E-UTRA Band 45 | 1447 – 1467 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 45 |
| WA E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43 or 48 |
| WA E-UTRA Band 50 | 1432 – 1517 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 11, 21, 32, 74 or 75 |
| WA E-UTRA Band 52 | 3300 – 3400 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42 or 52 |
| WA E-UTRA Band 65 or NR Band n65 | 1920 - 2010 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 66 or NR Band n66 | 1710 - 1780 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 68 | 698 - 728 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 70 or NR Band n70 | 1695 - 1710 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 71 or NR Band n71 | 663 - 698 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 72 | 451 - 456 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 73 | 450 - 455 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 74 | 1427 – 1470 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50 |
| WA NR Band n77 | 3.3 – 4.2 GHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 52 |
| WA NR Band n78 | 3.3 – 3.8 GHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 52 |
| WA NR Band n79 | 4.4 – 5.0 GHz | -96 dBm | 100 kHz |  |
| WA NR Band n80 | 1710 – 1785 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n81 | 880 – 915 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n82 | 832 – 862 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n83 | 703 – 748 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n84 | 1920 – 1980 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 85 | 698 - 716 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n86 | 1710 – 1780 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 87 | 410 - 415 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 88 | 412 - 417 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n89 | 824 – 849 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n92 | 832 – 862 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n94 | 880 – 915 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n95 | 2010 - 2025 MHz | -96 dBm | 100 kHz |  |

The power of any spurious emission shall not exceed the limits of Table 6.6.4.4.1-2 for a Local Area BS where requirements for co-location with a BS type listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.6.4.4.1-2 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.6.4.4.1-2 apply for the operating band supported at that antenna connector.

Table 6.6.4.4.1-2: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| Pico GSM900 | 876-915 MHz | -70 dBm | 100 kHz |  |
| Pico DCS1800 | 1710 - 1785 MHz | -80 dBm | 100 kHz |  |
| Pico PCS1900 | 1850 - 1910 MHz | -80 dBm | 100 kHz |  |
| Pico GSM850 | 824 - 849 MHz | -70 dBm | 100 kHz |  |
| LA UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 - 1910 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 - 1785 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 - 849 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 - 2570 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 - 915 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50, 51, 75 or 76 |
| LA UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 - 716 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 788 - 798 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 17 | 704 - 716 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 18 | 815 - 830 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 - 862 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 32, 50 or 75 |
| LA UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42 |
| LA E-UTRA Band 23 | 2000 - 2020 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1850 – 1915 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 27 | 807 - 824 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 44 |
| LA E-UTRA Band 30 or NR Band n30 | 2305 – 2315 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 40 |
| LA E-UTRA Band 31 | 452.5 – 457.5 MHz | -88 dBm | 100 kHz |  |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 33 |
| LA UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 34 |
| LA UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 35 |
| LA UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 2 and 36 |
| LA UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| LA UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 38. |
| LA LUTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880 – 1920MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 33 and 39 |
| LA UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 30 or 40 |
| LA E-UTRA Band 41 or NR Band n41 | 2496 – 2690 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 41 or 53 |
| LA E-UTRA Band 42 | 3400 – 3600 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48, 49 or 52 |
| LA E-UTRA Band 43 | 3600 – 3800 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43, 48 or 49 |
| LA E-UTRA Band 44 | 703 – 803 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 28 or 44 |
| LA E-UTRA Band 45 | 1447 – 1467 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 45 |
| LA E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 46 |
| LA E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43, 48 or 49 |
| LA E-UTRA Band 49 | 3550 – 3700 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43, 48 or 49 |
| LA E-UTRA Band 50 | 1432 – 1517 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 11, 21, 32, 51, 74, 75 or 76 |
| LA E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50, 75 or 76 |
| LA E-UTRA Band 52 | 3300 – 3400 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42 or 52 |
| LA E-UTRA Band 53 or NR Band n53 | 2483.5 – 2495 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 41 or 53 |
| LA E-UTRA Band 65 or NR Band n65 | 1920 - 2010 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 66 or NR Band n66 | 1710 - 1780 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 68 | 698 - 728 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 70 or NR Band n70 | 1695 - 1710 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 71 or NR Band n71 | 663 - 698 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 72 | 451 - 456 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 73 | 450 - 455 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 74 | 1427 – 1470 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50 or 51 |
| LA NR Band n77 | 3.3 – 4.2 GHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48, 49 or 52 |
| LA NR Band n78 | 3.3 – 3.8 GHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48, 49 or 52 |
| LA NR Band n79 | 4.4 – 5.0 GHz | -88 dBm | 100 kHz |  |
| LA NR Band n80 | 1710 – 1785 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n81 | 880 – 915 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n82 | 832 – 862 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n83 | 703 – 748 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n84 | 1920 – 1980 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 85 | 698 - 716 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n86 | 1710 – 1780 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 87 | 410 - 415 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 88 | 412 - 417 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n89 | 824 – 849 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n91 | 832 – 862 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n92 | 832 – 862 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n93 | 880 – 915 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n94 | 880 – 915 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n95 | 2010 - 2025 MHz | -88 dBm | 100 kHz |  |

The power of any spurious emission shall not exceed the limits of Table 6.6.4.4.1-3 for a Medium Range BS where requirements for co-location with a BS type listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.6.4.4.1-3 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.6.4.4.1-3 apply for the operating band supported at that antenna connector.

Table 6.6.4.4.1-3: BS Spurious emissions limits for Medium range BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| Micro/MR GSM900 | 876-915 MHz | -91 dBm | 100 kHz |  |
| Micro/MR DCS1800 | 1710 - 1785 MHz | -91 dBm | 100 kHz |  |
| Micro/MR PCS1900 | 1850 - 1910 MHz | -91 dBm | 100 kHz |  |
| Micro/MR GSM850 | 824 - 849 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 - 1910 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 - 1785 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 - 849 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 850 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 - 2570 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 - 915 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50 or 75 |
| MR UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 - 716 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 788 - 798 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 17 | 704 - 716 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 18 | 815 - 830 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 - 862 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 32, 50 or 75 |
| MR UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42 |
| MR E-UTRA Band 23 | 2000 - 2020 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1850 – 1915 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 814 – 849 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 27 | 807 - 824 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 44 |
| MR E-UTRA Band 30 or NR Band n30 | 2305 – 2315 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 40 |
| MR E-UTRA Band 31 | 452.5 – 457.5 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 33 | 1900 - 1920 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 33 |
| MR E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 34 |
| MR E-UTRA Band 35 | 1850 – 1910 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 35 |
| MR E-UTRA Band 36 | 1930 - 1990 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 2 and 36 |
| MR E-UTRA Band 37 | 1910 - 1930 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| MR E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 38. |
| MR E-UTRA Band 39 or NR Band n39 | 1880 – 1920MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 33 and 39 |
| MR E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 30 or 40 |
| MR E-UTRA Band 41 or NR Band n41 | 2496 – 2690 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 41 or 53 |
| MR E-UTRA Band 42 | 3400 – 3600 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 52 |
| MR E-UTRA Band 43 | 3600 – 3800 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43 or 48 |
| MR E-UTRA Band 44 | 703 – 803 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 28 or 44 |
| MR E-UTRA Band 45 | 1447 – 1467 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 45 |
| MR E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 46 |
| MR E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42, 43 or 48 |
| MR E-UTRA Band 50 | 1432 – 1517 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 11, 21, 32, 74 or 75 |
| MR E-UTRA Band 52 | 3300 – 3400 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 42 or 52 |
| MR E-UTRA Band 53 or NR Band n53 | 2483.5 – 2495 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 41 or 53 |
| MR E-UTRA Band 65 or NR Band n65 | 1920 - 2010 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 66 or NR Band n66 | 1710 - 1780 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 68 | 698 - 728 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 70 or NR Band n70 | 1695 - 1710 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 71 or NR Band n71 | 663 - 698 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 72 | 451 - 456 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 73 | 450 - 455 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 74 | 1427 – 1470 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50 |
| MR NR Band n77 | 3.3 – 4.2 GHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 52 |
| MR NR Band n78 | 3.3 – 3.8 GHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 22, 42, 43, 48 or 52 |
| MR NR Band n79 | 4.4 – 5.0 GHz | -91 dBm | 100 kHz |  |
| MR NR Band n80 | 1710 – 1785 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n81 | 880 – 915 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n82 | 832 – 862 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n83 | 703 – 748 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n84 | 1920 – 1980 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 85 | 698 - 716 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n86 | 1710 – 1780 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 87 | 410 - 415 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 88 | 412 - 417 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n89 | 824 – 849 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n92 | 832 – 862 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n94 | 880 – 915 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n95 | 2010 - 2025 MHz | -91 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in Table 6.6.4.4.1-1 to Table 6.6.4.4.1-3 do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Table 5.5-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8].

NOTE 2:	Table 6.6.4.4.1-1 to Table 6.6.4.4.1-3 assume that two operating bands, where the corresponding BS transmit and receive frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations (except in Band 46), special co-location requirements may apply that are not covered by the 3GPP specifications.

7	Receiver characteristics

7.1	General

The requirements in clause 7 are expressed for a single receiver antenna connector. For receivers with antenna diversity, the requirements apply for each receiver antenna connector.

Unless otherwise stated, the receiver characteristics are specified at the BS antenna connector (test port A) with a full complement of transceivers for the configuration in normal operating conditions. For FDD operation the requirements in clause 7 shall be met with the transmitter(s) on. If any external apparatus such as a RX amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B).

NOTE:	In normal operating conditions the BS in FDD operation is configured to transmit and receive at the same time. The transmitter may be off for some of the tests as specifed in 36.141 [4].

Unless otherwise stated the requirements in clause 7 apply during the base station receive period.

Figure 7.1: Receiver test ports

The throughput requirements defined for the receiver characteristics in this clause do not assume HARQ retransmissions.

When the BS is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier. For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower Base Station RF Bandwidth edge and positive offsets of the interfering signal apply relative to the upper Base Station RF Bandwidth edge.

NOTE:	Requirements may only be supported for certain frequency ranges within the operating band(s). These frequency ranges could be different for NB.-IoT comparing to E-UTRA.

NOTE:	For E-UTRA BS with NB-IoT (in band and/or guard band) or standalone NB-IoT BS, requirements are defined for 15 kHz sub-carrier spacing and 3.75 kHz sub-carrier spacing. A NB-IoT Base Station supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both.

7.6	Blocking

7.6.1	General blocking requirement

The blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which are either a 1.4MHz, 3MHz or 5MHz E-UTRA signal for in-band blocking or a CW signal for out-of-band blocking. The interfering signal shall be an E-UTRA signal as specified in Annex C.

7.6.1.1	Minimum requirement

For E-UTRA, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in Tables 7.6.1.1-1, 7.6.1.1-1a, 7.6.1.1-1b, 7.6.1.1-1c and 7.6.1.1-2. The reference measurement channel for the wanted signal is identified in Table 7.2.1-1, 7.2.1-2, 7.2.1-3 and 7.2.1-4 for each channel bandwidth and further specified in Annex A.

The blocking requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in Table 7.6.1.1-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a BS capable of multi-band operation, the requirement in the in-band blocking frequency ranges applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in Table 7.6.1.1-2.

For a BS capable of multi-band operation, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to Tables 7.6.1.1-1, 7.6.1.1-1a and 7.6.1.1-1c shall be excluded from the out-of-band blocking requirement.

Table 7.6.1.1-1: Blocking performance requirement for Wide Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1. Note 2: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1.1-1 assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1.1-1a: Blocking performance requirement for Local Area BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13-14, 18,19, 21-23, 24, 27, 30, 33-45, 48-53, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1. 1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1. 1-2 | See table 7.6.1.1-2 |
|  | 1 | to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
|  | (FUL_high +20) | to |  |  |  |  |  |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1. 1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 46 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | (FUL_low -500) (FUL_high +20) | to to | (FUL_low -20) (FUL_high +500) | -35 | PREFSENS +6dB (Note 1) |  | CW carrier |
|  | 1 (FUL_high +500) | to to | (FUL_low -500) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1. 1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-2 Note 2: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1.1-1a assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed  in the same geographical area.

Table 7.6.1.1-1b: Blocking performance requirement for Home BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the channel edge of the wanted signal [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-44, 48, 50-52, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 74 | (FUL_low -20) | to | (FUL_high +5) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +14dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-3. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1.1-1b assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1.1-1c: Blocking performance requirement for Medium Range BS for E-UTRA

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset to the lower/higher Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52-53, 65, 66, 68, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 46 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | (FUL_low -500) (FUL_high +20) | to to | (FUL_low -20) (FUL_high +500) | -35 | PREFSENS +6dB (Note 1) |  | CW carrier |
|  | 1 (FUL_high +500) | to to | (FUL_low -500) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +12) | to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-4. Note 2: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to PREFSENS + 1.4 dB. |  |  |  |  |  |  |  |

NOTE:	Table 7.6.1.1-1c assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1.1-2: Interfering signals for blocking performance requirement

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4 MHz E-UTRA signal |
| 3 | ±4.5 | 3 MHz E-UTRA signal |
| 5 | ±7.5 | 5 MHz E-UTRA signal |
| 10 | ±7.5 | 5 MHz E-UTRA signal |
| 15 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±7.5 | 5 MHz E-UTRA signal (Note 1) |
| 20 | ±30 | 20 MHz E-UTRA signal (Note 2) |
| Note 1: This type of interfering signal is not applied for Band 46. Note 2: This type of interfering signal is only applied for Band 46. |  |  |

For NB-IoT standalone operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in Tables 7.6.1.1-3, 7.6.1.1-3a, 7.6.1.1-3b, 7.6.1.1-3c and 7.6.1.1-4. The reference measurement channel for the wanted signal is identified in Table 7.2.1-5, 7.2.1-5a, 7.2.1-5b and 7.2.1-5c and further specified in Annex A.

The blocking requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 7.6.1.1-3: Blocking performance requirement for Wide Area BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS is specified in Table 7.2.1-5. Note 2: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

Table 7.6.1.1-3a: Blocking performance requirement for Local Area BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6dB (Note 2) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 2) |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB (Note 2) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 2) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -35 | PREFSENS +6dB (Note 2) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 2) |  | CW carrier |
| Note 1: PREFSENS is specified in Table 7.2.1-5a. Note 2: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

Table 7.6.1.1-3b: Blocking performance requirement for Home BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7,11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 74 | (FUL_low -20) | to | (FUL_high +5) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +14dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS is specified in Table 7.2.1-5b. Note 2: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

Table 7.6.1.1-3c: Blocking performance requirement for Medium Range BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS is specified in Table 7.2.1-5c. Note 2: Up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Tables 7.6.1.1-3, 7.6.1.1-3a, 7.6.1.1-3b and 7.6.1.1-3c assumes that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1.1-4: Interfering signals for blocking performance requirement for NB-IoT standalone operation

| NB-IoT channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 0.2 | ±7.5 | 5MHz E-UTRA signal |

For E-UTRA with NB-IoT in-band/guard band operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in Tables 7.6.1.1-5, 7.6.1.1-5a, 7.6.1.1-5b, 7.6.1.1-5c and 7.6.1.1-6. The reference measurement channel for the wanted signal is identified in Table 7.2.1-1, 7.2.1-2, 7.2.1-3 and 7.2.1-4 for each channel bandwidth for E-UTRA, Table 7.2.1-5, 7.2.1-5a, 7.2.1-5b and 7.2.1-5c for NB-IoT and further specified in Annex A.

The blocking requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 7.6.1.1-5: Blocking performance requirement for Wide Area BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7,11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 73, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1 for E-UTRA and is specified in Table 7.2.1-5 for NB-IoT. Note 2: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power is equal to PREFSENS + 1.4 dB. Note 3: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

Table 7.6.1.1-5a: Blocking performance requirement for Local Area BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1 for E-UTRA and is specified in Table 7.2.1-5a for NB-IoT. Note 2: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power is equal to PREFSENS + 1.4 dB. Note 3: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

Table 7.6.1.1-5b: Blocking performance requirement for Home BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 74 | (FUL_low -20) | to | (FUL_high +5) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -27 | PREFSENS +14dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +14dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1 for E-UTRA and is specified in Table 7.2.1-5b for NB-IoT. Note 2: (Void) Note 3: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

Table 7.6.1.1-5c: Blocking performance requirement for Medium Range BS for E-UTRA with NB-IoT in-band/guard band operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 42, 43, 65, 66, 70 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 8, 26, 28 | (FUL_low -20) | to | (FUL_high +10) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 12 | (FUL_low -20) | to | (FUL_high +13) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 17 | (FUL_low -20) | to | (FUL_high +18) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 20, 71 | (FUL_low -11) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 25 | (FUL_low -20) | to | (FUL_high +15) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 31, 72, 74, 87, 88 | (FUL_low -20) | to | (FUL_high +5) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| 85 | (FUL_low -20) | to | (FUL_high +12) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-6 | See table 7.6.1.1-6 |
|  | 1 (FUL_high +12) | to to | (FUL_low -20) 12750 | -15 (Note 3) | PREFSENS +6dB (Note 1) |  | CW carrier |
| Note 1: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1 for E-UTRA and is specified in Table 7.2.1-5c for NB-IoT. Note 2: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, the wanted signal mean power is equal to PREFSENS + 1.4 dB. Note 3: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |

NOTE:	Tables 7.6.1.1-5, 7.6.1.1-5a and 7.6.1.1-5b assume that two operating bands, where the downlink operating band (see Table 5.5-1) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.6.1.1-6: Interfering signals for blocking performance requirement for E-UTRA with NB-IoT in-band/guard band operation

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 3(Note) | ±4.5 | 3MHz E-UTRA signal |
| 5 | ±7.5 | 5MHz E-UTRA signal |
| 10 | ±7.5 | 5MHz E-UTRA signal |
| 15 | ±7.5 | 5MHz E-UTRA signal |
| 20 | ±7.5 | 5MHz E-UTRA signal |
| Note: 3 MHz channel bandwidth is not applicable to guard band operation. |  |  |

7.6.2	Co-location with other base stations

This additional blocking requirement may be applied for the protection of E-UTRA and NB-IoT BS receivers when GSM, CDMA, UTRA, E-UTRA, NR or NB-IoT BS operating in a different frequency band are co-located with an E-UTRA or NB-IoT BS. The requirement is applicable to all channel bandwidths supported by the E-UTRA BS and E-UTRA with NB-IoT in-band/guard band operation.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and E-UTRA or NB-IoT BS receiver and are based on co-location with base stations of the same class.

7.6.2.1	Minimum requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in Table 7.6.2.1-1 for Wide Area BS, in Table 7.6.2.1-2 for Local Area BS and in Table 7.6.2.1-3 for Medium Range BS. The reference measurement channel for the wanted signal is identified in Tables 7.2.1-1, 7.2.1-2 and 7.2.1-4 for each channel bandwidth for E-UTRA, Table 7.2.1-5 for NB-IoT and further specified in Annex A.

Table 7.6.2.1-1: Blocking performance requirement for E-UTRA and NB-IoT Wide Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Macro GSM850 or CDMA850 | 869 – 894 | +16** | PREFSENS + 6dB* | CW carrier |
| Macro GSM900 | 921 – 960 | +16** | PREFSENS + 6dB* | CW carrier |
| Macro DCS1800 | 1805 – 1880 | +16** | PREFSENS + 6dB* | CW carrier |
| Macro PCS1900 | 1930 – 1990 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 – 2170 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 – 1990 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 – 1880 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 – 894 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 – 2690 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 – 960 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 –1495.9 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 - 746 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 758 - 768 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 17 | 734 - 746 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 18 | 860 - 875 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 – 1510.9 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 23 | 2180 - 2200 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 24 | 1525 – 1559 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 859 – 894 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 27 | 852 - 869 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 28 or NR band n28 | 758 – 803 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 29 or NR Band n29 | 717-728 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 30 or NR Band n30 | 2350 – 2360 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 31 | 462.5-467.5 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA FDD Band XXXII or E-UTRA Band 32 | 1452-1496 (NOTE 3) | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900-1920 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010-2025 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band b) or E-UTRA Band 35 | 1850-1910 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band b) or E-UTRA Band 36 | 1930-1990 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band c) or E-UTRA Band 37 | 1910-1930 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570-2620 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880-1920 | +16** | PREFSENS + 6dB* | CW carrier |
| WA UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300-2400 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 41 or NR band n41 | 2496 - 2690 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 42 | 3400-3600 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 43 | 3600-3800 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 44 | 703-803 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 45 | 1447-1467 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 48 or NR Band n48 | 3550-3700 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 50 or NR band n50 | 1432 – 1517 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 52 | 3300-3400 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 65 or NR Band n65 | 2110 – 2200 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 66 or NR band n66 | 2110 – 2200 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 67 | 738-758 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 68 | 753-783 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 69 | 2570-2620 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 70 or NR band n70 | 1995 – 2020 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 71 or NR band n71 | 617 – 652 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 72 | 461 – 466 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 73 | 460 – 465 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 74 or NR band n74 | 1475 - 1518 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 75 | 1432 – 1517 | +16** | PREFSENS + 6dB* | CW carrier |
| WA NR band n77 | 3300-4200 | +16** | PREFSENS + 6dB* | CW carrier |
| WA NR band n78 | 3300-3800 | +16** | PREFSENS + 6dB* | CW carrier |
| WA NR band n79 | 4400-5000 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 85 | 728 - 746 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 87 | 420 – 425 | +16** | PREFSENS + 6dB* | CW carrier |
| WA E-UTRA Band 88 | 422 – 427 | +16** | PREFSENS + 6dB* | CW carrier |
| WA NR band n92 | 1432 – 1517 | +16** | PREFSENS + 6dB* | CW carrier |
| WA NR band n94 | 1432 – 1517 | +16** | PREFSENS + 6dB* | CW carrier |
| Note*: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-1 for E-UTRA and is specified in Table 7.2.1-5 for NB-IoT. Note**: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |
| NOTE 1: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8]. NOTE 3: For a BS operating in band 11, 21 or 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 4: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

Table 7.6.2.1-2: Blocking performance requirement for E-UTRA and NB-IoT Local Area BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Pico GSM850 | 869 – 894 | -7** | PREFSENS + 6dB* | CW carrier |
| Pico GSM900 | 921 – 960 | -7** | PREFSENS + 6dB* | CW carrier |
| Pico DCS1800 | 1805 – 1880 | -4** | PREFSENS + 6dB* | CW carrier |
| Pico PCS1900 | 1930 – 1990 | -4** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 – 2170 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 – 1990 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 – 1880 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 – 894 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 – 2690 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 – 960 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 - 746 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 758 - 768 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 17 | 734 - 746 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 18 | 860 - 875 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 – 1510.9 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 23 | 2180 - 2200 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 24 | 1525 – 1559 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 859 – 894 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 27 | 852 - 869 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 28 or NR band n28 | 758 – 803 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 29 or NR Band n29 | 717 - 728 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 30 or NR Band n30 | 2350 – 2360 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 31 | 462.5 - 467.5 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 (NOTE 3) | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band b) or E-UTRA Band 35 | 1850 - 1910 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 - 2620 | -6** | PREFSENS + 6dB* | CW carrier |
| LA UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 - 1920 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA UTRA TDD Band e) or Band 40 or NR band n40 | 2300 - 2400 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 41 or NR band n41 | 2496 - 2690 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 42 | 3400 - 3600 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 43 | 3600 - 3800 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 44 | 703 - 803 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 45 | 1447 - 1467 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 46 or NR Band n46 | 5150 - 5925 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 48 or NR Band n48 | 3550 - 3700 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 49 | 3550 - 3700 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 50 or NR band n50 | 1432 – 1517 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 51 or NR band n51 | 1427 – 1432 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 52 | 3300 - 3400 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 53 or NR Band n53 | 2483.5 - 2495 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 65 or NR Band n65 | 2110 – 2200 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 66 or NR band n66 | 2110 – 2200 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 67 | 738 - 758 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 68 | 753 - 783 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 69 | 2570 - 2620 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 70 or NR band n70 | 1995 – 2020 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 71 or NR band n71 | 617 – 652 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 72 | 461 – 466 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 73 | 460 – 465 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 74 or NR band n74 | 1475 - 1518 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 75 or NR band n75 | 1432 – 1517 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 76 or NR band n76 | 1427 – 1432 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n77 | 3300 - 4200 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n78 | 3300 - 3800 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n79 | 4400 - 5000 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 85 | 728 - 746 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 87 | 420 – 425 | -6** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band 88 | 422 – 427 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n91 | 1427 – 1432 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n92 | 1432 – 1517 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n93 | 1427 – 1432 | -6** | PREFSENS + 6dB* | CW carrier |
| LA NR band n94 | 1432 – 1517 | -6** | PREFSENS + 6dB* | CW carrier |
| Note*: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-2. Note**: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |
| NOTE 1: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8]. NOTE 3: For a BS operating in band 11, 21 or 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 4: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations (except in Band 46), special co-location requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

Table 7.6.2.1-3: Blocking performance requirement for E-UTRA and NB-IoT Medium Range BS when co-located with BS in other frequency bands.

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Micro/MR GSM850 | 869 – 894 | +8** | PREFSENS + 6dB* | CW carrier |
| Micro/MR GSM900 | 921 – 960 | +8** | PREFSENS + 6dB* | CW carrier |
| Micro/MR DCS1800 | 1805 – 1880 | +8** | PREFSENS + 6dB* | CW carrier |
| Micro/MR PCS1900 | 1930 – 1990 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 – 2170 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 – 1990 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 – 1880 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 – 894 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 – 2690 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 – 960 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 –1495.9 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 – 746 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XIIII or E-UTRA Band 13 | 746 – 756 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 758 – 768 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 17 | 734 – 746 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 18 | 860 – 875 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XIX or E-UTRA Band 19 | 875 – 890 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 – 821 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 – 1510.9 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 23 | 2180 – 2200 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 24 | 1525 – 1559 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 – 1995 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 859 – 894 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 27 | 852 – 869 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 28 or NR band n28 | 758 – 803 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 29 or NR Band n29 | 717 – 728 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 30 or NR Band n30 | 2350 – 2360 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 31 | 462.5 – 467.5 | +8** | PREFSENS + 6dB* | CW carrier |
| MR UTRA FDD Band XXXII or E-UTRA Band 32 | 1452-1496 (NOTE 3) | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 33 | 1900 – 1920 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 34 or NR band n34 | 2010 – 2025 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 35 | 1850 – 1910 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 36 | 1930 – 1990 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 37 | 1910 – 1930 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 38 or NR band n38 | 2570 – 2620 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 39 or NR band n39 | 1880 – 1920 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 40 or NR band n40 | 2300 – 2400 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 41 or NR band n41 | 2496 – 2690 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 42 | 3400 – 3600 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 43 | 3600 – 3800 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 44 | 703 – 803 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 45 | 1447 – 1467 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 46 or NR Band n46 | 5150 – 5925 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 48 or NR Band n48 | 3550-3700 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 50 or NR band n50 | 1432 – 1517 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 52 | 3300 – 3400 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 53 or NR Band n53 | 2483.5 - 2495 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 65 or NR Band n65 | 2110 – 2200 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 66 or NR band n66 | 2110 – 2200 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 67 | 738-758 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 68 | 753-783 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 69 | 2570-2620 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 70 or NR band n70 | 1995 – 2020 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 71 or NR band n71 | 617 – 652 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 72 | 461 – 466 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 73 | 460 – 465 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 74 or NR band n74 | 1475 - 1518 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 75 or NR band n75 | 1432 – 1517 | +8** | PREFSENS + 6dB* | CW carrier |
| MR NR band n77 | 3300-4200 | +8** | PREFSENS + 6dB* | CW carrier |
| MR NR band n78 | 3300-3800 | +8** | PREFSENS + 6dB* | CW carrier |
| MR NR band n79 | 4400-5000 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 85 | 728 - 746 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 87 | 420 – 425 | +8** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band 88 | 422 – 427 | +8** | PREFSENS + 6dB* | CW carrier |
| MR NR band n92 | 1432 – 1517 | +8** | PREFSENS + 6dB* | CW carrier |
| MR NR band n94 | 1432 – 1517 | +8** | PREFSENS + 6dB* | CW carrier |
| Note*: PREFSENS depends on the channel bandwidth as specified in Table 7.2.1-4. Note**: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |
| NOTE 1: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [8]. NOTE 3: For a BS operating in band 11, 21 or 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 4: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations (except in Band 46), special co-location requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |