3GPP TSG-RAN WG4 Meeting #96-e	R4-2010981

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion on URLLC UE demodulation requirements with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2.1

Document for:	Discussion

High reliability

In this paper, the test cases of URLLC high reliability with higher BLER and low latency are discussed.

Background:

For URLLC PDSCH with BLER target of 10^-2 test case, three open issues are remained from previous meeting. The open issues are listed below:

Open issues of #95:

- MCS:

  - Option 1: MCS13

  - Option 2: MCS16

  - Option 3: MCS19

- Methodology for MCS selection

  - Option 1: Higher or equal to -6 dB for average ideal 4 Rx SNR alignment results.

  - Option 2: Higher or equal to -3 dB for average ideal 4 Rx SNR alignment results

  - Option 3: Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM)

- FR2 requirements for High reliability

  - FFS whether to define FR2 requirements

  - Interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios

Discussion

MCS:

According to the simulation results shown in R4-2010982, MCS13 is suitable for both of 2x4 and 4x4.

Proposal 1: We propose to use MCS13 for this case.

Methodology for MCS selection:

With options listed above, option 1 and option 3 are similar. To make the decision more straightforward, we prefer option 3

Proposal 2: We prefer option 3 that select the MCS higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM).

FR2 requirements for high reliability:

For current scenarios, FR2 is not a common use case for URLLC. We propose do not to define FR2 requirements for high reliability.

Proposal 3: Do not define UE FR2 URLLC requirements for high reliability.

Low latency:

Two test cases are defined for URLLC UE low latency.

PDSCH mapping type B and processing capability 2:

Background:

In this paper, the simulation results for PDSCH mapping type B and PDSCH processing capability 2 are provided based on the agreed parameters. In addition, the open issues are discussed. Open issues of #95 e-meeting are listed below based on the WF [1]:

Open Issues:

- Number of HARQ process for TDD:

  - Option 1: 2

  - Option 2: 4

- MCS:

  - Option 1: Only MCS 4

  - Option 2: Only MCS 17

  - Option 3: MCS 4 and MCS 17

Discussion

Number of HARQ process for TDD:

For TDD, only S slot will be scheduled, the corresponding HARQ process only needs to counter S, no need to consider D slot, i.e.

Thus, 2 HARQ processes are enough for “DDDSU”.

MCS:

Based on the RAN1’s study about the URLLC low latency, URLLC prefer to allocate full bandwidth with small symbol length to reduce latency. With full bandwidth allocation, the payload size of MCS17 is not typical for URLLC use cases. Thus, we propose to configure MCS4 in this case.

Proposal 4: HARQ process is 2 for TDD.

Proposal 5: Only define MCS4 to verify mapping Type B and processing capability 2.

Pre-emption indication

Background:

In this paper, open issues are discussed based on the agreed parameters. Our proposals are given.

Open Issues:

    - Pre-emption probability

      - Option 1: 10% within 1 radio frame

      - Option 2: 20% within 1 radio frame

    - eMBB MCS

      - Option 1: MCS13 in Table 1

      - Option 2: MCS4 in Table 1

    - Test metric

      - Option 1: 70% of max T-put

      - Other options are not precluded

    - Companies are encouraged to prepare comparison analysis of UE with and without HARQ buffer flushing of pre-empted bits to decide on options above

Discussion

Based on our simulation results present in R4-2010982, the SNR differences between 10% probability and 20% probability for MCS4 and MCS10 are presented in table below:

Table 2.2.2-1: SNR difference for pre-emption performance requirements

| Probability | MCS4 | MCS13 |
| --- | --- | --- |
| 10% | 0.1 | 0.9 |
| 20% | 0.2 | 4.8 |

Table 2.2.2-1 shows that the SNR difference between buffer flushed or not is very small for MCS4 which is about 0.1dB with 10% probability and 0.2dB for 20% probability, it is very difficult to verify whether the eMBB service is pre-empted or not. For MCS13, the SNR loss for pre-emption with 10% probability is around 0.9dB. And the SNR loss for pre-emption with 20% probability is around 4.8dB. By considering the real use cases, 10% probability with MCS 13 is more reasonable.

Proposal 6: We propose to use MCS13 for the pre-emption verification.

Proposal 7: We propose 10% probability for pre-emption periodicity.

Test applicability

Background

As presented in WF R4-2008807, the release independent and whether to introduce RAN4 features and capability signaling were discussed during last meeting. The open issues are listed below:

Open issues:

- FFS whether UE URLLC requirements for Rel-15 features are release independent from Rel-15

- FFS necessity of introduction of RAN4 features and UE applicability for URLLC requirements for Rel-15 features

Discussion

This issue was also discussed in other topics. Considering all UE URLLC performance requirements will be defined based on the Rel-15 core requirements, it is feasible to be release independent from Rel-15. And specifying it in TS 38.307 that is similar as LTE did for TS 36.307.

Moreover, we do not think additional capability is needed as requirements are defined release independent from Rel-15.

Proposal 8: UE URLLC requirements for Rel-15 features are release independent from Rel-15.

Proposal 9: No need to introduce RAN4 features and capability for URLLC requirements for Rel-15 features.

Proposals:

Proposal 1: We propose to use MCS13 for this case.

Proposal 2: We prefer option 3 that select the MCS higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM).

Proposal 3: Do not define UE FR2 URLLC requirements for high reliability.

Proposal 4: HARQ process is 2 for TDD.

Proposal 5: Only define MCS4 to verify mapping Type B and processing capability 2.

Proposal 6: We propose to use MCS13 for the pre-emption verification.

Proposal 7: We propose 10% probability for pre-emption periodicity.

Proposal 8: UE URLLC requirements for Rel-15 features are release independent from Rel-15.

Proposal 9: No need to introduce RAN4 features and capability for URLLC requirements for Rel-15 features.

Reference

- R4-2008807, “Way forward for NR UE URLLC performance requirements”, Intel Corporation.