3GPP TSG-RAN WG4 Meeting #96-e	R4-2010984

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Simulation results on URLLC BS demodulation requirements with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2.3

Document for:	Discussion

Background

In this paper, URLLC BS PUSCH demodulation requirements for high reliability and low latency are provided based on the agreed parameters.

Simulation

High reliability

Simulation assumption

According to the WF [1], the test parameters according to the agreements of #94 and #94bis e-meeting are concluded in table below:

Table 2-1: Parameters for BS PUSCH performance requirements for high reliability

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA Low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | n2 for 30 kHz SCS FFS 15 kHz SCS |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Propagation condition |  | TDLB100-400 |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| Frequency domain resource |  | Full Bandwidth |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Maximum number of HARQ transmissions |  | 4 |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |

Simulation result

- TDD

Based on the parameters listed in Table 2-1, the BLER vs SNR curve for TDD pattern “7D1S2U” is shown in figure below.

Figure 2-1: BLER vs SNR for PUSCH TDD pattern “7D1S2U”

Figure 2-1 shows that the SNR value is around -8.5dB with target BLER=0.01.

Low latency

Simulation assumption

The test parameters according to the agreements of #94 and #94bis e-meeting and the parameters we proposed are concluded in table below:

Table 2-2 Simulation assumption for PUSCH mapping Type B

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, low |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | 0 |
| Time domain resource assignment | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 2 |
|  | PUSCH aggregation factor | 1 |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| MCS Table |  | Table 3, MCS 10 |
| Propagation condition |  | TDLC300-100 Low |
| Maximum number of HARQ transmissions |  | 1 |
| Frequency domain resource assignment |  | Full bandwidth |
| Test metric |  | 70% TP |

Simulation results

The performance for PUSCH mapping Type B for TDD with symbol length of 2 is presented in figure below:

- TDD

Based on the agreed and proposed parameters listed in Table 2-2, the simulated performances for TDD are shown in Figure 2-2 for symbol length of 2.

Figure 2-2: Throughput vs SNR for PUSCH TDD with symbol length of 2

The SNR values of 70% throughput for TDD are list in table below:

Table 2-3: SNR values of 70% throughput for TDD

| Symbol length SNR Antenna configuration | 1x2 |
| --- | --- |
| L=2 | -2.8 |

Reference

[1] R4-2005528, Way forward on NR URLLC BS performance requirements, #94bis, Huawei, Hisilicon.