3GPP TSG-RAN WG4 Meeting #96-e	R4-2010996

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion on PDSCH requirements for NR DL 256QAM for FR2

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.1

Document for:	Discussion

Background

During RAN4#95-e meeting, way forward [1] for NR DL 256QAM for FR2 was approved. In this contribution, we share our views on PDSCH requirements for NR DL 256QAM for FR2.

Discussion

Testable SNR for PDSCH requirements

The mainly factor of testability is the testable SNR, which is in a certain range due to the limitation of the transmitted signal power of TE. The maximum SNR for single band UE is referenced from the latest TS 38.810 [2], as the Table 2.1-1 shows below.

Table 2.1-1: Maximum testable SNR for single band UE

| Test methodology | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| DNF | 100 MHz | [22.6 dB] |
|  | 200 MHz | [19.6 dB] |
| DFF | 100 MHz | [19.4 dB] |
|  | 200 MHz | [16.4 dB] |
| IFF | 100 MHz | [19.7 dB] |
|  | 200 MHz | [16.7 dB] |

UE can’t assume which test method is used, therefore only the maximum SNR 19.4dB can be ensured which is the maximum SNR using DFF method. For channel bandwidth 50MHz, we calculate that the maximum testable SNR using DFF method is 22.6dB derived from spreadsheet in TS 38.810.

Observation 1: For NR DL 256QAM for FR2, the maximum testable SNR is 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

To ensure the UE performance under 256QAM for FR2, RAN4 should define the performance requirements for NR DL 256QAM for FR2 with the cases that satisfying the demand that required SNR is less than maximum testable SNR.

Proposal 1: RAN4 should define the performance requirements for NR DL 256QAM for FR2 with the cases that satisfying the demand that required SNR is less than 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

Simulations

The following simulation results is based on the agreed simulation assumption in WF [1].

Table 2.2-1 Simulation results for PDSCH demodulation

| Case number | CBW/MHz | MIMO | Rank | Propagation condition | MCS | SNR@70%max throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 1 | 50 | 1x2 | 1 | AWGN | 20 | 15.92 | 18.42 |
| 2 | 50 | 2x2 | 1 | TDL-D30-35 | 20 | 16.70 | 19.20 |
| 3 | 50 | 2x2 | 1 | TDL-A30-300 | 20 | 17.94 | 20.44 |
| 4 | 50 | 2x2 | 2 | AWGN | 20 | 18.94 | 21.44 |
| 5 | 50 | 2x2 | 2 | TDL-D30-35 | 20 | N/A | N/A |
| 6 | 50 | 2x2 | 2 | TDL-A30-300 | 20 | N/A | N/A |
| 7 | 100 | 1x2 | 1 | AWGN | 20 | 15.72 | 18.22 |
| 8 | 100 | 2x2 | 1 | TDL-D30-35 | 20 | 15.65 | 18.15 |
| 9 | 100 | 2x2 | 1 | TDL-A30-300 | 20 | 17.44 | 19.94 |
| 10 | 100 | 2x2 | 2 | AWGN | 20 | 18.89 | 21.35 |
| 11 | 100 | 2x2 | 2 | TDL-D30-35 | 20 | N/A | N/A |
| 12 | 100 | 2x2 | 2 | TDL-A30-300 | 20 | N/A | N/A |
| Note: N/A means large than 25dB or cannot achieve 70% max throughput. |  |  |  |  |  |  |  |

Observation 2: Fading channel with rank 2 is not feasible.

Observation 3: For 100MHz CBW, only cases for AWGN/TDL-D30-35 with MCS 20 and rank 1 are feasible.

Test setup

As per simulations in section 2.2, we prefer to select the parameters for the test setup as following:

Rank&MCS

From the simulation results and Observation 2, comparing to the testable SNR analyzed in Section 2.1, we propose only define cases with MCS 20 and rank 1.

Channel bandwidth

From the simulation results and Observation 3, we prefer to define 50MHz CBW for PUSCH performance requirement for NR DL 256QAM for FR2.

Propagation condition

Considering the extra workload for simplifying TDL-D channel model, we are OK to define TDL-C cases.

Rx phase noise model

In Rel-15, there is no explicit phase noise modelling and all cases are band agnostic. Also, phase noise model in TR 38.803 is just an example. It is not suitable to use such example model when defining performance requirements. Therefore we propose to use the same principle as Rel-15 that not explicit modelling Rx impairment and leave it to UE implementation.

As above, we can get the following proposal:

Proposal 2: Define requirements for NR DL 256QAM for FR2 with the following parameters:

| Parameters | Value |
| --- | --- |
| Rank | 1 |
| MCS | 20 |
| CBW | 50MHz |
| Propagation condition | TDL-C30-300 |
| Rx impairment | No explicit modelling |

Proposals

In this contribution, we discuss on PUSCH requirements for NR DL 256QAM for FR2. Our observations and proposals are:

Observation 1: For NR DL 256QAM for FR2, the maximum testable SNR is 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

Observation 2: Fading channel with rank 2 is not feasible.

Observation 3: For 100MHz CBW, only cases for AWGN/TDL-D30-35 with MCS 20 and rank 1 are feasible.

Proposal 1: RAN4 should define the performance requirements for NR DL 256QAM for FR2 with the cases that satisfying the demand that required SNR is less than 22.6dB for 50MHz bandwidth and 19.4dB for 100MHz bandwidth respectively.

Proposal 2: Define requirements for NR DL 256QAM for FR2 with the following parameters:

| Parameters | Value |
| --- | --- |
| Rank | 1 |
| MCS | 20 |
| CBW | 50MHz |
| Propagation condition | TDL-C30-300 |
| Rx impairment | No explicit modelling |

Reference

R4-2008817, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4#95-e, China Telecom

TS 38.810, Study on test methods