3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2011090

# Electronic Meeting, 17 August – 28 August, 2020

Agenda item:	6.2.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of Rel-16 NB-IoT

Document for:	Discussion

- Introduction

For the great efforts during the past meetings, the core requirements of Rel-16 NB-IoT has been completed in the last RAN4#95e meeting. The discussion of the performance part shall be triggered in this meeting, thus we present our thoughts about the performance part requirements in this paper.

- Discussion

In this paper, we present our views on whether the test cases are necessary for the introduced features in Rel-16 one by one.

Group WUS

In Rel-15, we didn’t define performance test cases for WUS reception. So for Group WUS in Rel-16 it is suggested not to define test cases for these feature.

Proposal 1: It is suggested not to define test cases for Group WUS.

DL Quality report in Msg3 for non-anchor in IDLE mode

As RAN4 has defined the test cases for quality report on anchor carrier in Rel-14, it is suggested to have the corresponding test cases for Msg3-based quality report on non-anchor carrier in Rel-16.

Proposal 2: RAN4 to define the Msg-3 based channel quality report on non-anchor carrier.

DL Quality report in connected mode

RAN4 should have test cases for DL quality report in connected mode in Rel-16 which could take the Msg3-based quality report in Rel-14 as the starting point.

Proposal 3: RAN4 to define test cases for channel quality report in connected mode.

PUR

RAN4 has defined the NRSRP based TA validation requirement and transmit timing accuracy for PUR transmission. It is suggested to define corresponding test to avoid interference when using PUR. Companies had raised the concern that whether it could be guaranteed that the UE will use PUR for data transmission in IDLE model. From our understanding, it is feasible to enable the feature by particular signalling (i.e. AT command) in the test mode.

Proposal 4: RAN4 to defined test case for TA validation requirement and transmit timing accuracy for PUR transmission

Measurement on non-anchor carrier

It is quite similar as the PUR cases that UE may not perform serving cell measurement on non-anchor carrier if the feature is supported and the conditions are met. So it is suggested not to define test cases for measurement on non-anchor carrier.

Proposal 5: It is suggested not to define test cases for measurement on non-anchor carrier.

UE specific DRX

RAN4 has updated the corresponding RRM requirements for the new introduced UE specific DRX cycles length (320ms and 640ms). The corresponding test cases has been defined for UE in IDLE mode for legacy DRX values, so it is straightforward to define the corresponding test cases for UE using short DRX cycles.

Proposal 6: RAN4 to define test cases for the new introduced short DRX cycles length.

Based on the analysis above, we also present the accompanied paper for the test cases lists in detail [1].

- Conclusions

Proposal 1: It is suggested not to define test cases for Group WUS.

Proposal 2: RAN4 to define the Msg-3 based channel quality report on non-anchor carrier.

Proposal 3: RAN4 to define test cases for channel quality report in connected mode.

Proposal 4: RAN4 to defined test case for TA validation requirement and transmit timing accuracy for PUR transmission

Proposal 5: It is suggested not to define test cases for measurement on non-anchor carrier.

Proposal 6: It is suggested to define test cases for the new introduced short DRX cycles length.

References

[1] R4-2011091, Test cases list for Rel-16 NB-IoT