3GPP TSG-RAN4 Meeting #96-e	R4-2011153

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1081 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on requirements for SCell dormancy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The delay and interruption requirements for SCell dormancy are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update delay requirements for SCell dormancy - Clarify the SCS for Z - Add optimized requirement - Add multi-CC dormancy dormancy switch requirement Update interruption requirements for SCell dormancy - Add interruption for RRM/CSI measurement during dormancy - Add interruption for multiple CC dormancy switch - Clarify the applicability of interrutption due to SCS change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The delay and interruption requirements for SCell dormancy are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2, 8.6.2A, 8.2.1.2.15, 8.2.2.2.12, 8.2.3.2.13, 8.2.4.2.11, 8.2.4.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.6.2	DCI and timer based BWP switch delay on a single CC

The requirements in this clause only apply to the case that the BWP switch is performed on a single CC.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + X which starts from the beginning of DL slot n, where

-	TBWPswitchDelay is defined in Table 8.6.2-1 corresponding to the smaller value between the SCS of the serving cell where UE receives BWP switching request and the SCS of the serving cell where BWP switching occurs;

-	X=1 slot with respect to the SCS of the serving cell where BWP switching occurs, if the SCS of the serving cell where UE receives BWP switching request is smaller than or equal to the SCS of the serving cell where BWP switching occurs, X=0 otherwise.

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n except DCI triggering BWP switch on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay after bwp-InactivityTimer [2] expires on the cell where timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in Clause 8.10 in the new BWP.

If the BWP switch is triggered within DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 7] and the configurations of the two BWPs differ in any other parameter than PDCCH-ConfigCommon or PDCCH-Config, UE shall be able to complete active BWP switching within

-	TBWPswitchDelay+ X, provided that the BWP switching request is received in any of the first 3 OFDM symbols of a slot in the serving cell on which DCI for dormancy indication is received, or

-	TBWPswitchDelay + X + Z, provided that the BWP switching request is received after the first 3 OFDM symbols of a slot in the serving cell on which DCI for dormancy indication is received, where Z=1 slot with respect to the SCS of the serving cell on which DCI for dormancy indication is received.

If the BWP switch is triggered within DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 7] and the configurations of the two BWPs differ only in PDCCH-ConfigCommon or PDCCH-Config, the above requriements with TBWPswitchDelay corresponding to Type 1 UE capability apply.

8.6.2A	DCI based BWP switch delay on multiple CCs

The requirements in this clause only apply to the case when the same type of BWP switch (DCI based BWP switch) is performed on multiple CCs simultaneously or over partially overlapping time period.

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs

The delay requirements for simultaneous DCI based BWP switch on multiple CCs in this clause apply only if the timing difference among the first symbol of slot carrying DCI for all CCs is received within the MRTD for inter-band CA as defined in clause 7.6.4.

For DCI-based BWP switch on multiple CCs, after the UE receives BWP switching request, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWPs on the serving cells on which BWP switch on the first DL or UL slot occurs right after a time duration of TMultipleBWPswitchDelay which starts from the beginning of DL slot n, where slot n is slot which UE receives the earliest BWP switching request among CCs on which UE is performing simultaneous DCI-based BWP switching.

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of TMultipleBWPswitchDelay which starts from the beginning of DL slot n except DCI triggering BWP switch on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths on any serving cell.

UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay, which is defined as:

TMultipleBWPswitchDelay = TBWPswitchDelay + N*(D -1)

Where:

-	TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay should be based on the smallest SCS among all SCS values of all involved CCs.

-	D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability [13].

-	N is the number of CCs undergoing simultaneous BWP switch.

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in Clause 8.10 in the new BWP.

If the BWP switch is triggered on multiple CCs simultaneously within DRX active time, and one of the two BWPs on ewach CC in a BWP switching is a dormant BWP [TS 38.321, 7] and the configurations of the two BWPs differ in any other parameter than PDCCH-ConfigCommon or PDCCH-Config, UE shall be able to complete active BWP switching within

-	TMultipleBWPswitchDelay, provided that the BWP switching request is received in any of the first 3 OFDM symbols of a slot in the serving cell on which DCI for dormancy indication is received, or

-	TMultipleBWPswitchDelay + Z provided that the BWP switching request is received after the first 3 OFDM symbols of a slot in the serving cell on which DCI for dormancy indication is received, where Z=1 slot with respect to the SCS of the serving cell on which DCI for dormancy indication is received.

If the BWP switch is triggered on multiple CCs simultaneously within DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 7] and the configurations of the two BWPs differ only in PDCCH-ConfigCommon or PDCCH-Config, the above requriements with TMultipleBWPswitchDelay corresponding to Type 1 UE capability apply.

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs

In non-simultaneous case, the DCI-based BWP switch on multiple CCs is triggered over partially overlapping time period between CCs or multiple CCs in different Cell groups. The delay requirements for non-simultaneous DCI based BWP switch on multiple CCs in this clause apply only if:

-	the timing difference among the first symbol of slot carrying DCI for all CCs involved in non-simultaneous BWP switch is received exceeds the MRTD for inter-band CA as defined in clause 7.6.4, and

-	UE is operating in NR-DC (FR1+FR2), and

-	UE is capable of per-FR gap, and

-	BWP switch does not involve SCS change

For non-simultaneous DCI based BWP switch on multiple CCs, BWP switching delay requirements defined in clause 8.6.2 apply when BWP switching occurs on single CC in the cell group. BWP switching delay requirements defined in clause 8.6.2A.1 apply when simultaneous BWP switching occurs on multiple CCs in the cell group.

<End of Change 1>

<Start of Change 2>

8.2.1.2.15	Interruptions due to SCell dormancy

8.2.1.2.15.1	Interruptions due to SCell dormancy switch

When one SCell in SGC is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on any active serving cell in SCG as defined in clause 8.2.1.2.7, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP.

-	If the UE is not capable of per-FR gap, or if the dormancy switching involves SCS changing, the interruption is allowed to other active serving cell in SCG. If the UE is capable of per-FR gap and the the dormancy switching does not involve SCS changing, UE is allowed to cause interruption to other active serving cells in SCG in the same frequency range wherein the UE is performing dormancy switching.

-	The starting time of interruption is only allowed within the dormancy switching delay as defined in clause 8.6.2.

When more than one SCells in SGC are switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] simultaneously when UE is in DRX active time, the interruption requirements described above apply for dormancy switch on each SCell.

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells in SCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PSCell or any activated SCell with up to [0.5]% probability of missed ACK/NACK.

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells in SCG are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on PSCell or any activated SCell with up to [0.5]% probability of missed ACK/NACK.

<End of Change 2>

<Start of Change 3>

8.2.2.2.12	Interruptions due to SCell dormancy

8.2.2.2.12.1	Interruptions due to SCell dormancy switch

When one SCell is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on any active serving cell as defined in clause 8.2.2.2.5, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP.

-	If the UE is not capable of per-FR gap, or if the dormancy switching involves SCS changing, the interruption is allowed to other active serving cell. If the UE is capable of per-FR gap and the the dormancy switching does not involve SCS changing, UE is allowed to cause interruption to other active serving cells in the same frequency range wherein the UE is performing dormancy switching.

-	The starting time of interruption is only allowed within the dormancy switching delay as defined in clause 8.6.2.

When more than one SCells are switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] simultaneously when UE is in DRX active time, the interruption requirements described above apply for dormancy switch on each SCell.

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on any activated SCell with up to [0.5]% probability of missed ACK/NACK.

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on any activated SCell with up to [0.5]% probability of missed ACK/NACK.

<End of Change 3>

<Start of Change 4>

8.2.3.2.13	Interruptions due to SCell dormancy

8.2.3.2.13.1	Interruptions due to SCell dormancy switch

When one SCell in MCG is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on any active serving cell in MCG as defined in clause 8.2.3.2.7, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP

-	If the UE is not capable of per-FR gap, or if the dormancy switching involves SCS changing, the interruption is allowed to other active serving cell in MCG. If the UE is capable of per-FR gap and the the dormancy switching does not involve SCS changing, UE is allowed to cause interruption to other active serving cells in MCG in the same frequency range wherein the UE is performing dormancy switching.

-	The starting time of interruption is only allowed within the dormancy switching delay as defined in clause 8.6.2.

When more than one SCells in MGC are switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] simultaneously when UE is in DRX active time, the interruption requirements described above apply for dormancy switch on each SCell.

8.2.3.2.13.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

8.2.3.2.13.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on PCell or any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

<End of Change 4>

<Start of Change 5>

8.2.4.2.11	Interruptions due to SCell dormancy

8.2.4.2.11.1	Interruptions due to SCell dormancy switch

When one SCell is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on any active serving cell as defined in clause 8.2.4.2.5, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP

-	If the UE is not capable of per-FR gap, or if the dormancy switching involves SCS changing, the interruption is allowed to other active serving cell. If the UE is capable of per-FR gap and the the dormancy switching does not involve SCS changing, UE is allowed to cause interruption to other active serving cells in the same frequency range wherein the UE is performing dormancy switching.

-	The starting time of interruption is only allowed within the dormancy switching delay as defined in clause 8.6.2.

When more than one SCells are switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] simultaneously or over partially overlapping period when UE is in DRX active time, the interruption requirements described above apply for dormancy switch on each SCell.

8.2.4.2.11.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PSCell and any activated SCell in SCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

When one or more SCells in SCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PCell and any activated SCell in MCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

8.2.4.2.11.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells in MCG are in dormancy, the UE is allowed due to RRM measurements on the SCell(s):

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PSCell and any activated SCell in SCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

When one or more SCells in SCG are in dormancy, the UE is allowed due to CQI measurements on the SCell(s):

-	an interruption on PSCell and any activated SCell in SCG with up to [0.5]% probability of missed ACK/NACK.

-	an interruption on PCell and any activated SCell in MCG with up to [0.5]% probability of missed ACK/NACK for synchronous NR-DC.

-	an interruption on PCell and any activated SCell in MCG with up to [1]% probability of missed ACK/NACK for asynchronous NR-DC.

<End of Change 5>

<Start of Change 6>

8.2.4.2A	Void

<End of Change 6>