3GPP TSG-RAN4 Meeting #96-e	R4-2011158

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1083 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for measurement requirements for PRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-07-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRS-RSRP measurement requirements are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define PRS-RSRP measurement requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for the WI objective are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.3	PRS-RSRP measurements

9.9.3.1	Introduction

If UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] with prsrsrpReq in nr-RequestedMeasurements-r16 set to 1, UE shall measure and report PRS-RSRP defined in TS38.215 [4], and meet the requirements defined in clause 9.9.2.

If UE has received NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with prsrsrpReq in nr-RequestedMeasurements-r16 set to 1, UE shall measure and report PRS-RSRP defined in TS38.215 [4], and meet the requirements defined in clause 9.9.4.

If UE has received NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], UE shall measure and report PRS-RSRP defined in TS38.215 [4], and meet the requirements defined in remaining of clause 9.9.3.

9.9.3.2	Requirements applicability

The requirements in clause 9.9.3 apply, provided:

-	PRS resource being measured is detectable.

A PRS resource shall be considered detectable, provided:

-	PRS-RSRP related side conditions given in clauses 10.X.Y and 10.X.YY for FR1 and FR2, respectively, are met for a corresponding Band.

9.9.3.3	Measurement Reporting Requirements

When the NR-DL-AoD Assistance Data are provided via NR-DL-AoD-ProvideAssistanceData message, the UE shall be able to detect and measure PRS-RSRP from configured PRS resources for configured TRPs on configured PRS frequency layers according to the reported UE capabilities in NR-DL-AoD-ProvideCapabilities message, including the reference resource(s), within TPRS-RSRP,total ms as given below:

TPRS-RSRP, total=iCSSFi*TPRS-RSRP,i

where i is the index of PRS frequency layer, CSSFi is scaling factor for gap sharing between PRS layer i and RRM measurements, and TPRS-RSRP,i is the measurement period for PRS frequency layer i, and

TPRS-RSRP,i=Nproc,i*Nbeam,i*4*Teffect,i+Tproc,i

where

Nproc,i is the scaling factor for UE buffering and processing capability for PRS layer i defined as

Nproc,i=LPRS,iNi*NPRS,islotNi'/Teffect,iTi
where LPRS,i is the length of the aggregate PRS occasion of PRS layer i that falls within the effective measurement time of a MG, NPRS,islot is the maximum number of PRS resources per slot among all slots within LPRS,i, and the requirements do not apply for resources spanning over two sampling periods of Ni;

Nbeam,i is the scaling factor for Rx beam sweeping, and Nbeam=1 if PRS layer i is in FR1 and Nbeam=8 if PRS layer i is in FR2;

Teffect,i is the effective periodicity of the PRS layer i defined as

Teffect,i=TiTPRS,i'*TPRS,i'

where TPRS,i' is the least common multiple of TPRS,i and MGRP, TPRS,i is the maximum PRS resource periodicity among all PRS resources of PRS layer i, and requirements apply provided that PRS occasions are not dropped due to collision with SSB;

Tproc,i is the sampling and processing time for PRS layer i defined as

Tproc,i=Ti*NPRS,islotNi'/NPRS,islotNi'/Teffect,iTi

where Ni is the duration of DL PRS symbols in units of ms a UE can process every Ti ms assuming maximum DL PRS bandwidth in MHz, and Ni' is the maximum number of DL PRS resources that UE can process in a slot. Ni, Ti and Ni' are reported UE capability.

The time TPRS-RSRP,total starts from the first subframe of the PRS positioning occasion closest in time after both the NR-DL-AoD-RequestLocationInformation and the assistance data in the NR-DL-AoD-ProvideAssistanceData message as specified in TS 37.355 [34], are delivered to the physical layer of the UE.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy as specified in the clauses 10.X.Y and 10.X.YY for FR1 and FR2, respectively.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing positioning measurement session. The UE shall also meet the PRS-RSRP measurement and accuracy requirements. However in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+K*Teffect+THO   ms

where

K is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all PRS layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

9.9.3.3.1	Measurement reporting delay

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported DL RSTD measurement values contained in periodically triggered measurement reports shall be based on the measurement report mapping requirements specified in sections 10.1.24.3.

<End of Change 1>