3GPP TSG-RAN4 Meeting #96-e	R4-2011171

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1086 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on multiple SCell activation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2020-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some issues in the newly introduced multiple SCell activation requirements. - The applicability in terms of number of SCells is already defined in clause 3.6.2 of 38.133, and it does not need to be repeated specifically for multiple SCell activation, which will be difficult for future maintenance. - For FR1 known SCell with measurement cycle <=160ms, the activation time should be based on first common SSB for all SCells to-be activated in the same band, which is TFirstSSB_MAX_multiple_scells. - For FR1 unknown SCell, if the SCell is contiguous to an intra-band active or known SCell and other conditions are met, cell detection can be skipped. This is not reflected in current requriements. - N2 is not used anywhere in the requirements, so it should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the multiple SCell activation requirements for above issues. Some other small clarifications are also made to make the requirements more clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are some issues in multiple SCell activation requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells

The requirements in this section shall apply for the UE configured with more than one SCells.

In EN-DC, NE-DC, standalone NR, or in one CG of NR-DC, the requirements in this section shall apply when the following conditions are met:

-	UE only receives one single MAC command for multiple SCell activation within the activation period defined in this section

-	in each single CG, there are no other ongoing SCell activation, deactivation, addition or release before activation is completed for all the SCells activated by the single MAC CE in this section, and

-	in EN-DC and NE-DC, there are no ongoing E-UTRAN SCell activation, deactivation, addition or release before multiple SCell activation is completed in this section, and

-	any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band

In two CGs of NR-DC, the requirements in this section shall apply when the following conditions are met:

-	UE receives one MAC command per CG for multiple SCell activation within the activation period defined in this section, and

-	UE supports per-FR measurement gap capability, and

-	any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band

The delay within which the UE shall be able to activate the deactivated SCell with other downlink to-be-activated SCell(s) depends upon the specified conditions.

Upon receiving SCell activation command in slot n for more than one SCell, for each of the to-be-activated SCell, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot  n+THARQ+Tactivation_time_multiple_scells+TCSI_ReportingNR slot length, where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario.

If the SCell is known and belongs to FR1 and the SCell measurement cycle is equal to or smaller than 160ms, Tactivation_time_multiple_scells is:

-	TFirstSSB_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 known Scell with the SCell measurement cycle larger than 160ms but does not have any parallel to-be-activated SCell which is FR1 unknown SCell.

-	TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown Scell

-	otherwise, TFirstSSB_MAX_multiple_scells + 5ms.

If the SCell is known and belongs to FR1 and the SCell measurement cycle is larger than 160ms, Tactivation_time_multiple_scells is:

-	TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown Scell

-	otherwise, TFirstSSB_MAX_multiple_scells + Trs + 5ms

If the SCell is unknown and belongs to FR1, Tactivation_time_multiple_scells is:

-	 TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs +5ms, if the SCell is not counted in N1

-	 otherwise, (TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells)+Trs*N1 +Trs +5ms

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in section 8.3.2.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE is not provided with any SMTC for the target SCell, Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in section 8.3.2

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in section 8.3.2.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time_multiple_scells is same as 	single SCell activation delay requirement as defined in section 8.3.2.

If the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_multiple_scells is:

-	3 ms + max(Tuncertainty_MAC +TFineTiming + 2ms, Tuncertainty_SP). Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time. Tuncertainty_SP=0 if UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_multiple_scells is:

-	max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ). Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time

Where,

N1 is the number counting for parallel FR1 unknown to-be-activated SCell(s) only except the ones which fulfilled the following conditions:

-	contiguous to an active serving cell in the same band, or to a known SCell in the same band being activated by the same MAC PDU, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the unknown SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 known cell or contiguous FR1 active serving cell, and

-	its SSB DL Tx beam is same as the corresponding SSB DL Tx beam at the same SSB position of contiguous FR1 known cell or contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 known cell or contiguous FR1 active serving cell

However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	contiguous to an active serving cell in the same band, or to a known SCell in the same band being activated by the same MAC PDU, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the unknown SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of FR1 known cell or FR1 active serving cell, and

-	its SSB DL Tx beam is different from the corresponding SSB DL Tx beam at the same SSB position of FR1 known cell or FR1 active serving cell, and

-	its SMTC offset is same as the one of FR1 known cell or FR1 active serving cell

TSMTC_MAX_multiple_scells:

-	In FR1, in case of intra-band SCell activation, TSMTC_MAX_multiple_scells is the longest SMTC periodicity between active serving cells and SCells being activated on the same band provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX_multiple_scells is the longest SMTC periodicity of SCells being activated on the same band.

-	In FR2, TSMTC_MAX_multiple_scells is the longest SMTC periodicity between active serving cells and SCell(s) being activated in FR2 intra-band CA.

-	TSMTC_MAX_multiple_scells is bounded to a minimum value of 10ms.

TFirstSSB_MAX_multiple_scells: is the time to the end of the first complete SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCells being activated are transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

Trs, TFineTiming, TL1-RSRP, measure, TL1-RSRP, report, Tuncertainty_MAC, Tuncertainty_RRC, TRRC_delay, and Tuncertainty_SP is defined in section 8.3.2.

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

The condition of known SCell in FR1 or FR2 is defined in section 8.3.2.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX_multiple_scell follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is same as single SCell activation requirement in section 8.3.2.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

<End of Change 1>