3GPP TSG RAN WG4 Meeting #96-e	R4-2011252

Electronic Meeting, August 17-28, 2020

Agenda Item:	10.20.2

Source: 	Ericsson, SoftBank

Title:	Analysis of RRM core requirements for FR2 FWA UE power class

Document for:	Discussion

- Introduction

The RF requirements for FWA UE with the maximum EIRP of 43dBm and maximum TRP of 23dBm are being discussed under the Rel-17 WI: NR_FR2_FWA_Bn257_Bn258 [1].

RAN4 has not yet finalized the UE RF requirement for the new power class. One main contentious issue is whether to introduce one or two power classes [2]. In any case according to RRM agreements the RRM core requirements will reuse the requirements defined for FR2 PC1 which is also for FWA UE [3].

# Analysis of RRM requirements

In the last RAN4 meeting the following was agreed according to the RRM WF [3]:

The following RRM core requirements will be impacted if FR2 FWA UE is not defined as power class 1:

- FR2 measurement requirements in idle mode

- Known condition for FR2 SCell activation requirements

- FR2 intra- and inter-frequency measurement requirements

We agree with the above conclusions. However, since the FR2 FWA UE can also be capable of LTE so inter-RAT FR2 requirements in idle state and inter-RAT FR2 requirements in connected states in TS 36.133 shall also be impacted to enable mobility between LTE and NR FR2. Furthermore, the FR2 FWA UE can also be configured to operate in EN-DC. Therefore, inter-RAT FR2 requirements for EN-DC in TS 36.133 shall also be impacted. For requirements in TS 36.133 the existing FR2 PC1 requirements can also be reused.

- Summary

Based on the analysis in this paper following are the main observation and proposals:

- Observation 1: UE supporting new FR2 FWA power class may also be capable of LTE.

- Observation 2: UE supporting new FR2 FWA power class may also be configured in EN-DC.

- Proposal 1: If FR2 FWA UE is not defined as power class 1, then in addition to the agreement in [3], the core requirements for new FR2 FWA power class shall also be defined for the following inter-RAT scenarios in TS 36.133:

  - Inter-RAT FR2 measurement requirements in idle state.

  - Inter-RAT FR2 measurement requirements in RRC connected state and

  - Inter-RAT FR2 measurement requirements in EN-DC

- Proposal 2: The existing inter-RAT requirements defined for PC1 should be re-used for the new FWA UE power class

The CRs to 38.133 [4] and 36.133 [5] are provided assuming new FR2 power class 5 is introduced for the FRA. The CRs can be revised based on final agreements osn the FWA power class in the RF group.

- References

- RP-201215, FR2 FWA UE with TRP of 23dBm for n257 and n258.

- R4-2008921, WF on FR2 FWA evaluations, Qualcomm

- R4-2008663, WF on FR2 new FWA UE RRM requirements, Huawei, HiSilicon

- R4-2011253, RRM core requirements for FR2 FWA UE power class in 38.133, Ericsson, Softbank

- R4-2011254, RRM core requirements for FR2 FWA UE power class i 36.133, Ericsson, SoftBank