3GPP TSG-RAN WG4 Meeting #96-e	R4-2011316

Electronic Meeting, 17 Aug. – 28 Aug., 2020

Source:	ZTE

Title:	Remaining open issues on measurement requirements for CSI-RS based L3 mobility

Agenda item:	7.14.1.4

Document for:	Discussion

1. Introduction

In the RANP#88-e meeting, following issues were captured in the exception sheet for WID of RRM requirements for CSI-RS based L3 measurement [1].

- UE capability - New UE capability on the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell - New UE capability for minimum separation between two slots - Note: can be handled with low priority - Cell identification requirement - whether to introduce 2 different requirements for with index and without index - the tuning time of inter-frequency GAP - time-domain restriction on CSI-RS resources configuration - note: CMTC for CSI-RS L3 measurement is out of scope - Scheduling restriction on the following cases - when UE is not able to support mixed numerology of data and CSI-RS L3 mobility - when UE performs CSI-RS intra-frequency measurements in a TDD band - when UE performs RX beam sweeping - The collision case between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell - CSSF for CSI-RS based measurement within measurement gap and outside of measurement gap. - synchronization assumption for CSI-RS measurement

In this contribution we further provide our views on UE measurement requirements for CSI-RS based L3 mobility.

2. Discussion

New UE capability

There was proposal to introduce new UE capability on the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell. However it needs to be clarified what the use case of the UE capability. Firstly it should only be applicable for CSI-RS based and SSB based intra-frequency measurement without gaps. The UE can perform measurement on SSB and CSI-RS resources simultaneously. To take use of this UE measurement capability requirements should be specified by taking the UE capability into consideration, which means both SSB based RRM requirements and CSI-RS based RRM requirements need to be revised or defined with further consideration. We don’t think this would be the typical use case. Moreover it is simpler to define SSB based measurements and CSI-RS based measurements as independently as it was agreed that there are 1 SSB intra-frequency layer and [1] CSI-RS intra-frequency layer per serving cell. So there is no need to have this UE capability in Rel-16.

Proposal 1: Not to introduce new UE capability on the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell.

Cell identification requirement

To measure neighbor cell CSI-RS resources, UE need to know the SFN and frame timing information of neighbor cell. When deriveSSB-IndexFromCell is indicated by high layer, UE can derive necessary timing information based on serving cell. In this case there is no need to detect SSB index. When deriveSSB-IndexFromCell is not indicated by high layer, then UE has to decode PBCH to obtain necessary timing information. Therefore it is necessary to define requirements for both with and without SSB index detection.

Proposal 2: Introduce 2 different requirements for both with and without SSB index detection.

Regarding the RF retuning time for CSI-RS based measurement within gap, there was concern that exiting RF retuning time for SSB based measurement may not be feasible for some UE implementation. Extra retuning time is needed due to CSI-RS measurement processor and SSB measurement processor may not be the similar implementation. We understand the implementation issue. It is preferable to have the same RF retuning time. If it cannot be reused it would be fine to define UE capability for the retuning time.

Proposal 3: Introduce UE capability if RF retuning time for SSB based measurement within gap cannot be reused.

It was agreed to introduce timing domain restriction on CSI-RS resources configuration. For inter frequency measurement, which is conducted within measurement gap, it is naturally to confine the CSI-RS resources within the gap. Considering RF retuning time, the CSI-RS resources should be further confined within the gap excluding RF retuning time. For intra frequency measurement, which is conducted without gap, there is generally no need to confine time domain configuration. To reduce UE complexity, it is reasonable to confine the CSI-RS within certain duration. We propose to confine CSI-RS within a 5ms window, which is the maximum STMC duration.

Proposal 4: CSI-RS resources for inter inter-frequency measurement with measurement gaps are confined within measurement gap length excluding RF retuning timing.

Proposal 5: CSI-RS resources for inter intra-frequency measurement without measurement gaps are confined within a 5ms window.

Scheduling restriction

Scheduling restriction was discussed on following aspects.

  - FFS when UE is not able to support mixed numerology of data and CSI-RS L3 mobility, UE is not expected to transmit or receive on [2] data OFDM symbols impacted by CSI-RS resource symbols to be measured

  - FFS when UE performs CSI-RS intra-frequency measurements in a TDD band, UE is not expected to transmit or receive on [2] data OFDM symbols impacted by CSI-RS resource symbols to be measured

  - FFS when UE performs RX beam sweeping, UE is not expected to transmit or receive [1] data symbol before and after CSI-RS symbol to be measured due to Rx beam sweeping.

The first issue is about mixed numerology. Since intra frequency measurement is that SCS of all the cells are the same. The SCS of CSI-RS resources of serving cell and is the same as SCS of serving cell data. So there should be no such mixed numerology case for CSI-RS based measurement.

Proposal 6: Mixed numerology case is not valid for CSI-RS based intra frequency measurement.

The second issue is about scheduling restriction during intra frequency measurement. This may not need to be the TDD band. Depending on UE implementation of CSI-RS resources of intra-frequency cells, it could be for any deployment scenarios.

Before determining this scheduling restriction requirements the timing synchronization assumption for CSI-RS based measurement should be decided firstly. It is related to how UE will measure CSI-RS resources if multiple CSI-RS resources from multiple cells are in the same OFDM symbol. Three options was left to decide in the last meeting.

- For CSI-RS measurement with associated SSB,

  - Option 1: RAN4 to address the issue of timing difference between the arrival of the CSI-RS and UE’s FFT timing in the performance part.

  - Option 2: the corresponding timing of CSI-RS resources should be assume the same as the timing of the cell given by the cellId of the CSI-RS resource configuration.

  - Option 3: introduce the UE capability to differentiate the following 2 types of UEs.

    - Type 1: UE supporting using only single timing for CSI-RS measurement per frequency layer

    - Type 2: UE supporting using timing of associated SSB for respective CSI-RS measurement

Option 1 is aligned with WID objective.

- Assumptions on Synchronization

  - Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements.

Option 2 is aligned with procedure for CSI-RS mobility in TS 38.214. However the timing in option 2 should be in slot level

- If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration.

Option 1 and Option 2 are the implementations for different CSI-RS configurations and network deployment in our view. It is not necessary to be choosing one over another.

If the CSI-RS resources from multiple intra frequency cells are overlapped in the same symbol, there may be different implementations.

If the time difference is within certain limit, e.g. half CP length, then UE can measure all the CSI-RS resources by using of FFT, based on the timing of anyone of the cell. If there is CSI-RS resources on serving cell, then serving cell timing can be used. In this case no scheduling restriction should be allowed.

If the time difference is larger than certain limit, e.g. half CP length, then UE should measure CSI-RS resources of the cell based on the timing of cell estimated by associated SSB, otherwise the CSI-RS measurement accuracy may not meet the requirements. In this case scheduling restriction is needed since the FFT window is based on neighbor cell and there is no guarantee of serving cell performance.

In asynchronous network, it is not possible for UE to measure neighbor cell CSI-RS resources based on serving cell timing. UE has to measure CSI-RS resources of the neighbor cell based on the timing of the cell estimated by associated SSB.

It may not be possible to address timing difference issue in performance part. For async network, the measurement performance can be unacceptable given the time different could be half of the symbol. If option 1 is selected, then there is restriction on network deployment. Even for sync network, the time difference may be too large that the measurement results cannot not be used for mobility.

Unless there is agreement to limit deployment scenarios in Rel-16, otherwise option 2 would be used as synchronization assumption to define RRM requirements for CSI-RS based L3 mobility.

Proposal 7: The corresponding timing of CSI-RS resources should assume the same as the timing of the cell given by the cellId of the CSI-RS resource configuration

Then for scheduling during intra frequency measurement, restrictions on CSI-RS resources to be measured should be defined. There are two symbols that will be impacted but it is unknown to network whether the symbol before or after CSI-RS resource symbol will be impacted. So from requirements perspective 3 symbols will be impacted.

Proposal 8: UE is not expected to transmit or receive on [2] data OFDM symbols impacted by CSI-RS resource symbols to be measured during intra frequency measurement.

Since it was agreed that UE is not required to perform CSI-RS based L3 measurement when associated SSB is not QCLed with CSI-RS for FR2, so Rx beaming is not needed for CSI-RS based measurement in FR2.

Proposal 9: Rx beam sweeping is not needed in FR2.

Collision between L1 and L3 measurement

Two options were provided in the last meeting.

- Option 1: Do not define CSI-RS measurement requirements for the collision case.

- Option 2: Network should configure L1 measurement resource to avoid collision with CSI-RS L3 measurement resource of neighbour cell.

It may not be possible to avoid the collision from network side. So in Rel-16 it would be fine not to define CSI-RS measurement requirements for the collision case.

Proposal 10: Do not define L3 CSI-RS measurement requirements if there is collision with L1 CSI-RS measurements.

CCSF

Two options were provided in the WF [2].

- Option 1:

  - All CSI-RS in the same MO should follow the same time-domain relation with gap, e.g., either fully overlapped with gap, partially overlapped with gap or fully non-overlapped with gap

- Option 2:

  - If additional dedicated searcher is assumed for CSI-RS measurement, no impact on existing CSSF defined for SSB based measurement specified in 38.133.

  - Otherwise, the CSSFs for FR1/FR2 SCC shall be updated by considering the CSI-RS based intra-frequency and inter-frequency measurement without gap and within gap respectively.

The SSB based and CSI-RS based measurement include two steps, cell search and measurement. For cell search the SSB based measurement and CSI-RS based measurement can be done only once if associated SSB is on the frequency for SSB based measurement, e.g. SSB based measurement and CSI-RS based measurement are configured in the same MO. This could save cell search time for CSI-RS based measurement. For other cases the cell search is independent. For measurement of SSB or CSI-RS resources, it cannot be done simultaneously. Dedicated processor for CSI-RS measurement is needed. To simplify requirements dedicated searcher is assumed for CSI-RS measurement.

Proposal 11: dedicated searcher is assumed for CSI-RS measurement.

3. Conclusion

In this contribution, we further provided our views on on UE measurement requirements for CSI-RS based L3 mobility. Based on the observations following proposals are present.

Proposal 1: Not to introduce new UE capability on the simultaneous reception of CSI-RS of neighbour cell and SSB of serving cell.

Proposal 2: Introduce 2 different requirements for both with and without SSB index detection.

Proposal 3: Introduce UE capability if RF retuning time for SSB based measurement within gap cannot be reused.

Proposal 4: CSI-RS resources for inter inter-frequency measurement with measurement gaps are confined within measurement gap length excluding RF retuning timing.

Proposal 5: CSI-RS resources for inter intra-frequency measurement without measurement gaps are confined within a 5ms window.

Proposal 6: Mixed numerology case is not valid for CSI-RS based intra frequency measurement.

Proposal 7: The corresponding timing of CSI-RS resources should assume the same as the timing of the cell given by the cellId of the CSI-RS resource configuration

Proposal 8: UE is not expected to transmit or receive on [2] data OFDM symbols impacted by CSI-RS resource symbols to be measured during intra frequency measurement.

Proposal 9: Rx beam sweeping is not needed in FR2.

Proposal 10: Do not define L3 CSI-RS measurement requirements if there is collision with L1 CSI-RS measurements.

Proposal 11: dedicated searcher is assumed for CSI-RS measurement.

4. References

- RP-201340	Rel-16 WI exception for Core part: RRM requirement for CSI-RS based L3 measurement

- R4-209009	WF on CSI-RS based L3 measurement capability and requirements, OPPO