3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2011381

# Electronic Meeting, 17-28 Aug., 2020

Agenda item:	7.3.7.1

Source:	Qualcomm Inc.

Title:	On NR V2X Demod Requirement

Document for:	Discussion

# Introduction

In this discussion paper, we provide our view on the scope and simulation assumptions for NR V2X demod requirement

# Discussion

## Requirement scope and applicability rule

Based on LTE demod requirement scope, we propose the following requirement and applicability rule based on UE capability [1]:

| Requirement | Applicable if UE supports the following capability |
| --- | --- |
| PSCCH and PSSCH demodulation without 256QAM reception capability | UE with support of Receiving NR sidelink |
| PSSCH and PSCCH decoding capability | UE with support of Receiving NR sidelink |
| PSSCH demodulation with 256QAM reception capability | UE with support of receiving 256QAM, the 64QAM demod test can be skipped if UE fulfills this requirement |
| PSBCH demod | UE can receive S-SSB in NR sidelink |
| Demod for gNB as synchronization source | UE can transmit or receive NR sidelink based on the synchronization to an gNB |
| SDR | UE supporting concurrent operation band for NR Uu and NR SL |
| HARQ soft combining test | UE with support of Receiving NR sidelink but not concurrent operation for NR Uu and NR SL |
| PSFCH detection | UE with support of Receiving NR sidelink |

Table 2-1 Applicability rule for demodulation requirements

The above list most follows LTE V2X demod requirement, and we explain the difference below:

- The soft buffer test is replaced by HARQ soft combining test. This is following NR Uu change compared to LTE, since the UE capability is specified in terms of number of HARQ process, the requirement is modified accordingly.

- Since 256QAM transmission is an optional capability, to align transmission and reception capabilities, we propose to set 256QAM reception as an optional capability. 256QAM is mostly applied in unicast and group-cast scenarios instead of broadcast safety information, in which packet size is small and requires high reliability, hence making it optional provides more flexibility for UE implementation selection without impact network operation feasibility. Given that 256QAM reception is optional capability, a separate demod test is proposed for 256QAM.

Proposal 1: Replace soft buffer test by HARQ buffer soft combining test, following NR Uu.

Proposal 2: 256QAM reception is an optional UE capability.

Proposal 3: Adapt applicability rule for demod requirement as Table 2-1.

## General setting for demod requirement

Since 30kHz is mandatory for NR-V2X, we propose to configure 30kHz SCS with 40MHz BW for all the demod tests. 14 symbols are configured for all the tests. In practice, UE typically use one subchannel per transmission and small subchannel size to accommodate more UEs, hence we propose 10RB allocation for demod test unless specified otherwise.

Since there are high speed scenarios to be tested, high speed with large delay spread is not feasible for maintaining a link with good quality, hence among the options listed in 38.101-4 B.2.1.1, we propose to use TDL-A 30ns. Doppler spread is determined in each test separately depending on the scenario considered.

Proposal 4: Common demod test configurations are listed in the following table (subject to change for any specific tests)

| Bandwidth | 40MHz |
| --- | --- |
| SCS | 30kHz |
| Allocation size | 10RB |
| Propagation condition | TDL-A 30ns in 38.101-4 B.2.1.1 |

Table 2-2 Common demod test configurations

## PSSCH demod test

In LTE, there are two categories of requirement: low speed with high MCS, and high speed with low MCS. For NR, we follow this methodology to propose the following simulation assumptions for 3 tests: low speed 64QAM, low speed 256QAM, high speed QPSK. Note that 256QAM is applicable only to UE supports 256QAM reception, and if UE passed 256QAM low speed test, the 64QAM low speed test doesn’t apply to this UE. Simulation results presented below the table show that 10% BLER can be achieved within reasonable SNR range.

| Test num. | Bandwidth /SCS | PSSCH Reference channel | Propagation condition | Number of DMRS | PSSCH BLER (%) | Antenna configuration | PSFCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40MHz/30kHz | 64QAM, 0.5 (MCS 21), 10RB | TDL-A 30ns 30kmph (162Hz) | 2 | 10 | 1x2 Low | Not configured |
| 2 | 40MHz/30kHz | QPSK, 0.4 (MCS 5), 10RB | TDL-A 30ns 500kmph (2700Hz) | 4 |  |  |  |
| 3 | 40MHz/30kHz | 256QAM, 0.67 (MCS 20), 10RB | TDL-A 30ns 30kmph (162Hz) | 2 |  |  |  |
| Note: timing offset = CP/2-12Ts, frequency offset = 600Hz |  |  |  |  |  |  |  |

Table 2-3 PSSCH demod simulation assumption

Figure 2-1 PSSCH demod performance with 64QAM MCS table

Figure 2-2 PSSCH demod in high speed with 64QAM MCS table

Figure 2-3 PSSCH demod with 256QAM MCS table

For PSSCH demod with gNB as synchronization source test, we propose the following simulation assumption based on LTE counterpart:

| Bandwidth/SCS | PSSCH Reference channel | Propagation condition | Number of DMRS | Timing and frequency offset | PSSCH BLER (%) | Antenna configuration | PSFCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 40MHz/30kHz | QPSK, 0.3 (MCS 4), 10RB | TDL-A 30ns 30kmph 2 DMRS (162Hz) | 2 | CP/2-24Ts, 1300Hz | 10 | 1x2 Low | Not configured |

Table 2-4 PSSCH demod with gNB as synchronization source simulation assumption

Proposal 5: Use Table 2-3 and Table 2-4 as PSSCH demod simulation assumptions.

## PSCCH and PSBCH demodulation test

Following LTE test setting, we propose simulation assumption for PSCCH and PSBCH decoding tests in the following:

PSCCH:

| Bandwidth /SCS | PSCCH Reference channel | Propagation condition | Probability of missed PSCCH (%) | Antenna configuration |
| --- | --- | --- | --- | --- |
| 40MHz/30kHz | 2 symbol, 10 RB, SCI-1 payload length 28 bits | TDL-A 30ns 250kmph (1350Hz) | 1 | 1x2 Low |
| Note: timing offset = CP/2-12Ts, frequency offset = 600Hz |  |  |  |  |

Table 2-5 PSCCH demod simulation assumptions

Note that SCI-1 payload size is calculated from the following table, to align with PSSCH demod test. Simulation results presented below the table show that 10% BLER can be achieved within reasonable SNR range.

| Field | Number of bits |  |
| --- | --- | --- |
| Priority | 3 |  |
| Frequency resource assignment | 6 | #subchannel = 10, max num reserve = 2 |
| Time resource assignment | 5 | max num reserve = 2 |
| Resource reservation period | 0 | Multi reserve resource is not configured |
| DMRS pattern | 2 | 14 symbols, 3 DMRS pattern |
| 2nd stage SCI format | 2 |  |
| Beta offset indicator | 2 |  |
| Number of DMRS ports | 1 |  |
| MCS | 5 |  |
| Reserved | 2 | NumReserveBits = 2 |
| Total | 28 |  |

Figure 2-4 PSCCH simulation results

PSBCH:

| Bandwidth /SCS | PSBCH Reference channel | Propagation condition | Probability of missed PSBCH (%) | Antenna configuration |
| --- | --- | --- | --- | --- |
| 40MHz/30kHz | 8 symbols, 40bits payload | TDL-A 30ns 30kmph (162Hz) | 1 | 1x2 Low |

Table 2-6 PSBCH demod simulation assumptions

Proposal 6: Use Table 2-5 and Table 2-6 as PSCCH and PSBCH demod simulation assumptions.

## HARQ soft combining test

The following HARQ soft combining test is defined for NR Uu:

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | 1.1 |

Table 2-7 NR Uu HARQ soft combining test

Following the MCS used in NR Uu HARQ soft combining test, we propose the following simulation assumption for V2X, and number of HARQ process depends on UE capability:

| Bandwidth /SCS | Modulation format and code rate | Propagation condition | PSSCH BLER | Antenna configuration | PSFCH |
| --- | --- | --- | --- | --- | --- |
| 40MHz/30kHz | 16QAM, 0.5 (MCS 13), 10RB | AWGN | 5% | 1x2 Low | Not configured |

Table 2-8 NR V2X HARQ buffer soft combining simulation assumptions

Since the purpose of this test is to verify how many HARQ process UE can support, PSFCH is not configured in this test and TE blindly retransmit all the TBs regardless of successful decoding or not. To verify that UE perform recombining on the number of HARQ process it claims to support, the propagation condition and target (overall) BLER should be chosen so that most of the TBs fail first Tx and passes the ReTx. Instead of using 30% throughput as reference condition and fading propagation condition designed for unicast in NR Uu, we propose to use LTE settings, AWGN and 5% BLER, as a starting point, and adjustment can be made based on simulation results. With AWGN, channel is flat in frequency, hence we expect uniform average BLER across all RBs with high probability. Channel is also flat in time domain, HARQ combining with ReTx is likely to get around 3dB gain, hence with small but non-zero BLER, we expect first Tx to fail uniformly, and ReTx pass with high probability.

Number of active SL UEs is determined by the number of HARQ process UE can support, the active SL UE configuration (besides BW, SCS, RB allocation and MCS) is in the following table:

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i ≤ n (Note 4) |  |
| Sidelink UE i, 0 ≤ i ≤ n | Sidelink Transmissions |  | PSCCH + PSSCH |  |
|  | Time gap between initial transmission and retransmission | Subframe | n (Note 4) |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Antenna configuration |  | 1x2 |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: 15 sidelink UEs should transmit one by one circularly for every subframe. Note 4: n is the number of HARQ process UE can support |  |  |  |  |

Table 2-9 Active SL UE configuration

Proposal 7: Use Table 2-8 and Table 2-9 as simulation assumptions for HARQ buffer soft combining test.

## PSCCH and PSSCH decoding capability and SDR

For the decoding capability test, LTE test setting can be applied to NR with appropriate modifications as below. Reference condition for this test is PSCCH BLER <=1%

| Parameter |  | Unit |  |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | Single Rx pool 10 subchannels, 10RB subchannel size |
| Sidelink UE i, 0 ≤ i ≤ 9 (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | QPSK, 0.3 (MCS 4), 10RB |
|  | PSCCH RMC |  | The same as PSCCH decoding |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Cyclic shift for PSCCH DMRS is randomly selected between {0, 4, 8} for each PSCCH transmission. Note 4: Each UE occupies one subchannel, hence all the subchannels are filled by the 10 active SL UEs. |  |  |  |

Table 2-10 PSCCH and PSSCH decoding capability test configuration

Note that peak MCS is used in PSSCH RMC, since soft buffer test (with high MCS) is replaced by HARQ buffer soft combining test with mid MCS.

For sustained data rate test applying to UE supports concurrent NR SL and NR Uu operation, the NR SL configuration follows Table 2-10, and NR Uu configuration follows clause 5.5.1 in 38.101-4.

Proposal 8: Use Table 2-10 as simulation assumption for PSCCH and PSSCH decoding capability test.

## PSFCH detection requirement

Since PSFCH signal is designed based on PUCCH format 0, we propose the following simulation assumptions based on NR Uu. Note that we propose to have 2 cyclic shift on the same RB, to test UE’s ability to detect overlapping PSFCH.

| Parameter |  |
| --- | --- |
| Number of information bits | 1 |
| Number of PRBs | 1 |
| Number of cyclic shift pairs | 2 |
| PSFCH period | 1 |
| MinTimeGapPSFCH | 2 |
| Hopping ID | 0 |
| Propagation condition | TDL-A 30ns 30kmph (162Hz) |
| Antenna configuration | 1x2 Low |

Table 2-11 PSFCH detection requirement

Proposal 9: Use Table 2-11 as simulation assumption for PSFCH detection.

# Conclusions

Proposal 1: Replace soft buffer test by HARQ buffer soft combining test, following NR Uu.

Proposal 2: 256QAM reception is an optional UE capability.

Proposal 3: Adapt applicability rule for demod requirement as Table 2-1.

Proposal 4: Common demod test configurations are listed in the following table (subject to change for any specific tests)

| Bandwidth | 40MHz |
| --- | --- |
| SCS | 30kHz |
| Allocation size | 10RB |
| Propagation condition | TDL-A 30ns in 38.101-4 B.2.1.1 |

Proposal 5: Use Table 2-3 and Table 2-4 as PSSCH demod simulation assumptions.

Proposal 6: Use Table 2-5 and Table 2-6 as PSCCH and PSBCH demod simulation assumptions.

Proposal 7: Use Table 2-8 and Table 2-9 as simulation assumptions for HARQ buffer soft combining test.

Proposal 8: Use Table 2-10 as simulation assumption for PSCCH and PSSCH decoding capability test.

Proposal 9: Use Table 2-11 as simulation assumption for PSFCH detection.

# References

[1] R1-2005110