3GPP TSG-RAN WG4 Meeting #96-e	R4-2011415

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0247 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for introduction of EESS protection into additional spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For EESS protection, the following were agreements in R4-2009141: - -10dBm/100MHz and 1dBm/200MHz protection requirements are specified with NS_202 for n257 and n258 - 1dBm/200MHz protection requirements is specified with NS_203 for n258 - -5dBm/200MHz protection requirements is specified with NS_204 for n257 and n258 - 7dBm/1GHz and -13dBm/MHz are specified with NS_205 for n260. - Explicit signaling for a UE to report newly supported NS value(s) for a legacy band to the network (reuse modifiedMPR bits) - A-MPR values proposed in R4-2006788. - For repurposing NS_201, considering the concern on already existing devices supporting NS_201, this CR uses another NS value as NS_206. - For enforcement of the time of “UE brought into use”, as discussed in R4-2010615, this CR use same wording of “UE brought into use” as WRC-19 decision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce EESS protection with new NS and A-MPR approved in R4-2009141. - Add description so that modified MPR can be used to explucit signalling for a UE to report newly supported NS value(s) for a legacy band. - For repurposing NS_201, introduce another NS value as NS_206 with -10dBm/100MHz and -5dBm/200MHz protection. - For enforcement of the time of “UE brought into use”, put NOTE describing supportiveness of relevant NS, i.e., NS_204 and NS_206 shall be mandatory for UE brought into use after changeover date of 1 September 2027. - A-MPR for NS_206 is calculated as MAX(A-MPR for NS_202, A-MPR f or NS_204) since NS_202 is associated with -10dBm/100MHz and 1dBm/200MHz protection and NS_204 is associated with -5dBm/200MHz protection, and NS_206 is with -10dBm/100MHz and -5dBm/200MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Introduction of EESS protection approved in WRC-19 is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2A.3, 6.5.3.2, 6.5A.3.2, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

6.2.3	UE maximum output power with additional requirements

6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2.1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of and the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2.2.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_201 | 6.5.3.2.2 | n258 |  |  | 6.2.3.2 |
| NS_202 | 6.5.3.2.3 | n257, n258 |  |  | 6.2.3.3 |
| NS_2031 | 6.5.3.2.4 | n258 |  |  | 6.2.3.4 |
| NS_2041 | 6.5.3.2.5 | n257, n258 |  |  | 6.2.3.5 |
| NS_2051 | 6.5.3.2.6 | n260 |  |  | 6.2.3.6 |
| NS_2061 | 6.5.3.2.7 | n257, n258 |  |  | 6.2.3.7 |
| NOTE 1: The NS applies subject to indication in the field modifiedMPRbehavior as defined in ANNEX H. |  |  |  |  |  |

Table 6.2.3.1-2: Mapping of Network Signaling label

| NR Band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_202 | NS_204 | NS_206 |  |  |  |  |
| n258 | NS_200 | NS_201 | NS_202 | NS_203 | NS_204 | NS_206 |  |  |
| n260 | NS_200 | NS_205 |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [13]. |  |  |  |  |  |  |  |  |

6.2.3.2	A-MPR for NS_201

6.2.3.2.1	A-MPR for NS_201 for power class 1

For power class 1, A-MPR for NS_201 shall be 9 dB.

Table 6.2.3.2.1-1: (Void)

6.2.3.2.2	A-MPR for NS_201 for power class 2

For power class 2, A-MPR specified in clause 6.2.3.2.3 applies

Table 6.2.3.2.2-1: (Void)

6.2.3.2.3	A-MPR for NS_201 for power class 3

Table 6.2.3.2.3-1: AMPR for NS_201 for power class 3

| Offset Frequency | Channel Bandwidth, MHz |
| --- | --- |
|  | 400 |
|  | Outer RB allocations |
| v | ≤ 1.5 |
| > 100 MHz, ≤ 300 MHz | 0 |
| > 300 MHz | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from 24.25 GHz to the lower channel edge. NOTE 2: The allowable back off is max(MPR, AMPR), where the MPR is defined in Table 6.2.2.3-1 NOTE 3: Any undefined region, MPR applies |  |

6.2.3.2.4	A-MPR for NS_201 for power class 4

For power class 4, A-MPR for NS_201 specified in clause 6.2.3.2.3 applies.

6.2.3.3	A-MPR for NS_202

6.2.3.3.1	A-MPR for NS_202 for power class 1

For power class 1, A-MPR for NS_202 shall be 11.0 dB.

6.2.3.3.2	A-MPR for NS_202 for power class 2

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.3.3	A-MPR for NS_202 for power class 3

For power class 3, A-MPR for NS_202 shall be 1.0 dB.

6.2.3.3.4	A-MPR for NS_202 for power class 4

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

#### 6.2.3.4	A-MPR for NS_203

##### 6.2.3.4.1	A-MPR for NS_203 for power class 1

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCC

Where:

AMPRCC is 3.0 if Offset frequency < BWchannel, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel

##### 6.2.3.4.2	A-MPR for NS_203 for power class 2

For power class 2, AMPR for NS_203 specified in subclause 6.2.3.4.3 applies

##### 6.2.3.4.3	A-MPR for NS_203 for power class 3

For power class 3, AMPR for NS_203 shall be 0 dB.

##### 6.2.3.4.4	A-MPR for NS_203 for power class 4

For power class 4, AMPR for NS_203 specified in subclause 6.2.3.4.3 applies.

#### 6.2.3.5	A-MPR for NS_204

##### 6.2.3.5.1	A-MPR for NS_204 for power class 1

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCC

Where:

AMPRCC is 7.0 if Offset frequency < BWchannel, 6.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel

##### 6.2.3.5.2	A-MPR for NS_204 for power class 2

For power class 2, AMPR for NS_204 specified in subclause 6.2.3.5.3 applies

##### 6.2.3.5.3	A-MPR for NS_204 for power class 3

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCC

Where:

AMPRCC is 1.0 if Offset frequency < BWchannel, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel

##### 6.2.3.5.4	A-MPR for NS_204 for power class 4

For power class 4, AMPR for NS_204 specified in subclause 6.2.3.5.3 applies.

#### 6.2.3.6	A-MPR for NS_205

##### 6.2.3.6.1	A-MPR for NS_205 for power class 1

For power class 1, AMPR for NS_205 shall be 0.0 dB unless the allocation meets the following conditions to qualify as a low-edge allocation:

RBstart < BWchannel/10 MHz

RBstart  LCRB/2

Offset frequency < 10% of BWchannel

Where:

Offset frequency is defined as the frequency from 37.0 GHz to the lower channel edge

The AMPR for low-edge allocations shall be per table 6.2.3.6.1-1.

Table 6.2.3.6.1-1: AMPR for low-edge allocations for NS_205 for power class 1

|  | BWchan (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| A-MPR(dB) | 16.0 | 14.0 | 7.0 | 6.0 |

##### 6.2.3.6.2	A-MPR for NS_205 for power class 2

For power class 2, AMPR for NS_205 specified in subclause 6.2.3.6.3 applies

##### 6.2.3.6.3	A-MPR for NS_205 for power class 3

For power class 3, AMPR for NS_205 shall be 0.0 dB unless the allocation meets the following conditions to qualify as a low-edge allocation:

RBstart < BWchannel/10 MHz

RBstart  LCRB/2

Offset frequency < 10% of BWchannel

Where:

Offset frequency is defined as the frequency from 37.0 GHz to the lower channel edge

The AMPR for low-edge allocations shall be per table 6.2.3.6.3-1.

Table 6.2.3.6.3-1: AMPR for low-edge allocations for NS_205 for power class 3

|  | BWchan (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| A-MPR(dB) | 12.0 | 10.0 | 3.0 | 2.0 |

##### 6.2.3.6.4	A-MPR for NS_205 for power class 4

For power class 4, AMPR for NS_205 specified in subclause 6.2.3.6.3 applies.

6.2.3.7	A-MPR for NS_206

6.2.3.7.1	A-MPR for NS_206 for power class 1

For power class 1, A-MPR for NS_206 shall be 11.0 dB.

6.2.3.7.2	A-MPR for NS_206 for power class 2

For power class 2, A-MPR for NS_206 specified in clause 6.2.3.7.3 applies.

6.2.3.7.3	A-MPR for NS_206 for power class 3

For power class 3, A-MPR for NS_206 shall be 1.0 dB.

6.2.3.7.4	A-MPR for NS_206 for power class 4

For power class 4, A-MPR for NS_206 specified in clause 6.2.3.7.3 applies.

<Sections are omitted>

6.2A.3	UE maximum output power with additional requirements for CA

6.2A.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2A.1. Unless stated otherwise, an A-MPR of 0 dB shall be used. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2A.3.1-1 is allowed for all serving cells of the applicable uplink contiguous CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell.

Table 6.2A.3.1-1 specifies the additional requirements and allowed A-MPR with corresponding network signalling label and operating band. The mapping between network signalling labels and the additionalSpectrumEmission IE defined in TS 38.331 [13] is specified in Table 6.2A.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

Table 6.2A.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_200 |  |  |  |  | N/A |
| CA_NS_201 | 6.5.3.2.2 | n258 |  |  | 6.2A.3.2 |
| CA_NS_202 | 6.5.3.2.3 | n257, n258 |  |  | 6.2A.3.3 |
| CA_NS_2031 | 6.5.3.2.4 | n258 |  |  | 6.2A.3.4 |
| CA_NS_2041 | 6.5.3.2.5 | n257, n258 |  |  | 6.2A.3.5 |
| CA_NS_2051 | 6.5.3.2.6 | n260 |  |  | 6.2A.3.6 |
| CA_NS_2061 | 6.5.3.2.7 | n257, n258 |  |  | 6.2A.3.7 |
| NOTE 1: The NS applies subject to indication in the field modifiedMPRbehavior as defined in ANNEX H. |  |  |  |  |  |

Table 6.2A.3.1-2: Value of additionalSpectrumEmission

| NR Band | Value of additionalSpectrumEmission / NS number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | CA_NS_200 | CA_NS_202 | CA_NS_204 | CA_NS_206 |  |  |  |  |
| n258 | CA_NS_200 | CA_NS_201 | CA_NS_202 | CA_NS_203 | CA_NS_204 | CA_NS_206 |  |  |
| n260 | CA_NS_200 | CA_NS_205 |  |  |  |  |  |  |
| n261 | CA_NS_200 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [13]. |  |  |  |  |  |  |  |  |

6.2A.3.2	A-MPR for CA_NS_201

6.2A.3.2.1	A-MPR for CA_NS_201 for power class 1

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.1-1: (Void)

For power class 1 CA non-contiguous RB allocations, the following rule for AMPR (dB) applies:

AMPR = max(AMPRC_CA, -10*A +12.0)

Where AMPRC_CA is 9.0

6.2A.3.2.2	A-MPR for CA_NS_201 for power class 2

For intra-band contiguous CA, A-MPR specified in sub-clause 6.2A.3.2.3 applies.

Table 6.2A.3.2.2-1: (Void)

6.2A.3.2.3	A-MPR for CA_NS_201 for power class 3

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.3-1: Contiguous Allocations, AMPRC_CA for CA_NS_201 for power class 3

| Offset Frequency | Cumulative Aggregated Bandwidth, MHz |  |
| --- | --- | --- |
|  | < 400 | ≥ 400, ≤ 800 |
| 0 MHz, ≤ 100 MHz | ≤ 1.5 | ≤ 3.0 |
| > 100 MHz, ≤ 300 MHz | 0 | 0 |
| > 300 MHz | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from 24.25 GHz to the lower channel edge. NOTE 2: The allowable back off is max(MPR, AMPR), where the MPR is defined in Table 6.2A.2.4-1. NOTE 3: Any undefined region, MPR applies. |  |  |

For power class 3 CA non-contiguous RB allocations, the following rule for AMPR applies:

AMPR = max(AMPRC_CA, - 10*A + 5.0) , Offset Frequency ≤ 550 MHz

6.2A.3.2.4	A-MPR for CA_NS_201 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_201 specified in sub-clause 6.2A.3.2.3 applies.

6.2A.3.3	A-MPR for CA_NS_202

6.2A.3.3.1	A-MPR for CA_NS_202 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 11.0 dB.

6.2A.3.3.2	A-MPR for CA_NS_202 for power class 2

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

6.2A.3.3.3	A-MPR for CA_NS_202 for power class 3

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 2.0 dB.

6.2A.3.3.4	A-MPR for CA_NS_202 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

#### 6.2A.3.4	A-MPR for CA_NS_203

##### 6.2A.3.4.1	A-MPR for CA_NS_203 for power class 1

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCA

Where:

AMPRCA is 6.5 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the the CC closest to the low band edge

##### 6.2A.3.4.2	A-MPR for CA_NS_203 for power class 2

For intra-band CA, AMPR specified in sub-clause 6.2A.3.4.3 applies.

##### 6.2A.3.4.3	A-MPR for CA_NS_203 for power class 3

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCA

Where:

AMPRCA is 2.5 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the the CC closest to the low band edge

##### 6.2A.3.4.4	A-MPR for CA_NS_203 for power class 4

For intra-band CA, AMPR specified in sub-clause 6.2A.3.4.3 applies.

#### 6.2A.3.5	A-MPR for CA_NS_204

##### 6.2A.3.5.1	A-MPR for CA_NS_204 for power class 1

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCA

Where:

AMPRCA is 9.0 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 6.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the the CC closest to the low band edge

##### 6.2A.3.5.2	A-MPR for CA_NS_204 for power class 2

For intra-band CA, AMPR specified in sub-clause 6.2A.3.5.3 applies.

##### 6.2A.3.5.3	A-MPR for CA_NS_204 for power class 3

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCA

Where:

AMPRCA is 5.0 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the the CC closest to the low band edge

##### 6.2A.3.5.4	A-MPR for CA_NS_204 for power class 4

For intra-band CA, AMPR specified in sub-clause 6.2A.3.5.3 applies.

#### 6.2A.3.6	A-MPR for CA_NS_205

##### 6.2A.3.6.1	A-MPR for CA_NS_205 for power class 1

For intra-band CA, the following rule for AMPR (dB) applies:

AMPR = max(AMPRnonEdge, AMPREdge,CA)

Where

AMPREdge,CA= AMPR as determined per sub-section 6.2.3.6.1, for the CC closest to low band edge and the contiguous RB allocation closest to lower edge of that CC

AMPRnonEdge is 5.0 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 0.0 otherwise

Offset frequency is defined as the frequency from 37.0 GHz to the lower edge of the the CC closest to the low band edge

##### 6.2A.3.6.2	A-MPR for CA_NS_205 for power class 2

For intra-band CA, AMPR specified in sub-clause 6.2A.3.6.3 applies.

##### 6.2A.3.6.3	A-MPR for CA_NS_205 for power class 3

For intra-band CA, the following rule for AMPR (dB) applies:

AMPR = max(AMPRnonEdge, AMPREdge,CA)

Where

AMPREdge,CA= AMPR as determined per sub-section 6.2.3.6.3, for the CC closest to low band edge and the contiguous RB allocation closest to lower edge of that CC

AMPRnonEdge is 1.0 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 0.0 otherwise

Offset frequency is defined as the frequency from 37.0 GHz to the lower edge of the the CC closest to the low band edge

##### 6.2A.3.6.4	A-MPR for CA_NS_205 for power class 4

For intra-band CA, AMPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

6.2A.3.7	A-MPR for CA_NS_206

6.2A.3.7.1	A-MPR for CA_NS_206 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_206 shall be 11.0 dB.

6.2A.3.7.2	A-MPR for CA_NS_206 for power class 2

For intra-band contiguous CA, A-MPR for CA_NS_206 specified in sub-clause 6.2A.3.7.3 applies.

6.2A.3.7.3	A-MPR for CA_NS_206 for power class 3

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCA

Where:

AMPRCA is 5.0 if Offset frequency < frequency separation or agg. BW of the  UL CA configuration, 2.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the the CC closest to the low band edge

6.2A.3.7.4	A-MPR for CA_NS_206 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_206 specified in sub-clause 6.2A.3.7.3 applies.

<Sections are omitted>

6.5.3.2	Additional spurious emissions

6.5.3.2.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious

emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for

a specific deployment scenario as part of the cell handover/broadcast message.

6.5.3.2.2	Additional spurious emission requirements for NS_201

When "NS_201" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.2-1: Additional requirements (NS_201)

| Frequency Range (GHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |  |  |
| 23.6 f 24 | -5 | -5 | -5 | -5 | 200 MHz | 1 |
| NOTE 1: The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |

6.5.3.2.3	Additional spurious emission requirements for NS_202

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |  |
| --- | --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 | 100 MHz |  |  |
| 23.6 GHz f 24.0 GHz | +1 | 200 MHz | 1 |  |
| NOTE 1: The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

##### 6.5.3.2.4	Additional spurious emission requirements for NS_203

When "NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.4-1: Additional requirements (NS_203)

| Frequency Range (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |  |
| --- | --- | --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz | 1 |  |
| NOTE 1: The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

##### 6.5.3.2.5	Additional spurious emission requirements for NS_204

When "NS_204" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

NOTE: -5 dBm / 200 MHz limit is applicable to new devices that become operational in Sept 2027 onwards

Table 6.5.3.2.5-1: Additional requirements (NS_204)

| Frequency Range (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |  |
| --- | --- | --- | --- | --- |
| 23.6 f 24.0 | -5 | 200 MHz | 1 |  |
| NOTE 1: The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

##### 6.5.3.2.6	Additional spurious emission requirements for NS_205

When "NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.6-1: Additional requirements (NS_205)

| Frequency Range (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |  |
| --- | --- | --- | --- | --- |
| 36.0 f 37.0 | +7 | 1000 MHz | 1 |  |
|  | -13 | 1 MHz | 1 |  |
| NOTE 1: The protection of frequency range 36000 - 37000 MHz is meant for protection of satellite passive services. |  |  |  |

##### 6.5.3.2.7	Additional spurious emission requirements for NS_206

When "NS_206" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

NOTE: -5 dBm / 200 MHz limit is applicable to new devices that become operational in Sept 2027 onwards

Table 6.5.3.2.7-1: Additional requirements (NS_206)

| Frequency Range | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |  |
| --- | --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 | 100 MHz |  |  |
| 23.6 GHz f 24.0 GHz | -5 | 200 MHz | 1 |  |
| NOTE 1: The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

<Sections are omitted>

6.5A.3.2	Additional spurious emissions

6.5A.3.2.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.2.2	Additional spurious emission requirements for CA_NS_201

When "CA_NS_201" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in clause 6.5A.3.

6.5A.3.2.3	Additional spurious emission requirements for CA_NS_202

When "CA_NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

##### 6.5A.3.2.4	Additional spurious emission requirements for CA_NS_203

When "CA_NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

##### 6.5A.3.2.5	Additional spurious emission requirements for CA_NS_204

When "CA_NS_204" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

##### 6.5A.3.2.6	Additional spurious emission requirements for CA_NS_205

When "CA_NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

##### 6.5A.3.2.6	Additional spurious emission requirements for CA_NS_206

When "CA_NS_206" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

<Sections are omitted>

Annex H (Normative)

Modified MPR behavior

H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. A modified A-MPR behaviour for a supported NR band can also indicate support of an NS value and associated allowed A-MPR specified in a given version of this specification.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n257 |
|  | 1 | - NS_204 as defined in clause 6.2.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n257 and brought into use after 1 September 2027. |
|  | 2 | - CA_NS_204 as defined in clause 6.2A.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n257 and brought into use after 1 September 2027. |
|  | 3 | - NS_206 as defined in clause 6.2.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n257 and brought into use after 1 September 2027. |
|  | 4 | - CA_NS_206 as defined in clause 6.2A.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n257 and brought into use after 1 September 2027. |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n258 |
|  | 1 | - AMPR for NS_201 as defined in clausue 6.2.3.2 of 38.101-2 v15.7.0 | - This bit may be set to 1 by a UE supporting n258 |
|  | 2 | - NS_203 as defined in clause 6.2.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 3 | - CA_NS_203 as defined in clause 6.2A.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 4 | - NS_204 as defined in clause 6.2.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n258 and brought into use after 1 September 2027. |
|  | 5 | - CA_NS_204 as defined in clause 6.2A.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n258 and brought into use after 1 September 2027. |
|  | 6 | - NS_206 as defined in clause 6.2.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n258 and brought into use after 1 September 2027. |
|  | 7 | - CA_NS_206 as defined in clause 6.2A.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n258 and brought into use after 1 September 2027. |
| n260 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n260 |
|  | 1 | - NS_205 as defined in clause 6.2.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n260 |
|  | 2 | - CA_NS_205 as defined in clause 6.2A.3.1 of 38.101-2 v15.10.0 | - This bit shall be set to 1 by a UE supporting n260 |
| n261 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n261 |

<End of change>