3GPP TSG-RAN WG4 Meeting #96-e	R4-2011416

# Electronic meeting, 17-28 Aug, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1108 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on scheduling restriction for CSI-RS based intra-frequency measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  |  | Date: |  | 2020-08-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CSI-RS L3 measurement was introduced to RAN4 in Rel-16. The CR aims to add restrictions in the scheduling availability for UE during CSI-RS L3 intra-frequency measurements. The section number follows the structure of R4-2009007, which has been technically endorsed to have CSI-RS related requirements in a separate section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the scheduling restriction for CSI-RS based intra-frequency measurement for RRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based UE measurement requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.x.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.x.2.x	Scheduling availability of UE during CSI-RS based intra-frequency measurements

UE shall be capable of measuring without measurement for the intra-frequency measurements based on CSI-RS resources as configured in ResourceConfigMobility [2] for RRM. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction.

##### 9.x.2.x.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to the UL and DL collision.

- The UE is not expected to transmit PUCCH/PUSCH/SRS on CSI-RS symbol to be measured and on 1 OFDM symbol before each CSI-RS symbol to be measured and 1 OFDM symbol after each CSI-RS symbol to be measured within the configured slot as indicated in slotConfig [2] of the corresponding CSI-RS resource.

##### 9.x.2.x.2	Scheduling availability of UE performing CSI-RS based measurements with non-serving UE Rx beam in FR2

When the UE performs CSI-RS based intra-frequency measurements for L3 mobility management, if the associated SSB is configured and QCLed with the corresponding CSI-RS, UE may switch to an alternative Rx beam for measuring a neighbor cell according to its associated SSB. The following restrictions apply.

- The UE is not expected to receive PDCCH/PDSCH/TRS on CSI-RS symbols to be measured and on 1 OFDM symbol before each CSI-RS symbol to be measured and 1 OFDM symbol after each CSI-RS symbol to be measured within the configured slot as indicated in slotConfig [2] of the corresponding CSI-RS resource.

- If the associated SSB is not configured, above restriction is not needed.

<End of Change 1>