3GPP TSG-RAN WG4 Meeting#96-e	R4- 2011425

Online, 17th Aug – 28th Aug, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on FR2 DL 256QAM SDR Tests

Agenda item:	7.10.3.2

Document for:	Discussion

# Introduction

In WF [1], some issues were still open and different options were provided for open issues. In this paper, we discuss our views on open issues for FR2 DL 256QAM tests.

# Testable SNR for FR2

Using the Demod SNR range calculator spreadsheet in 38.810, we computed the testable SNR for SDR scenario (Mode 2) as presented in below observation assuming multi-band relaxation factor of 1.7dB in Cell K5 as per maximum value in Table 6.2.1.3-4 in 38.101-2.

For deriving testable SNR for SDR requirements, we set Cell M11 in the spreadsheet to 20dB and vary the number of RBs in Cell K28 based on CBW.

Observation 1: Maximum testable SNRs for FR2 band n260 under Mode 2 (used for SDR requirements) are given in Table 1.

Table 1: Maximum testable SNR for FR2 band n260 for SDR requirements with 120kHz SCS

| CBW (MHz) | 50 | 100 | 200 | 400 | 800 | 1200 |
| --- | --- | --- | --- | --- | --- | --- |
| Number of RBs | 32 | 66 | 132 | 264 | 528 | 792 |
| Max Testable SNR in dB | 28.1 | 25.0 | 22.0 | 18.9 | 15.9 | 14.2 |

# SDR Requirements

In Table 2, we present the required SNR to achieve 85% of peak throughput for fixed MCS under static channel condition assuming 50MHz/120kHz and 3% Tx EVM. Requirement column is derived by adding 0.8dB IM (as agreed in Rel-15 FR1 256QAM tests) to impairment results.

Table 2: Required SNR for 50MHz/120kHz with 256QAM at 85% of peak throughput under static channel

| MCS | Rank | Alignment SNR (dB) | Impairment SNR (dB) | Requirement SNR (dB) |
| --- | --- | --- | --- | --- |
| 20 | 1 | 17.56 | 19.56 | 20.36 |
| 21 | 1 | 18.28 | 20.28 | 21.08 |
| 22 | 1 | 19.82 | 21.82 | 22.62 |
| 23 | 1 | 21.50 | 23.50 | 24.30 |
| 24 | 1 | 23.30 | 25.30 | 26.10 |
| 25 | 1 | 25.71 | 27.71 | 28.51 |
| 20 | 2 | 20.75 | 22.75 | 23.55 |
| 21 | 2 | 21.64 | 23.64 | 24.44 |
| 22 | 2 | 23.23 | 25.23 | 26.03 |

Based on above preliminary simulation results, we can get to higher throughput than 64QAM for some cases. Therefore, we propose the following.

Proposal 1: Define SDR requirements for FR2 256QAM.

# Conclusions

This paper discusses open issues for FR2 256QAM test cases. Following has been observed and proposed.

Observation 1: Maximum testable SNRs for FR2 band n260 under Mode 2 (used for SDR requirements) are given in Table 1.

Proposal 1: Define SDR requirements for FR2 256QAM.

# References

[1] R4-2008817, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, 3GPP TSG RAN WG4 Meeting #95-e, Online, May 25 - Jun 5, 2020.