3GPP TSG-RAN WG4 Meeting#96-e	R4- 2011433

Online, 17th Aug – 28th Aug, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on FR2 DL 256QAM CSI Reporting Tests

Agenda item:	7.10.3.3

Document for:	Discussion

# Introduction

In WF [1], some issues were still open and different options were provided for open issues. In this paper, we discuss our views on open issues for FR2 DL 256QAM tests.

# Testable SNR for FR2

Using the Demod SNR range calculator spreadsheet in 38.810, we computed the testable SNR for Mode1 (with Noc) as presented in below observation assuming multi-band relaxation factor of 1.7dB in Cell K5 as per maximum value in Table 6.2.1.3-4 in 38.101-2.

Observation 1: Maximum testable SNR for FR2 band n260, 50MHz/120kHz under Mode 1 (used for fixed MCS, CQI reporting requirements) is 21.0 dB.

# CQI Reporting Requirements

Under the assumptions of CQI reporting (No HARQ, No data FDM’ed with DMRS), we present the simulation results for fixed MCS under different channel conditions. Table 1 shows the required SNR to achieve 90% of peak throughput for fixed MCS assuming 50MHz/120kHz and 3% Tx EVM. Requirement column is derived by adding 0.8dB IM (as agreed in Rel-15 FR1 256QAM tests) to impairment results.

Table 1: Required SNR for 50MHz/120kHz with 256QAM at 90% of peak throughput for 2x2 XP High

| Case | Alignment SNR (dB) | Impairment SNR (dB) | Requirement SNR (dB) |
| --- | --- | --- | --- |
| TDLA30-35, Rank1, MCS20 | 21.74 | 23.74 | 24.54 |
| TDLD30-35, Rank1, MCS20 | 18.61 | 20.61 | 21.41 |
| TDLA30-35, Rank1, MCS21 | 22.52 | 24.52 | 25.32 |
| TDLD30-35, Rank1, MCS21 | 19.44 | 21.44 | 22.24 |

Minimum 256QAM CQI is mapped to MCS21. Based on above results and Observation 1, we have following observation and proposal.

Observation 2: It is not possible to reach 256QAM CQI under fading condition due to the testable SNR limit for FR2.

Proposal 1: Do not define FR2 256QAM fading CQI reporting requirements.

# Conclusions

This paper discusses open issues for FR2 256QAM test cases. Following has been observed and proposed.

Observation 1: Maximum testable SNR for FR2 band n260, 50MHz/120kHz under Mode 1 (used for fixed MCS, CQI reporting requirements) is 21.0 dB.

Observation 2: It is not possible to reach 256QAM CQI under fading condition due to the testable SNR limit for FR2.

Proposal 1: Do not define FR2 256QAM fading CQI reporting requirements.

# References

[1] R4-2008817, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, 3GPP TSG RAN WG4 Meeting #95-e, Online, May 25 - Jun 5, 2020.