3GPP TSG-RAN4 WG4 Meeting # 96-e	R4-2011488

# Electronic meeting, 17 - 28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	On new FWA UE RF requirement for FR2

Agenda Item:	10.20.1

Document for:	Approval

Introduction

Remaining issues for the FR2 new FWA UE RF requirement can be summarized as below:

- Peak EIRP/EIS

- UE capability on power class

- Beam correspondence and MBR requirement

- MPRnarrow requirement

This paper provides further proposal on FR2 new FWA UE RF requirement in Rel-17.

Discussion

Peak EIRP and EIS

Uplink limited system:FR2

Considering high frequency of FR2 and high pathloss/penetration loss, FR2 network coverage is limited. For handheld type UE, the output power is limited by antenna array size, PA number, power consumption and form factor. Hence FR2 Handheld UE is highly limited by uplink power, it makes hot-spot coverage for FR2 handheld more popular.

Meanwhile, such limitation triggers the FWA application into commercial usage on FR2. It has few limitation on power consumption, form factor and antenna array scale. In Rel-15, FWA UE PC1 is specified with minimum EIRP 40dBm which requires for more than 16 elements per polarization to reach. Such FR2 high power application can introduce 5G signal outdoor into building, and enhance on the uplink coverage distance. By professional install, the UL coverage radius can be more than 2km.

Under regulatory requirement with max TRP/EIRP 23dBm/43dBm, Rel-17 New FWA UE targets for enhance FR2 uplink coverage under such condition.

To have evaluation on the performance of UL coverage, we provide UL and DL budget for each modulation order in Table 1. The comparison on UL and DL coverage can be seen clearly.

Table 1-1. UL coverage performance evaluation parameters for 28GHz

| UE Tx Parameter(28GHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Modulation | QPSK |  | 16QAM |  | 64QAM |  |
| Bandwidth | 200MHz |  |  |  |  |  |
| UE min peak EIRP 1 | 32dBm[1] |  |  |  |  |  |
| UE min peak EIRP 2 | 28.7dBm[2] |  |  |  |  |  |
| UE min peak EIRP 3 | 26dBm[3] |  |  |  |  |  |
| MPR(inner allocation for CP-OFDM) | 3.5dB |  | 5dB |  | 7.5dB |  |
| UE antenna height(hUT) | 1.5m |  |  |  |  |  |
| gNB Rx parameter |  |  |  |  |  |  |
| Element number | 256 |  |  |  |  |  |
| NF | 9dB |  |  |  |  |  |
| Thermal noise floor | -89dBm |  |  |  |  |  |
| Element gain | 8dBi |  |  |  |  |  |
| Array gain | 24dB |  |  |  |  |  |
| Total antenna Gain | 32dB |  |  |  |  |  |
| Rx total noise floor | -80dBm |  |  |  |  |  |
| BS height(hBS) | 10m |  |  |  |  |  |
| Demod threshold | QPSK |  |  | 16QAM |  | 64QAM |
|  | 2dB |  |  | 10dB |  | 15dB |
| Max path loss | 135dB |  |  | 125dB |  | 118dB |
| Path Loss for Link-Umi-LOS |  |  |  |  |  |  |
| Shadow fading std | 4dB |  |  |  |  |  |
| d’BP | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL1 | 32.4+21log(d3D)+20log(fc) |  |  |  |  |  |
| PL2 | 32.4+21log(d3D)+20log(fc)-9.5log( (d’BP^2)+ (hBS-hUT)^2) |  |  |  |  |  |
| PL |  |  |  |  |  |  |
| Path Loss for Link-Umi-NLOS |  |  |  |  |  |  |
| Shadow fading std | 7.8dB |  |  |  |  |  |
| d’BP | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL | for |  |  |  |  |  |
| Other penetration loss assumption |  |  |  |  |  |  |
| Glass |  | 10dB(single film) or 20dB(bi-layer film) |  |  |  |  |
| Foliage |  | 10dB |  |  |  |  |

The relation among d3D, d2D, hUT and hBS can be provided as below from TR 38.901:

From the budget calculation based on above assumptions, we get the following UL coverage radius for LOS and NLOS respectively:

Table 1-2 UL coverage radius for different UE Tx power

|  | Umi-LOS(single film glass) |  |  | Umi-NLOS(single film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE min peak EIRP | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| 32dBm | 222m | 78m | 33m | 32m | 15.6m | 6.8m |
| 28.7dBm | 142m | 49m | 20m | 24m | 10.7m | N/A |
| 26dBm | 114.6m | 40m | 15.5m | 20.5m | 8.5m | N/A |

|  | Umi-LOS(bi-film glass) |  |  | Umi-NLOS(bi-film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE min peak EIRP | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| 32dBm | 73.8m | 24.8m | 7.78m | 15m | 3.6m | N/A |
| 28.7dBm | 47m | 14.5m | N/A | 10.14m | N/A | N/A |
| 26dBm | 37.4m | 10.5m | N/A | 7.9m | N/A | N/A |

Table 2-1. DL coverage performance evaluation parameters for 28GHz

| gNB Tx Parameter(28GHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element number | 256 |  |  |  |  |  |  |
| Element gain | 8dBi |  |  |  |  |  |  |
| Total array gain | 32dB |  |  |  |  |  |  |
| Total EIRP | QPSK |  |  | 16QAM |  |  | 64QAM |
|  | 64dBm |  |  | 63dBm |  |  | 61dBm |
| UE Rx parametere |  |  |  |  |  |  |  |
| UE total noise floor | -82dBm |  |  |  |  |  |  |
| UE max peak EIS 1 | -93.3dBm@50MHz [1] |  |  |  |  |  |  |
| Demod threshold | QPSK |  |  |  | 16QAM | 64QAM |  |
|  | 2dB |  |  |  | 10dB | 15dB |  |
| Path Loss for Link-Umi-LOS |  |  |  |  |  |  |  |
| Shadow fading std |  | 4dB |  |  |  |  |  |
| d’BP |  | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL1 |  | 32.4+21log(d3D)+20log(fc) |  |  |  |  |  |
| PL2 |  | 32.4+21log(d3D)+20log(fc)-9.5log( (d’BP^2)+ (hBS-hUT)^2) |  |  |  |  |  |
| PL |  |  |  |  |  |  |  |
| Path Loss for Link-Umi-NLOS |  |  |  |  |  |  |  |
| Shadow fading std |  | 7.8dB |  |  |  |  |  |
| d’BP |  | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL |  | for |  |  |  |  |  |
| Path Loss |  |  |  |  |  |  |  |
| Other penetration loss assumption |  |  |  |  |  |  |  |
| Glass |  |  | 10dB(single film) or 20dB(bi-layer film) |  |  |  |  |
| Foliage |  |  | 10dB |  |  |  |  |

Table 2-2 DL coverage radius for different UE Tx power

|  | Umi-LOS(single film glass) |  |  | Umi-NLOS(single film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE max peak EIS | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| -93.3dBm | 1.9km | 805m | 373m | 127m | 70m | 44m |

|  | Umi-LOS(bi-film glass) |  |  | Umi-NLOS(bi-film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE max peak EIS | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| -93.3dBm | 721m | 269m | 124m | 65m | 36m | 21.8m |

From comparison for FR2 UL and DL coverage, we found that it is highly uplink limited system. It makes high Tx power crucial for this network. It is not exaggerated to say: 26~28.7Bm Tx power is far from the basic requirement that the system requires for.

When Tx power is lower than 30dBm, we can see 64QAM is generally not possible for uplink transmission, it will have big impact on throughput and user experience. Without 64QAM indoor experience, NR do not have evident gain compared with fiber cable into building.

Observation 1: When Tx power is lower than 30dBm, we can see 64QAM is generally not possible for uplink transmission on 28GHz.

EIRP

Reviewing on Japanese regulation requirements, actually the max antenna gain is defined as 20dBi. 20dBi requires for at least 32 antenna elements. Assumes 14dBm output power per element, it will be 32dBm TRP considering dual polarization gain which far exceeds the max TRP limitation. For 16 antenna elements, it can have about 16dBi antenna gain, and 29dBm TRP with 14dBm per element output power. We can see that reducing number of antenna elements will have impact on directional gain while we need to control the max TRP limit. To reach the balance between antenna gain and TRP, 14dBm Pout per element may be not suitable for the new FWA UE. We provide the comparison on the theoretical calculation in the below table, we can see the rule combined on Pout per element and elements number. Considering FWA usage, higher min peak EIRP than PC3 would be needed, it requires for adaptable TRP and antenna gain.

From values shown in table 1, we would like to choose the configuration with TRP little higher than 23dBm (considering there could be implementation loss) and antenna gain in the middle range. Thus, the best choice would be 11dBm Pout per element and 16dBi antenna gain.

| Pout per element | 14dBm | 14dBm | 11dBm | 14dBm | 14dBm |
| --- | --- | --- | --- | --- | --- |
| # of elements in an array | 32 | 16 | 16 | 8 | 4 |
| Max TRP without IM | 32dBm | 29dBm | 26dBm | 26dBm | 23dBm |
| Max TRP with IM | 28dBm | 25dBm | 22dBm | 22dBm | 18dBm |
| Antenna gain | 19dBi | 16dBi | 16dBi | 13dBi | 10dBi |
| Problem | Exceed max TRP | Exceed max TRP | meet max TRP High EIRP | Low EIRP | Low EIRP |

Observation 2: To make sure on min peak EIRP>30dBm and max TRP is ensured be ≤23dBm, 16 antenna elements is the best choice.

Proposal 1: Agrees on 16 antenna elements assumption for the new FWA UE.

Based the analysis above, we provide power budget for T/Rx in Table 2 and Table 3.

Table 2: Power budget for min peak EIRP

| Parameters | unit | Nominal | Worst |
| --- | --- | --- | --- |
| Band |  | n257, n258 |  |
| Pout per element | dBm | 11dBm |  |
| # of elements in an array |  | 16 |  |
| Total conducted power per polarization | dBm | 23 | -1 |
| Avg antenna element gain | dBi | 4 | 0 |
| Antenna rolloff loss versus frequency | dB | -1.5 | -0.2 |
| Realized antenna array gain | dBi | 14.5 | 0 |
| Polarization gain | dB | 2.8 | -0.5 |
| Mismatch and transmission line loss including load pull | dB | -2 | -0.7 |
| Beam forming loss(phase shifter and amplitude error) | dB | -0.5 | 0 |
| Finite beam table | dB | -0.1 | 0 |
| Beam forming loss(one beam table fits all) | dB | -0.25 | 0 |
| Form factor integration losses | dB | -2 | -1 |
| Total implementation loss(nominal) | dB | -4.85 |  |
| Total implementation loss(worst case) | dB |  | -8.3 |
| Total implementation loss(best case) | dB |  |  |
| Max TRP, should≤23dBm |  | 22dBm |  |
| Peak EIRP(nominal) | dBm | 35.48 |  |
| Tolerance(+/-) | dB |  | 3.5 |
| Peak EIRP(Minimum) | dBm | 32 |  |

Table 3: power budget for min peak EIS

| Parameter | Unit | Value |
| --- | --- | --- |
| Band |  | n257, n258 |
| modulation |  | QPSK |
| SNR |  | -1dB |
| Bandwidth | MHz | 50MHz |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure | dB | 12 |
| Number of antenna in an array |  | 16 |
| Array gain | dB | 11 |
| Element gain | dBi | 4 |
| Diversity gain | dB | 0 |
| Antenna gain roll-off over frequency | dB | -1.5 |
| Beamforming loss | dB | -0.25 |
| Total insertion loss | dB | -8.3 |
| REFSENS | dBm | -93.3 |

Proposal 2: Define minimum peak EIRP for the new FWA UE as 32dBm.

Proposal 3: Define maximum peak EIS for the new FWA UE as -93.3dBm.

UE capability on power class

In the last meeting, other evaluations on min peak EIRP are raised. We summarized them as below:

| Contribution No. | Antenna elements assumption | Min peak EIRP |
| --- | --- | --- |
| [1] | 16 | 32dBm |
| [2] | 16 | 28.7/28dBm |
| [3] | 8 | 26dBm |
| [4] | 8 | 28.3dBm |
| [5] | 8 | 27.4dBm |

As a UE vendor, we fully understand the implementation considerations from different perspectives. E.g. some UEs may adopt with 16 antenna elements, but prefer to leave a high margin in case there are stressful calibration work under OTA environment. However, TS 38.101-2 will not capture any limitation on antenna number, there is only Tx power requirement in the spec. From the UL performance analysis mentioned before, we don’t want to ignore the basic requirement from the whole FR2 system. From Rel-15, UL 64QAM is introduced for FR2 as mandatory feature, there are lots of discussion on PN model and other impairment on UE design, we cannot believe that finally this mandatory feature cannot be utilized in the real network.

Hence, we can compromise to accept to introduce 2 power classes for the new FWA UE. We provide 2 options on how to define these 2 power classes in the spec which is captured in WF[6]:

- Option 1: define 2 new power classes named PC5 and PC6

- Option 2: Repurpose existing power classes with additional signalling:

  - Example: PC2 in ‘FWA condition’ is expressed to produce +28dBm of peak EIRP, and 8dB degradation at 85%

Frankly saying, we prefer option 2 from simplifying TS 38.101-2 perspective. The EIRP value raised by companies are very close to the PC2 min Peak EIRP requirement, if we define PC6 for 28.7dBm UE and PC2 with 29dBm UE, how many power classes we need for FR2 in the future?

Proposal 4: Considering different implementations, introduce 2 power classes for the new FWA UE:

  - Power class 5 with 32dBm min peak EIRP

  - Power class 6 with 29dBm min peak EIRP

    - Or reuse Power class 2, with additional signalling on 85% spherical coverage requirement.

Beam correspondence and MBR requirement

For multi-band relaxation requirement, we can follow the agreement in Rel-16: specify multiband relaxation framework with per-band relaxation. Since this WI is only related to n257 and n258, only 1 multi-band combination (n257+n258) need to be considered.

Assuming 16 antenna elements for the new FWA UE, the beam width is finer for both DL and UL beam compared with PC3, it means that tiny inaccuracy on DL Beam measurement and corresponding beam selecting will lead to larger difference between best beam and corresponding beam. Meanwhile, larger scale antenna array will deteriorate the wide band antenna performance even worse. Hence, larger value for multiband relaxation would be needed for the new FWA UE.

For MBR value per band, there are still 3 options under discussion[7]:

- 0.7dB per band for both peak and spherical

- 0dB

- FFS

We prefer 0.7dB which follows the Rel-16 MBR requirements which agrees in FR2 enhanced RF requirement WI. We understand on the 0dB option proposed by one company for rel-17 enhancement, it can be further discussed in Rel-17 RF requirement enhancement WI.

Proposal 5: For new FR2 FWA UE, specify the multi-band relaxation requirement per band as 0.7dB for both peak and spherical requirement.

MPRnarrow for the new FWA UE

Generally, RAN4 assumes that MPR is defined based on the total TRP transmitted by all PAs. For new FWA UE, the output power for one PA maybe lower than PC3, this is because new FWA UE need more antenna elements to reach the higher EIRP, output power for each PA is reduced to ensure total TRP not exceed 23dBm. However, the total TRP for FWA UE is larger than PC3. For PC3, the max TRP is 23dBm, and min peak EIRP is 22.4dBm, the total TRP do not need to push to the max: 23dBm, it can be less than 23dBm, e.g. 16.5dBm, and UE still can meet the min peak EIRP requirement 22.4dBm.

As mentioned before, since the antenna elements number and the PA output power could be changed to reach the balance between high antenna gain and max TRP limitation, the MPR may need to be re-evaluated based on the PC3 MPR framework. For example, the min peak EIRP for PC3 is 22.4dBm while the max TRP is 23dBm with 4 antenna elements per polarization. Assuming 8.5dB antenna gain provided by most companies, the real TRP is about 25dBm(nominal peak EIRP)-8.5dB=16.5dBm; for the new FWA UE, we propose nominal peak EIRP with 35dBm with 14.5dB antenna gain, then the real TRP would be 20.5dBm. we can see a 4dB real TRP different between PC3 and PC5. Hence, the MPR may need to some re-evaluation.

Additionally, MPRnarrow is defined only for narrow RB allocation which is limited within 1.44MHz and only for the outer 1/3 position in the current spec(0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB). For such case, all the TRP transmission is focused on 1 RB and make PSD very high, then it leads to the emission problem.

Considering the potential TRP difference between PC3 and the new FWA UE, we propose to revise the MPRnarrow as 7dB for the new FWA UE. The MPR requirement other than MPRnarrow can reuse the values defined for PC3.

Proposal 6: Define MPRnarrow=7dB for the new FWA UE, other MPR requirement reuse the values defined for PC3.

Draft LS to RAN2 on new FWA UE RF requirement in Rel-17

Title:	draft LS on PTRS configuration for EVM measurement

Response to:

Release:	Release 17

Work Item:	NR_FR2_FWA_Bn257_Bn258

Source:	3GPP TSG RAN WG4

To:	RAN2

Cc:

Contact Person:

- Name:	Zhangqian

- E-mail Address:	zhangqian67@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

In Rel-17, RAN4 has agreed to introduce 2 power class requirements for NR new FR2 FWA UE. The 2 power class requirements are defined as below:

| No. | Max EIRP dBm | Max TRP dBm | Min peak EIRP dBm | Min spherical coverage EIRP@85% CDF dBm |
| --- | --- | --- | --- | --- |
| Power class 5 | 43 | 23 | 32 | 24 |
| Power class 2 with additional signalling on spherical coverage EIRP | 43 | 23 | 29 | 21 |

For UE capability signalling on power class, RAN4 agrees to introduce a new power class 5 for UE with min peak EIRP of 32dBm. For UE with min peak EIRP of 29dBm will reuse the capability of power class 2 with additional signalling on the spherical coverage requirement of 21dBm @85% CDF for the new FWA UE.

2. Actions:

To: 3GPP TSG RAN WG2.

ACTION: RAN4 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN4 Meeting #97-e		12th – 16th Oct 2020		               Electronic meeting

TSG RAN4 Meeting #98		1th – 5th March 2021	                  Athens, Greece

Conclusion

In this contribution we discussed on the open issues on new FWA UE RF requirement, according to the analysis, we have the following proposals:

Observation 1: When Tx power is lower than 30dBm, we can see 64QAM is generally not possible for uplink transmission on 28GHz.

Observation 2: To make sure on min peak EIRP>30dBm and max TRP is ensured be ≤23dBm, 16 antenna elements is the best choice.

Proposal 1: Agrees on 16 antenna elements assumption for the new FWA UE.

Proposal 2: Define minimum peak EIRP for the new FWA UE as 32dBm.

Proposal 3: Define maximum peak EIS for the new FWA UE as -93.3dBm.

Proposal 4: Considering different implementations, introduce 2 power classes for the new FWA UE:

  - Power class 5 with 32dBm min peak EIRP

  - Power class 6 with 29dBm min peak EIRP

    - Or reuse Power class 2, with additional signalling on 85% spherical coverage requirement.

Proposal 5: For new FR2 FWA UE, specify the multi-band relaxation requirement per band as 0.7dB for both peak and spherical requirement.

Proposal 6: Define MPRnarrow=7dB for the new FWA UE, other MPR requirement reuse the values defined for PC3.

- References

[1] R4-2008175, “New FR2 FWA UE RF requirement”, Huawei, HiSilicon, RAN4 #95-e

[2] R4-2006776, “FR2 23dBm FWA pk. EIRP and REFSENS budgets”, Qualcomm, RAN4 #95-e

[3] R4-2006703, “Spherical EIRP EIS of FR2 FWA UE with maximum TRP of 23dBm”, MediaTek, RAN4 #95-e

[4] R4-2007110, “UE RF requirements for new FR2 FWA use case”, Intel, RAN4 #95-e

[5] R4-2006432, “Discussion on new FR2 FWA UE”, Samsung, RAN4 #95-e

[6] R4-2008921, “WF on FR2 FWA evaluations”, Qualcomm, RAN4 #95-e

[7] R4-2008922, “WF on FR2 FWA RF requirement”, Huawei, HiSilicon, RAN4 #95-e

[8] R4-2008964, “email discussion on new FR2 FWA”, SoftBank, RAN4 #95-e