3GPP TSG-RAN WG4 Meeting #96-e		                               R4-2011523

Electronic Meeting, 17 August – 28 August, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	Transient Period Capability for NR FR1

Agenda Item:	7.19.1 UE transient period capability [TEI16]

Document for:	Approval

# Introduction

At RAN4#95-e, it has been agreed in Way Forward (WF) [1] that the RF requirements on the optional Transient Period “tp” capability are introduced in Release 16. No agreement was reached on testability issues. A draft-CR [2] was presented which proposes a set of EVM definition together with a new time-mask to verify the UE declared “tp” value. In this contribution, we present analytical and experimental measurements that confirm our previous online comments that some errors remains in one EVM definition in [2].We then re-iterate our previous set of EVM definition proposal to avoid introducing new FFT measurement windows. We also propose to align the UE declared ‘tp’ with the created EVM exclusion period for the sake of technical accuracy.

# Discussion

Draft-CR [2] proposes:

- Four transient period ‘tp’ capability values: 1,2,4,7s,

- To verify the UE ‘tp’declaration with new time mask reproduced here below in Figure 1

- To verify the impact of the UE transient by verifying EVM only the symbols where the transient occurs. In other words, the EVM in the remaining symbols is not verified,

- A set of four EVM definitions, one for each UE ‘tp’ declaration as reproduced here in

Figure 1: Proposed Time-mask in Draft-CR [2] to verify UE 'tp' declaration - Reproduced from [2] Figure 6.3.3.7-3

Table 1 Proposed set of EVM definitions in Draft-CR [2] to verify UE declared 'tp' – Reproduced from [2] Table 6.4.2.1a-1.

| Reported transient capability (us) | EVM definition | SCS |
| --- | --- | --- |
| 1 | EVM=max⁡(EVMl,EVMh) | 30 kHz |
| 2 | EVM=max⁡(EVMl,EVMh) | 15 kHz |
| 4 | EVM=EVMm, | 30 kHz |
| 7 | EVM=min⁡(EVMl,EVMh) | 15 kHz |
| NOTE 1: EVMl, EVMh, and EVMm are defined in Annex F |  |  |

We have previously commented online that the EVM definition for 4s UE ‘tp’ capability is erroneous at RAN4# 94-e-Bis and RAN4#95-e and we present in the next section some evaluation results. Based on this analysis, we propose to come back to the joint proposal that was discussed at length and in details throughout several ad-hocs meetings at RAN4#93 [3].

## Draft-CR [2] Evaluation Results

In [2], four EVM definitions have been proposed, one for each of the proposed UE ‘tp’ declaration of 1,2,4, and 7s. The goal of each EVM definition is to create an EVM Exclusion Period ‘ep’ so that EVM is excluded from the reported EVM for a duration that aims to be as close to the declared UE ‘tp’ duration as possible. In this paper we denote:

- EVM ‘ep’ lower edge to refer to the start position of the EVM exclusion period relative to the slot/symbol boundary where the transient occurs. We adopt a negative value expressed in microseconds for the lower edge to reflect the time-mask diagram reproduced in Figure 1,

- EVM ‘ep’ upper edge to refer to the end position of the EVM exclusion period, relatice to to the slot/symbol boundary where the transient occurs. We adopt a positive value expressed in microseconds for the upper edge to reflect the time-mask diagram reproduced in Figure 1,

- The width of the EVM ‘ep’ is then defined as “EVM ep upper edge” – “EVM ep lower edge”.

Table 2 below summarises the results of both theoretical calculation and the measured lower, upper and width the EVM exclusion period for each of the four EVM definitions proposed in [2]. Details of the measurement test setup and measurement results can be found in the Annex. The test setup is identical that used in our previous contribution [4].

Table 2 Evaluation results of Draft-CR [2] EVM definitions specified to verify UE 'tp' declaration

| Draft-CR [2] proposal |  |  | Evaluation Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Calculated EVM exclusion period boundaries (s) |  |  | Measured EVM exclusion period boundaries (s) |  |  |
| Reported transient capability (s) | EVM definition | SCS (kHz) | Lower Edge | Upper Edge | Exclusion Period “Width” | Lower Edge | Upper Edge | Exclusion Period “Width” |
| 1 | EVM=max⁡(EVMl,EVMh) | 30 | -0.586 | 1.107 | 1.693 | -0.6 | +1.1 | 1.7 |
| 2 | EVM=max⁡(EVMl,EVMh) | 15 | -1.172 | 1.693 | 2.865 | -1.15 | +1.65 | 2.8 |
| 4 | EVM=EVMm, | 30 | -1.172 | 1.693 | 2.865 | -1.15 | +1.65 | 2.8 |
| 7 | EVM=min⁡(EVMl,EVMh) | 15 | -3.516 | 4.036 | 7.552 | -3.5 | +4.0 | 7.5 |

We make several observations.

Observation 1: For a UE that declares 1s ‘tp’ capability, the effective EVM ‘ep’ width is approximately 1.7s wide, ie. in excess of 69% over the declared value,

Observation 2: For a UE that declares 2s ‘tp’ capability, the effective EVM ‘ep’ width is approximately 2.8s wide, ie. in excess of 22% over the declared value,

Observation 3: For a UE that declares 4s ‘tp’ capability, the effective EVM ‘ep’ width is approximately 2.8s wide. This is erroneous and cannot be used for verification purposes. As previously commented, we believe the right SCS for this EVM definition should be 15kHz.

Observation 4: For a UE that declares 7s ‘tp’ capability, the effective EVM ‘ep’ width is approximately 7.5s wide.

Observation 5: For each EVM definition, the EVM ‘ep’ is asymmetrical, with the lower edge being always positioned at a greater distance relative the slot/symbol boundary where the transient occurs than the lower edge.

About observation 5, it should be noted that no matter what EVM definition is used that is based on legacy FFT measurement windows, or combinations of the FFT_low, FFT_high as currently defined at RAN4, the resulting EVM window will always have a slight mis-alignment. This is inherent to the physical layer design where the Cyclic Prefix (CP) duration of the first symbol in a slot is always greater than the duration of the CP of the last symbol in a slot. We also not however, that this should impact uplink link budget since this the results of having selected only 2 FFT windows for EVM measurements, respectively defined as 25% and 75% of the CP length. An uplink demodulator would most likely have a lot more freedom to carefully select the FFT window that maximizes uplink SNR. Also note that the question of EVM ‘ep’ asymmetry could be resolved by adopting hybrid, custom tailored FFT positions (expressed as % of CP length) for each symbol where the transient occurs. By such careful selection, it is possible to create EVM ‘ep’ symmetrically centered. This is however not our preference. We prefer to adopt EVM definition that rely solely on legacy FFT windows so as to minimize impact on legacy EVM test equipment and minimize changes in EVM test procedure.

Observation 6: It is possible to create symmetrical EVM ‘ep’ by specifying hybrid, custom tailored FFT windows for each symbol where the transient occurs. This is not our preferred approach. We prefer adopting EVM definitions that rely solely on legacy FFT windows

## Counter proposal

The motivation to make counter proposal is two-fold:

- Errors in the proposed EVM definition for 4s ‘tp’ capability in [2] requires a technical correction,

- EVM ‘ep’ are for certain ‘tp’ capabilities somehow too “wide” and needs either an alternative EVM definition proposal or an alignment between the proposed ‘tp’ capability values and the effective EVM ‘ep’ width.

We therefore make the following proposals:

Proposal 1: To minimize impact on legacy test equipment, the EVM definition that leads to an effective EVM exclusion period shall rely solely on the legacy FFT_low, FFT_high measurement windows.

Proposal 2: 1s UE capability is not needed as it brings no benefit to operation at any FR1 SCS (15,30 or 60kHz)

Proposal 3: To align as closely as possible the UE declared ‘tp’ with the effective EVM ‘ep’ width, introduce UE ‘tp’ capability of 2.2, 4 and 7.5s with the set of EVM definitions proposed in Table 3:

Table 3 EVM definition set counter-proposal to verify UE 'tp' capability declaration

| Counter proposal |  |  | Evaluation Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Calculated EVM exclusion period boundaries (s) |  |  | Measured EVM exclusion period boundaries (s) |  |  |
| Reported transient capability (s) | EVM definition | SCS (kHz) | Lower Edge | Upper Edge | Exclusion Period “Width” | Lower Edge | Upper Edge | Exclusion Period “Width” |
| 1 | Capability not needed |  |  |  |  |  |  |  |
| 2.22 | EVM=min⁡(EVMl,EVMh) | 60 | -0.878 | 1.400 | 2.278 | -0.9 | +1.2 | 2.1 |
| 4 | EVM=min⁡(EVMl,EVMh) | 30 | -1.758 | 2.279 | 4.036 | -1.7 | +2.2 | 3.9 |
| 7.5 | EVM=min⁡(EVMl,EVMh) | 15 | -3.515 | 4.036 | 7.552 | -3.5 | +4.0 | 7.5 |
| NOTE 1: EVMl, EVMh, are defined in Annex F NOTE 2: 2.2s capability is restricted to UEs supporting SCS 60kHz |  |  |  |  |  |  |  |  |

Proposal 4: Since 2.2s ‘tp’ capability brings no benefit to operation at SCS 15kHz or at SCS 30kHz, introduce 2.2s ‘tp’ capability as being restricted to UEs supporting SCS60kHz.

There are four advantages to Proposal 3 and 4:

- It prevents adding/creating new FFT measurement windows in Annex F, thereby simplifying implementation in test equipment,

- Since both FFT_low and FFT_high measurements are captured and measured by test equipment symbol per symbol, this enables verifying not only the EVM in the symbols where the transient occurs, but also in the remaing symbols.

- Advantage 2 means that in turns, we can enable ensuring that long term effects, such Power Amplifier thermal drift after sudden power change, can be verified on the remaining symbols where the transient no longer occurs.

- Finally, since the EVM measurement is enabled in both the symbols with transients, and symbols where the transient does not occur, the conformance to legacy static EVM core requirements can also be checked simulateneously, thereby minimizing the number of additional test associated with this feature.

Observation 7: The alignment of UE ‘tp’ declaration bridges the gap with the effective EVM ‘ep’ width.

Observation 8: For a UE that declares 4s ‘tp’ capability, the EVM ‘ep’ is nearly exactly symmetrically centered.

Proposal 5: If no capability is signaled, the default transient period value of 10s applies and the UE is tested against legacy static EVM requirements only.

Proposal 6: When a UE signals a transient period capability, the UE must pass 2 core requirements using time-mask in Figure 1:

- For the each PUSCH symbols where the transient occurs:

  - rmsEVM shall not exceed [10%] for 64QAM and [5%] for 256 QAM.

  - The rms average of the basic EVM is averaged over [108] subframes for each symbol where the transient occurs,

- For the remaining PUSCH symbols where the transient does not occur:

  - rmsEVM shall not exceed [8%] for 64QAM and [3.5%] for 256QAM (Table 6.4.2.1-1 requirements),

  - The rms average of the basic EVM is averaged over [12] subframes.

Proposal 7: : When a UE signals a transient period capability, the static EVM does not need to be verified since it is verified in symbols where the transient does not occur (see proposal 6).

Observation 8: All measurements presented in this paper verified all EVM definitions using off the shelf commercial NR EVM meter equipment. This proves there are no testability issues.

# Conclusion

In this contribution we address WF [1] remaining open items and we make several proposals to verifiy UE transient period declaration.

Proposal 1: To minimize impact on legacy test equipment, the EVM definition that leads to an effective EVM exclusion period shall rely solely on the legacy FFT_low, FFT_high measurement windows.

Proposal 2: 1s UE capability is not needed as it brings no benefit to operation at any FR1 SCS (15,30 or 60kHz)

Proposal 3: To align as closely as possible the UE declared ‘tp’ with the effective EVM ‘ep’ width, introduce UE ‘tp’ capability of 2.2, 4 and 7.5s with the set of EVM definitions proposed in Table 3:

Table 3 EVM definition set counter-proposal to verify UE 'tp' capability declaration

| Counter proposal |  |  | Evaluation Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Calculated EVM exclusion period boundaries (s) |  |  | Measured EVM exclusion period boundaries (s) |  |  |
| Reported transient capability (s) | EVM definition | SCS (kHz) | Lower Edge | Upper Edge | Exclusion Period “Width” | Lower Edge | Upper Edge | Exclusion Period “Width” |
| 1 | Capability not needed |  |  |  |  |  |  |  |
| 2.22 | EVM=min⁡(EVMl,EVMh) | 60 | -0.878 | 1.400 | 2.278 | -0.9 | +1.2 | 2.1 |
| 4 | EVM=min⁡(EVMl,EVMh) | 30 | -1.758 | 2.279 | 4.036 | -1.7 | +2.2 | 3.9 |
| 7.5 | EVM=min⁡(EVMl,EVMh) | 15 | -3.515 | 4.036 | 7.552 | -3.5 | +4.0 | 7.5 |
| NOTE 1: EVMl, EVMh, are defined in Annex F NOTE 2: 2.2s capability is restricted to UEs supporting SCS 60kHz |  |  |  |  |  |  |  |  |

Proposal 4: Since 2.2s ‘tp’ capability brings no benefit to operation at SCS 15kHz or at SCS 30kHz, introduce 2.2s ‘tp’ capability as being restricted to UEs supporting SCS60kHz.

Proposal 5: If no capability is signaled, the default transient period value of 10s applies and the UE is tested against legacy static EVM requirements only.

Proposal 6: When a UE signals a transient period capability, the UE must pass 2 core requirements using time-mask in Figure 1:

- For the each PUSCH symbols where the transient occurs:

  - rmsEVM shall not exceed [10%] for 64QAM and [5%] for 256 QAM.

  - The rms average of the basic EVM is averaged over [108] subframes for each symbol where the transient occurs,

- For the remaining PUSCH symbols where the transient does not occur:

  - rmsEVM shall not exceed [8%] for 64QAM and [3.5%] for 256QAM (Table 6.4.2.1-1 requirements),

  - The rms average of the basic EVM is averaged over [12] subframes.

Proposal 7: : When a UE signals a transient period capability, the static EVM does not need to be verified since it is verified in symbols where the transient does not occur (see proposal 6).

Observation 8: All measurements presented in this paper verified all EVM definitions using off the shelf commercial NR EVM meter equipment. This proves there are no testability issues.

# References

[1]		R4-2008477, WF on feasibility of testing the transient periods, CMCC, 3GPP TSG-RAN WG4 Meeting #95-e.

[2] 	R4-2007132, Transient Period Capability in NR using existing window definitions, Qualcomm Incorporated, Verizon, Dish Network, Ericsson, CMCC, Keysight Technologies, Nokia, Nokia Shanghai Bell, AT&T, ZTE, Vodafone, Orange, T-Mobile USA, Deutsche Telekom, Telecom Italia, CHTTL, China Telecom, SGS Wireless, Interdigital, 3GPP TSG-RAN WG4 Meeting #95-e.

[3] 	R4-1916016, WF on Transient period capability, Qualcomm Incorporated, Skyworks Solutions Inc., Ericsson, Verizon, Dish Network, CMCC, Keysight Technologies, Nokia, Nokia Shanghai Bell, Sprint, AT&T, ZTE, Vodafone, Orange, T-mobile, Deutsche Telekom, Telecom Italia, CHTTL, China Telecom, SGS Wireless, Interdigital, Anritsu, 3GPP TSG-RAN WG4 Meeting #93.

[4] 	R4-2002143, EVM Measurements for FR1 Transient Period Capability Testability,Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #94-e.

# Annex – Experimental measurement results

## Test Conditions and PA Calibration

##### Tested Channel Combinations and Waveforms

20MHz Cell Bandwidth, fully allocated waveforms are used for all measurements. Only CP-OFDM waveforms are tested since, contrary to DFT-s-OFDM, CP-OFDM waveforms cannot benefit from FFT sample exclusion, ie, it is not possible to tailor the exclusion period with fine granularity. Therefore, verifying if a the UE declared “tp” is truly met remains a challenging testability topic for CP-OFDM waveforms.

Waveform properties:

- CP-OFDM, SCS 15,30 and 60kHz

- 20MHz CBW, fully allocated, ie 106RB0, 51RB0, and 24RB0 at SCS15,30 and 60kHz respectively.

- Modulation schemes: 64QAM and 256QAM

- 14 symbols / slot. We denote symbol 0 and 13 the first and last symbol of each slot respectively, and “OS” to refer to an OFDM Symbol.

- DMRS: CDM groups 2, Type A position 2, 2 additional DMRS positions, ie a total of 3 DMRS symbols per slot

- PUSCH mapping type A.

##### RF Transient Characteristics and EVM Measurement Procedure

The RF transient is generated with a 20dB step-up power change emulating a 1:100 RB allocation change at slot boundaries. All measurements are performed on a 2600MHz modulated carrier.

The transient profile characteristics are:

- 20dB step-up power change,

- Exponential rise profile as shown in Figure 2,

- RF Transient start position or Timing Advance: programmable in 10ns steps

- RF Transient duration: programmable in 10ns steps.

Figure 2: Exponential RF transient profile example used for measurements: measured PA RF transient vs digital IQ input stimulus (green - bottom right)

For EVM measurements:

- For every symbol, an RF Vector Signal Analyzer is used to capture both FFT_low and FFT_high EVM measurement reports. The EVM values are averaged over 10ms,

- Once captured, each EVM data is post-processed to evaluate both EVM=max(EVM_l,EVM_h) and EVM=min(EVM_l,EVM_h), symbol per symbol across all slots.

##### PA Intrinsic EVM Floor Calibration

Throughout this document we use “Pre-event” to refer to the slot prior to the start of RF transient event, and “Post-event” to refer to the slot where the transient settles/ends. We calibrate the modulated carrier EVM floor by adjusting the RF transient “high power level” to reach the following EVM levels using the current EVM definition of EVM=max(EVM_l, EVM_h):

- 256QAM: Post-event or high power level rmsEVM floor: 2% , Pre-event, or ‘Low power level rmsEVM’ <1%

- 64QAM: Post-event or high power level rmsEVM floor: 6%. Pre-event, or ‘Low power level rmsEVM’ <1%

Calibration results of the Pre and Post-event slot rmsEVM per symbol are reported in Figure 3 below.

Figure 3: rmsEVM per symbol in Pre-event and Post-event slots for 20dB step-up power change, 10ns RF transient period centered at slot boundaries. Left: 64QAM (transition from <1% to 5.9% rms EVM), Right: 256QAM (transition from <1% to 2% rms EVM). The vertical red dashed line represents the RF transient location.

The high power level calibration is performed with a near instantaneous RF transient period (tp=10ns) centered at slot boundaries as shown in Figure 4. It can be seen that the resulting PA high power levels are 25.1 dBm for 64QAM and 20.3 dBm for 256QAM.

Figure 4: PA output power profile for 20dB step-up power change to achieve 64QAM 0.8% to 6% EVM transition (left) and 256QAM 0.8% to 2% rms EVM transition (right) . The vertical red dashed line represents the RF transient location.

## Measurement Results

### Evaluation Results of Draft-CR EVM Definitions [2]

1.7s ‘ep’

for

2s ‘tp’

2.8s ‘ep’

for

2 & 4s ‘tp’

7.5s ‘ep’

for

7s ‘tp’

Figure 5: Pre-event and Post-event rmsEVM vs transient timing advance for each of the EVM definitions in [2].

### Evaluation Results of our EVM counter-Proposal

2.2s ‘ep’

for

2.2s ‘tp’

4s ‘ep’

for

4s ‘tp’

7.5s ‘ep’

for

7.5s ‘tp’

Figure 6: Pre-event and Post-event rmsEVM vs transient timing advance for each of the EVM definitions in [2].