3GPP TSG-RAN WG4 Meeting #96-e                                  R4-2012613

Electronic Meeting, 17 – 28 August, 2020

Source: 	Mediatek

Title: 	Simulation assumptions for NR V2X test cases

Agenda Item:	7.3.7.3

Document for:	Approval

1 Introduction

In this contribution, we provide the simulation assumptions for NR V2X Demod requirements only for evaluations initial alignment purpose if possible.

The company is encouraged to submit the simulation results but it doesn’t mean any test case will be introduced. Whether to introduce any test is pending on the agreement from test scope discussion.

The values with different options or [ ] need be updated in the next RAN4 meeting based on further discussion.

2 Simulation assumptions

- PSSCH Demodulation test

The simulation assumptions for PSSCH performance requirements are listed as below:

Option 1:

| Parameters | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 | [SCH_Test3] |
| Synchronization source |  | GNSS | GNSS | GNSS |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 | 600 |
| Propagation condition |  | [TDLA30-2700] | [TDLA30-180] | [TDLA30-180] |
| Channel bandwidth | MHz | 20 | 20 | 20 |
| Allocated resource blocks |  | [15] | [15] | [15] |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| CP-OFDM symbols per slot (Note 1) |  | [9] | [9] | [9] |
| DMRS position in time domain (symbol index) |  | [#1,#4,#7] | [#1,#4,#7] | [#1,#4,#7] |
| Modulation order |  | QPSK | 64QAM | 256QAM |
| MCS index |  | [4] | [17] | [20] |
| The number of REs for 2nd stage SCI |  | [240] | [174] | [TBD] |
| Transport Block Size | Bits | [552] | [2536] | [TBD] |
| Transport block CRC | Bits | 24 | 24 | 24 |
| Maximum number of HARQ retransmissions |  | 0 | 0 | 0 |
| Binary Channel Bits (Note 2) | Bits | [1860] | [5976] | [TBD] |
| Antenna configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@[10]%BLER |  |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on [every slot]. Note 5: SCH Test 3 is listed for further technical discussion for 256QAM demodulation, whether to introduce this test case is pending agreement from scope discussion. The simulation result is optional in next meeting. |  |  |  |  |

- Option 2

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 |
| Synchronization source |  | GNSS | GNSS |
| Propagation condition |  | TDLA-30-1400 | TDLA-30-2700 |
| Channel bandwidth | MHz | 20MHz | 20MHz |
| Allocated resource blocks | RB | [15] | [15] |
| Subcarrier spacing | kHz | 30 | 30 |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 |
| CP-OFDM symbols per slot (Note 1) |  | [12] for slots without PSFCH 9 for slots with PSFCH | [12] for slots without PSFCH 9 for slots with PSFCH |
| The number of DMRS symbols |  | [3] for slot without PSFCH;[2] for slot with PSFCH | [4] for slot without PSFCH;[3] for slot with PSFCH |
| Modulation order |  | [4] | [2] |
| MCS index |  | [11](4,378/1024) | [4](2,308/1024) |
| The number of REs for 2nd stage SCI (SCI format 2-A) |  | [270](βoffset=7,α=1) | [276] (βoffset=3,α=1) |
| Transport Block Size | Bits | [2088] for slots without PSFCH [1320] for slots with PSFCH | [808] for slots without PSFCH [480] for slots with PSFCH |
| Transport block CRC | bits | 24 | 24 |
| Maximum number of HARQ transmissions |  | 1 | 1 |
| Binary Channel Bits (Note 2) | Bits |  |  |
| periodPSFCHresource |  | 4 | 4 |
| Antenna configuration |  | 1x2 Low | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@10%BLER |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on every [4] slot. |  |  |  |

- PSCCH Demodulation test

The simulation assumptions for PSCCH performance requirements are given in Table 2.

Table 2 Simulation assumption for PSCCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | [TDLA30-180, TDLA30-1350] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 2 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | [28] |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 360 |
| PSCCH performance metric (Note 1) |  | SNRPSCCH@1%BLER |
| Note 1: The performance metric is used for PSCCH simulation evaluation. Note 2: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 3: Two propagation conditions are listed as options for alignment purpose, one of the option will be chosen for defining requirement |  |  |

- PSBCH Demodulation test

The simulation assumptions for PSBCH performance requirements are given in Table 3.

Table 3 Simulation assumption for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | S-SSB |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period(NperiodS-SSB) |  | [1, 2] |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | 0 |
| Frequency offset | Hz | 0 |
| The number of symbols (Note 1) |  | 8 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| PSBCH performance metric (Note 2) |  | SNRPSBCH@1%BLER |
| Note 1: The first symbol is for AGC. Note 2: The performance metric is used for PSBCH simulation assumption. |  |  |

- PSFCH Demodulation test

The simulation assumptions for PSFCH performance requirements are given in Table 4.

Table 4 Simulation assumption for PSFCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH period | Slot | [1] |
| Cyclic shift pair index |  | 1 or 2 |
| PSFCH performance metric (Note 1) |  | [Pr(ACK miss) < 1%], [Pr(NACK to ACK) < 0.1%], [Pr(DTX to ACK)<1%] |
| Note 1: The performance metric is used for PSFCH simulation assumption. [Note 2: The 1st PSFCH which is used for AGC doesn’t need to be considered in performance evaluation.] |  |  |

- PSCCH and PSSCH decoding capability

The options for simulation assumptions for PSCCH and PSSCH decoding capability are given in Table 5-1, 5-2.

Option 1:

Table 5-1 Simulation assumption for PSCCH decoding demodulation performance

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | [20] |
| Communication resource pool configuration |  |  | [5] subchannels, [10]RB subchannel size |
| Sidelink UE i, 0 ≤ i ≤ [4] (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | [QPSK, 0.3 (MCS 4), 10RB] |
|  | PSCCH RMC |  | The same as PSCCH decoding |
| Target requirement |  | SNR @ PSCCH BLER 1% |  |
| Note 1: Time offset of Sidelink UE received signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 4: Each UE occupies one sub-channel, hence all the sub-channels are filled by the 5 active SL UEs. |  |  |  |

Option 2:

Table 5-2 Simulation assumption for PSCCH decoding demodulation performance

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | [40] |
| Communication resource pool configuration |  |  | [10] subchannels, [10]RB subchannel size |
| Sidelink UE i, 0 ≤ i ≤ [9] (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | [QPSK, 0.3 (MCS 4), 10RB] |
|  | PSCCH RMC |  | The same as PSCCH decoding |
| Target requirement |  | SNR @ PSCCH BLER 1% |  |
| Note 1: Time offset of Sidelink UE received signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 4: Each UE occupies one sub-channel, hence all the sub-channels are filled by the 5 active SL UEs. |  |  |  |

- PSFCH decoding capability

The simulation assumptions for PSFCH decoding capability are given in Table 6.

Table 6 Simulation assumption for PSFCH decoding demodulation performance

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| SCS |  | kHz | 30 |
| Active cell(s) |  |  | None |
| Sidelink UE i, 0 ≤ i ≤ [14] | Sidelink Transmissions |  | PSFCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSFCH period |  | [1] |
|  | Test metric |  | [1% of ACK missed detection probability] [1% of DTX to ACK probability] [0.1% NACK to ACK detection probability] |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: 15 sidelink UEs should transmit one by one for every RB per PSFCH |  |  |  |

3 Conclusions

In this contribution, we give the simulation assumptions for NR V2X Demod performance. Simulation results can be collected based on these assumptions in next meeting.

4 References