3GPP TSG-RAN4 Meeting #96-e	R4-2011100

# Electronic Meeting, 17th – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1052 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new default configurations for RMC scheduling_r16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Cat. A CR of R4-2012067 In current default configuration for RMC shceduling (CCR.1.1 TDD/FDD), 24 RBs are allocated for CORESET and Aggregation level is set to 8. These setting are incompatible with test frequencies setting given in 38.508-1 under some cases. Take band n3 for example. Test frequency for band n3 mid range is given in 38.508-1 Table 4.3.1.1.1.3-1 as follows So SSB for mid range of band n3 is 102 RB+2 subcarrier away from point A. If we want to verify TC A.6.6.1.3 on this band, then the start position of active DL BWP (DLBWP.1.2 is used in 6.6.1.3) shall be 103 RB away from point A according to table A.3.9.2.2-1 note 1: BWP Parameters; Unit; Values. Reference BWP; DLBWP.1.2. Starting PRB index; RBb Note 1. Bandwidth; RB; 25 for SCS = 15KHz, 51 for SCS = 30KHz, 32 for SCS = 120KHz. Note 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. Note 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10.. However, according to RAN1 specs, start position and allocated RB size of CORESET for scheduling RMC shall both be multiple of 6 RBs. As a result, start position of CORESET for scheduling RMC can only be 108 RBs away from point A. Then the 24RB CORESET can not be completely contained in active DL BWP, active DL BWP can only contain a 18 RB CORESET, But 18 RB is not enough for a PDCCH with aggregation level = 8 because 6 (CCE size)/2(duration of CORESET)*8(Aggregation level) = 24RB are needed. To solve this issue and avoid impact to RAN5’s other test works. there seems to be no other way but introduce a new configuration of 18RB size and Aggregation level = 4 for RMC scheduling, and use this new configuration in TC A.4.6.1.3/4/6 and A.6.1.1.3/4/6. |
| Summary of change: |  | - new configuration of 18RB size and Aggregation level = 4 for RMC scheduling is introduced for 15KHz cases. - Note 3 in table A.3.1.3.1-1/A.3.1.3.2-1 are updated according to Anritsu's comments received in 1st round. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE can’t pass the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.3.1, A.3.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.3.1.3	CORESET for RMC scheduling

A.3.1.3.1	FDD

Table A.3.1.3.1-1: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |  |  |
| Aggregation level | CCE | 8 | 4 |  |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

A.3.1.3.2	TDD

Table A.3.1.3.2-1: Control Channel RMC for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD |  |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |  |  |
| Aggregation level | CCE | 8 | 4 |  |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-2: Control Channel RMC for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 2 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-3: Control Channel RMC for TDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 24 |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 |  |  |  |  |
| monitoringSlotPeriodicityAndOffset |  | sl160 0 | sl160 0 | sl160 80 |  |  |  |  |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 |  |  |  |  |
| Duration of CORESET | slot | 1 | 1 | 1 |  |  |  |  |
| REG bundle size |  | 6 | 6 | 6 |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved |  |  |  |  |
| Interleave n_shift |  | 0 | 0 | 0 |  |  |  |  |
| Interleave size |  | 2 | 2 | 2 |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A |  |  |  |  |
| Aggregation level | CCE | 8 | 8 | 8 |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. |  |  |  |  |  |  |  |  |

<End of modified section 1>