3GPP TSG-RAN4 WG4 Meeting # 96-e		R4-2011724

# Electronic meeting, 17- 28 August, 2020

Title:	LS on FR1 intra-band UL CA UE capability

Response to:

Release:	Release 16

Work Item:	NR_RF_FR1-core

Source:	WG4

To:	RAN2

Cc:

Contact Person:

#### Name:	Zhangqian

E-mail Address:	zhangqian67@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

After 2nd round discussion on UE capability for FR1 intra-band UL CA in Rel-16, RAN4 has reached further agreement intra-band CA features in RAN4 feature list.

For FR1 intra-band non-contiguous CA, UL frequency separation class is agreed to be introduced which indicate the UL frequency separation class UE can support which includes both the aggregated bandwidth and gap bandwidth between two non-contiguous CCs, with per band combination manner. For the UL frequency separation class, it is agreed to introduce 3 classes as below:

| NR NC CA frequency separation class | frequency separation |
| --- | --- |
| I | 20 MHz ≤ BWChannel_NC_CA ≤ 100 MHz |
| II | 100 MHz < BWChannel_NC_CA ≤ 200 MHz |
| III | 200MHz<BWChannel_NC_CA≤ 600MHz |

For FR1 intra-band contiguous CA, it is agreed to clarify in the feature that components of UL Bandwidth class, PA architecture and MIMO layer are indicated with dependency relation. So bracket can be removed for component 2 and component 3 in feature index 7-3b.

In summary, feature group 7-3a and 7-3b in RAN4 feature list are further agreed as below:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7-3a | NR CA class List for intra-band non-contiguous CA | Indicate the UL frequency separation class that UE can support which includes both the aggregated bandwidth and the gap bandwidth between two non-contiguous CCs for intra-band non-contiguous CA Note: UL frequency separation class means maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier that UE can support in uplink | Intra-band UL non-contiguous CA band combination | Yes | N/A | Network cannot schedule intra-band non-contiguous UL CA transmission properly | Type 3 (per BC) | No need | FR1 only | N/A | Based on the agreed WF R4-2005660 both 1PA and 2PA architecture for intra-band non-contiguous UL CA will be considered for UE capability, and MIMO supporting with 4TX for 2PA UL NC CA should not be excluded. The maximum UL CC number for intra-band UL CA is 2 in Rel-16. NC CA ULseparation class candidate values: - ClassI: NC CA separation class≤ 100MHz - ClassII: 100< NC CA separation class≤ 200MHz - ClassIII: NC CA separation class > 200MHz and <600MHz | Optional with capability signalling |
| 7-3b | NR CA class List for Intra-band contiguous CA | - Indicate the contiguous CA bandwidth class that UE can support in uplink - On the condition that component 1 is indicated, indicate the PA architecture, i.e, 1PA or 2PA - On the condition that component 1 and component 2 are indicated, indicate the MIMO layer number for each UL CC separately NOTE1: there is dependency for the three components as given above NOTE2: component 1/2/3 are existing signaling from Rel-15, the dependency and conditioned relation need to be ensured in Rel-16 signalling. It is up to RAN2 to decide how to ensure dependency and conditioned relation or new Rel-16 signaling is needed. | Intra-band UL contiguous CA band combination | Yes | N/A | Network cannot schedule intra-band contiguous UL CA transmission properly | Type3 | No need | FR1 | N/A | for each contiguous CA bandwidth class, if 2PA architecture is indicated, MIMO is not supported for both UL CCs by default | Optional with capability signalling, |

2. Actions:

To: 3GPP TSG RAN WG2.

ACTION: RAN4 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #97-e		Oct. 26	– Nov.13, 2020			Online

TSG RAN WG4 Meeting #98		Mar. 01	– 05, 2021			Athens, Greece