3GPP TSG-RAN4 Meeting #97-e	R4-2014280

# Online, 2-13 Nov., 2020

Agenda item:	7.19.5

Source:	Apple

Title:		Discussion on R16 IDLE/INACTIVE RRM requirement with SMTC2-LP

Document for:	Discussion

# Introduction

In last RAN plenary #89e meeting we proposed this IDLE/INACTIVE RRM requirement with SMTC2-LP in R17 RRM further enhancement WI, but eventually it was not included in that WI. Since RAN2 has already specified SMTC2-LP in R16, it would be also reasonable to also capture the corresponding RAN4 requirement in TEI16

In this contribution, we discuss the RAN4 IDLE/INACTIVE RRM requirement with SMTC2-LP.

# Discussion

- The SMTC2-LP definition in RAN2 spec TS38.331:

In TS38.331, it was defined that,

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselecion), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

The signalling design is as below for both intra-frequency and inter-frequency measurement in IDLE/INACTIVE mode,

smtc2-LP-r16 Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP-r16. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP-r16 can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP-r16 cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP-r16 is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity.

smtc2-LP-r16 Measurement timing configuration for inter-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP-r16. The timing offset and duration are equal to the offset and duration indicated in smtc in InterFreqCarrierFreqInfo. The periodicity in smtc2-LP-r16 can only be set to a value strictly larger than the periodicity in smtc in InterFreqCarrierFreqInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP-r16 cannot be configured). The pci-List, if present, includes the physical cell identities of the inter-frequency neighbour cells with Long Periodicity. If smtc2-LP-r16 is absent, the UE assumes that there are no inter-frequency neighbour cells with a Long Periodicity.

- The SMTC related requirement in RAN4 spec TS38.133:

In most of RRM requirements for IDLE/INACTIVE, we use DRX cycle to define the delay, but in some side conditions and interruption requirements, we used SMTC as the periodicity to determine the requirement. We list the all SMTC related RRM requirement as below in IDLE/INACTIVE mode,

#### 4.2.2.2 Measurement and evaluation of serving cell The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1.

#### 4.2.2.3 Measurements of intra-frequency NR cells Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 5; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1..

#### 4.2.2.4 Measurements of inter-frequency NR cells The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 under the following conditions: - TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and - SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and - SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1].

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms.

- Some R16 new features for IDLE and CONNECTED mode:

In R16 TS38.133, we have HST, NR-U and Power-saving requirement for IDLE/INACTIVE mode and some of those requirements are also related with SMTC configuration. However, since RAN4 has never discussed SMTC2-LP together with those R16 features, in this paper we only propose to specify SMTC2-LP for the baseline IDLE/INACTIVE RRM requirement, and for those R16 new features we think more discussion is needed before we decide to apply SMTC2-LP for those R16 features as well.

Proposal 1: the SMTC2-LP shall be captured in the corresponding baseline RRM requirements of IDLE/INACTIVE mode without R16 features (e.g. NR-U, HST and power saving).

Proposal 2: The applicability of SMTC2-LP in corresponding IDLE/INACTIVE RRM requirement for NR-U, HST or power saving is FFS in RAN4.

# Proposed revisions

In this meeting we had a CR [1] to capture the SMTC2-LP in the current baseline RRM requirement in IDLE/INACTIVE mode. The main revision is to clarify some of the SMTC configurations could be decided by SMTC2-LP if indicated.

#### 4.2.2.2 Measurement and evaluation of serving cell The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1. If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc.

#### 4.2.2.3 Measurements of intra-frequency NR cells Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 5; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc..

#### 4.2.2.4 Measurements of inter-frequency NR cells The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 under the following conditions: - TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and - SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and - SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1]. - If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the above SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc.

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the high layer signalling of smtc2-LP is configured in TS 38.331 [2], the SMTC periodicity follows smtc2-LP; otherwise the SMTC periodicity follows smtc.

# Conclusion

In this contribution, we discuss the RAN4 IDLE/INACTIVE RRM requirement with SMTC2-LP.

Proposal 1: the SMTC2-LP shall be captured in the corresponding baseline RRM requirements of IDLE/INACTIVE mode without R16 features (e.g. NR-U, HST and power saving).

Proposal 2: The applicability of SMTC2-LP in corresponding IDLE/INACTIVE RRM requirement for NR-U, HST or power saving is FFS in RAN4.

# References

[1] R4-2014281, CR on R16 IDLE/INACTIVE RRM requirement with SMTC2-LP, Apple, RAN4 #97e