3GPP TSG-RAN WG4 Meeting #97-e	R4-2014376

# Electronic Meeting, 2-13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1163 | rev |  | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of active BWP switch test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In active BWP switching test case, i.e. A.7.5.6.1.1 and A.7.5.6.1.2, PCell is configured with two BWPs (BWP-1 and BWP-2). However, in current specification, the sentence “UE shall be continuously scheduled on PSCell’s BWP-1 during T3” is incorrect. It is fixed in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the SCell’s to PCell’s BWP-1 in A.7.5.6.1.1 - Correct the PSCell’s to PCell’s BWP-1 in A.7.5.6.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect parameters will exist in spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

### A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA

A.6.5.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.6.5.6.1.1.1-1 below. The test scenario comprises of one NR PCell (Cell 1) and one SCell (Cell 2) as given in Table A.6.5.6.1.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.1.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PCell, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 2 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

-	UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch on PCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell (Cell 1).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch of PCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell is carried out in the correct time span by monitoring ACK/NACK sent in SCell during BWP switch of PCell, respectively.

Table A.6.5.6.1.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD -FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD - TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.6.1.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.6.5.6.1.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 | FR1 |
| Duplex mode |  | Config 1 |  | FDD | FDD |
|  |  | Config 2,5 |  | TDD | TDD |
|  |  | Config 3 |  | TDD | FDD |
|  |  | Config 4 |  | FDD | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  |  | Config 4 |  | Not Applicable | TDDConf.1.1 |
|  |  | Config 5 |  | TDDConf.1.2 | TDDConf.1.2 |
| BWchannel |  | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 | 10 MHz: NRB,c = 52 |
|  |  | Config 5 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1, 2 | 3 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |  |
| Active DL BWP-0 Configuration |  |  |  | N.A. | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1Note4 | N.A. |
| Active DL BWP-2 Configuration |  |  |  | DLBWP.1.3Note4 | N.A. |
| Active UL BWP-0 Configuration |  |  |  | N.A. | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1Note4 | N.A. |
| Active UL BWP-2 Configuration |  |  |  | ULBWP.1.3Note4 | N.A. |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
| measurement channel |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  |  | Config 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  |  | Config 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET |  | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
| parameters |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  |  | Config 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  |  | Config 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
| parameters |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  |  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  |  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2,3,4 |  | SSB.1 FR1 |  |
|  |  | Config 5 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 |  | dBm/SCS | -104 | -104 |
|  | Config 5 |  |  | -101 | -101 |
| NocNote 2 |  |  | dBm/15KHz | [-104 | -104 |
| SS-RSRP Note 3 | Config 1,2,3,4 |  | dBm/SCS | -87 | -87 |
|  | Config 5 |  |  | -84 | -84 |
| Ês/Iot |  |  | dB | [17 | 17 |
| Ês/Noc |  |  | dB | [17 | 17 |
| IoNote3 |  | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  |  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Propagation Condition |  |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.1.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

### >>>>>End of Change 1<<<<<

### >>>>>Start of Change 2<<<<<

### A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA

A.7.5.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.7.5.6.1.1.1-1 below. The test scenario comprises of one PCell (Cell 1) and one SCell (Cell 2) as given in Table A.7.5.6.1.1.1-2. NR Cell-specific parameters are specified in Table A.7.5.6.1.1.1-3 below. OTA related test parameters are shown in table A.7.5.6.1.1.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

UE is configured with 2 different UE-specific downlink bandwidth parts for PCell, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 2 before starting the test.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch on PCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell(Cell 1).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the half subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

T	he starting time of SCell (Cell 2) interruption due to BWP switch of PCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell is carried out in the correct time span by monitoring ACK/NACK sent in SCell during BWP switch of PCell, respectively.

Table A.7.5.6.1.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD -TDD duplex mode |

Table A.7.5.6.1.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.7.5.6.1.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- |
| Frequency Range |  | FR2 | FR2 |
| Duplex mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel |  | 100 MHz: NRB,c = 66 |  |
| Active BWP ID |  | 1, 2 | 3 |
| Downlink initial BWP Configuration |  | DLBWP.0.2 |  |
| Uplink initial BWP Configuration |  | ULBWP.0.2 |  |
| Downlink active BWP-0 Configuration |  | - | DLBWP.0.2 |
| Downlink active BWP-1 Configuration |  | DLBWP.1.1 | - |
| Downlink active BWP-2 Configuration |  | DLBWP.1.3 | - |
| Uplink active BWP-0 Configuration |  | - | ULBWP.0.2 |
| Uplink active BWP-1 Configuration |  | ULBWP.1.1 | - |
| Uplink active BWP-2 Configuration |  | ULBWP.1.3 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |
| TRS configuration |  | TRS.2.1 TDD |  |
| TCI state |  | TCI.State.0 |  |
| RMSI CORESET parameters |  | CR.3.1 TDD |  |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |  |
| OCNG Patterns |  | OP.1 |  |
| SSB Configuration |  | SSB.1 FR2 |  |
| SMTC Configuration |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Propagation Condition |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.5.6.1.1.1-4: OTA related test parameters for BWP switching test case

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in clause A.3.15.1 |  |
| Assumption for UE beams Note 6 |  | Fine | Fine |
| Note1 | dBm/15kHz | -112 | -112 |
| Note1 | dBm/SCS | -103 | -103 |
| SS-RSRPNote2 | dBm/SCS Note3 | -85 | -85 |
|  | dB | 18 | 18 |
| IoNote4 | dBm/95.04 MHz Note4 | -56 | -56 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.6.1.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

### >>>>>End of Change 2<<<<<

### >>>>>Start of Change 3<<<<<

### A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA

A.7.5.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.7.5.6.1.2.1-1 below. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2). The general parameters are given in Table A.7.5.6.1.2.1-2. NR Cell-specific parameters are specified in Table A.7.5.6.1.2.1-3 below. OTA related test parameters are shown in table A.7.5.6.1.2.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

UE is configured with 2 different UE-specific downlink bandwidth parts for PCell, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 2 before starting the test.	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PCell no later than the first UL slot that occurs after the begining of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch on PCell shall occur within the BWP switch delay if the UE doesn’t support per-FR gap, otherwise no interruption due to BWP switch on SCell is allowed.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell(Cell 1).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the half subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch of PCell shall occur within the BWP switch delay if the UE doesn’t support per-FR gap, otherwise no interruption due to BWP switch on SCell is allowed.

The test equipment verifies the DL BWP switch time in PCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell is carried out in the correct time span by monitoring ACK/NACK sent in SCell during BWP switch of PCell, respectively.

Table A.7.5.6.1.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode SCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | PCell: NR 30 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode SCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.6.1.2.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 2 | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.7.5.6.1.2.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR2 |
| Duplex mode | Config 1 |  | FDD | TDD |
|  | Config 2,3 |  | TDD |  |
| TDD configuration | Config 1 |  | Not Applicable | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2 | MHz | 10 MHz: NRB,c = 52 | 100 MHz: NRB,c = 66 |
|  | Config 3 |  | 40 MHz: NRB,c = 106 |  |
| Active BWP ID |  |  | 1, 2 | 0 |
| Downlink initial BWP Configuration |  |  | DLBWP.0.2 |  |
| Uplink initial BWP Configuration |  |  | ULBWP.0.2 |  |
| Downlink active BWP-0 Configuration |  |  | - | DLBWP.0.2 |
| Downlink active BWP-1 Configuration |  |  | DLBWP.1.1 | - |
| Downlink active BWP-2 Configuration |  |  | DLBWP.1.3 | - |
| Uplink active BWP-0 Configuration |  |  | - | ULBWP.0.2 |
| Uplink active BWP-1 Configuration |  |  | ULBWP.1.1 | - |
| Uplink active BWP-2 Configuration |  |  | ULBWP.1.3 | - |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD | SR.3.1 TDD |
| measurement | Config 2 |  | SR.1.1 TDD |  |
| channel | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET | Config 1 |  | CR.1.1 FDD | CR.3.1 TDD |
| parameters | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated | Config 1 |  | CCR.1.1 FDD | CCR.3.1 TDD |
| CORESET | Config 2 |  | CCR.1.1 TDD |  |
| parameters | Config 3 |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR2 |
|  | Config 3 |  | SSB.2 FR1 |  |
| TRS configuration | Config 1,2,3 |  | - | TRS.2.1 TDD |
| TCI state | Config 1,2,3 |  | TCI.State.0 | TCI.State.0 |
| SMTC Configuration |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  | NA Link only, see clause A.3.7A | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Propagation Condition |  |  | NA Link only, see clause A.3.7A | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.7.5.6.1.2.1-4: OTA related test parameters for BWP switching test case

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | NA Link only, see clause A.3.7A | Setup 1 defined in clause A.3.15.1 |
| Assumption for UE beams Note 6 |  |  | Fine |
| Note1 | dBm/15kHz |  | -112 |
| Note1 | dBm/SCS |  | -103 |
| SS-RSRPNote2 | dBm/SCS Note3 |  | -85 |
|  | dB |  | 18 |
| IoNote4 | dBm/95.04 MHz Note4 |  | -56 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.6.1.2.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

If the UE doesn’t support per-FR gap,

During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

Otherwise no interruption due to BWP switch on SCell is allowed.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

### >>>>>End of Change 3<<<<<