3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014478

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	11.1.4

Source: 	Nokia, Nokia Shanghai Bell, ZTE

Title: 	TP to TR 38.921: Clarification of BS array antenna element peak gain for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. One set of parameters requested by ITU-R WP5D is the BS beamforming antenna characteristics in these frequencies. The reply LSs on this set of parameters for the coexistence study were agreed in RAN4#95-e [3] and RAN4#96-e [4], and corresponding TPs were approved to TR 38.921 [5].

A question has been raised on how the BS array antenna element peak gains were determined in the reply LSs, such information has been included in TR 38.820 [6] for the 7 - 24 GHz frequency range study but not in TR 38.921. However, TR 38.820 is an internal technical report and should not be referred to in TR 38.921 which is intended to be formally issued as publications. Therefore, this contribution provides a TP to include such information directly into TR 38.921.

# 2.	Text proposal

<Start of change>

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE		(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	(8.1.2-2)

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10		(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Rural | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- | --- |
| Am (dB) | N/A | 30 | 30 | 30 |
| SLAv (dB) | N/A | 30 | 30 | 30 |
| 3dB (deg.) | N/A | 90 | 90 | 90 |
| 3dB (deg.) | N/A | 65 | 90 | 90 |
| GE,max (dBi) | N/A | 6.4 | 5.5 | 5.5 |
| LE (dB) | N/A | 2.0 | 2.0 | 2.0 |
| (M, N) | N/A | (16, 8) | (16, 8) | (8,8) |
| dh (m) | N/A | 0.5 | 0.5 | 0.5 |
| dv (m) | N/A | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | N/A | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) | N/A | 90 to 100 | 90 to 120 | 90 to 120 |
| Total Radiated Power (dBm) | N/A | 46 | 46 | 37 |
| Mechanical downtilt (deg.) | N/A | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: TRP includes the powers from two polarizations Note 4: The vertical coverage range includes the mechanical downtilt. |  |  |  |  |

<End of change>

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	R4-2008924, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz)”, Ericsson.

[4]	R4-2011932, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, Ericsson.

[5]	R4-2015675, “Draft TR38.821 v0.2.0 Parameters for IMT studies on 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”, Huawei.

[6]	3GPP TR 38.820 v16.0.0, “NR; 7 - 24 GHz frequency range”.