3GPP TSG-RAN WG4 Meeting #97-e	R4-2015632

# Electronic Meeting, 2- 13 Nov, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4915 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of NPUSCH format1 performance requirements for multi-TB interleaved transmission in TS 36.104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Perf |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Performance requirements part for NPUSCH format 1 with multi-TB interleaved transmission agreed in R4-2012600 was not implemented in latest TS 36.104 version 16.7.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the performance requirements part for NPUSCH format 1 with multi-TB interleaved transmission |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirement will be incompleted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

8.5	Performance requirements for Narrowband IoT

8.5.1	Requirements for NPUSCH format 1

8.5.1.1	Requirements

The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A16. The performance requirements assume HARQ retransmissions.

An NB-IoT Base Station supports 15 kHz subcarrier spacing requirements, or 3.75 kHz subcarrier spacing requirements, or both.

For 15kHz subcarrier spacing single-subcarrier/multi-subcarrier, the demodulation requirements apply for the supported number of subcarriers.

Table 8.5.1.1-1: Test parameters

| Parameter | Value |
| --- | --- |
| Maximum number of HARQ transmissions | 4 |
| RV sequence | RV0, RV2 |

8.5.1.1.1	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in table 8.5.1.1.1-1 for the single-subcarrier of 3.75KHz subcarrier spacing,  in table 8.5.1.1.1-2 for 15KHz subcarrier spacing at the given SNR for 1Tx, and in table 8.5.1.1.1-3 for multi-subcarrier of 15KHz subcarrier spacing at the given SNR for 1Tx.

Table 8.5.1.1.1-1: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3.75KHz | 1 | ETU 1Hz Low | A16-1 | 1 | 70% | -1.9 |
|  |  |  |  |  |  | 16 | 70% | -9.2 |
|  |  |  |  |  |  | 64 | 70% | -12.2 |

Table 8.5.1.1.1-2: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, single subcarrier, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15KHz | 1 | ETU 1Hz Low | A16-2 | 1 | 70% | -2.1 |
|  |  |  |  |  |  | 16 | 70% | -8.8 |
|  |  |  |  |  |  | 64 | 70% | -12.6 |

Table 8.5.1.1.1-3: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15KHz | 3 | ETU 1Hz Low | A16-3 | 2 | 70% | -3.0 |
|  |  |  |  |  |  | 16 | 70% | -8.1 |
|  |  |  |  |  |  | 64 | 70% | -11.4 |
|  |  |  | 6 | ETU 1Hz Low | A16-4 | 2 | 70% | -0.6 |
|  |  |  |  |  |  | 16 | 70% | -6.8 |
|  |  |  |  |  |  | 64 | 70% | -10.5 |
|  |  |  | 12 | ETU 1Hz Low | A16-5 | 2 | 70% | -0.7 |
|  |  |  |  |  |  | 16 | 70% | -6.4 |
|  |  |  |  |  |  | 64 | 70% | -10.1 |

Table 8.5.1.1.1-4: Minimum requirements for NPUSCH format 1 with two HARQ processes and multiple TBs with interleaved transmission

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15KHz | 12 | ETU 1Hz Low | A16-6 | 64 | 70% | -13.9 |

<End of change>