3GPP TSG-RAN WG4 Meeting #97-e	R4-2015932

Electronic Meeting, 02 November – 13 November 2020

Source:	Ericsson

Title:	TP for TR 37.717-31-11: Including band combinations DC_1-3-40_n78

Agenda item:			10.5.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-31-11 to add DC_1A-3A-40A_n78A and DC_1A-3A-40C_n78A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_1-3-40_n78

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3-40_n78 | CA_1-3-40 | n78 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A | CA_1A-3A-40A CA_1A-3A-40C | n78A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_1-3-40_n78, the TIB,c and RIB,c values are reused from DC_1-3_n40-n78 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-40_n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | 40 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-40_n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | 40 | 0.4 |
|  | n78 | 0.5 |

### 5.1.x.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

---End of changes---

# Reference

[1]		RP-201572, “Revised WID on Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)” Ericsson