3GPP TSG-RAN WG4 Meeting #97-e 														R4-200XXXX

Electronic Meeting, 2-13 Nov., 2020

Agenda item:			12.2.2.2, 12.2.2.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [97e][134] NR_RF_FR1_enh_Part_2

Document for:	Information

# Introduction

This email thread discusses the RF requirements on Rel-17 Tx switching enhancement for inter-band SUL and uplink CA, including:

- Topic #1: 2Tx-2Tx switching between carrier 1 and carrier 2 (discussed in section 1)

- Topic #2: 1Tx-2Tx and 2Tx-2Tx switching between band A and band B, with 2 contiguous aggregated carriers on band B (discussed in section 2)

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to review the recommended WF in each sub-topic, and provide comments (if any) directly under each issue in section 1.2 and 2.2.

- 2nd round: TBA

# Topic #1: 2Tx switching between carrier 1 and carrier 2

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014717 | Qualcomm Incorporated | Observation: No new time masks are needed compared to the Rel-16 version of the switched tx feature. Proposal: Rel-17 version of switched tx can re-use switching period capabilities from Rel-16. |
| R4-2014739 | CMCC | Observation 1: time mask requirement including switching period is band agnostic. Proposal 1: R16 time mask requirements could still apply for 2Tx (1 carrier)-> 2Tx (1 carrier) switching scenario. Proposal 2: to enhance UL capacity, it is suggested all the following band combinations could support SUL cases 1,2,3 and related switching scenarios in R17. - Band n80 + Band n41 or n79 - Band n81 + Band n41 or n79 - Band n95+ Band n41 or n79 - Band n97+ Band n41 or n79 - Band n98+ Band n41 or n79 Proposal 3: to enhance UL capacity, it is suggested all the following band combinations could support CA cases 1,2,3 and related switching scenarios in R17. - Band n3 + Band n41 or n79 - Band n8 + Band n41 or n79 - Band n39+ Band n41 or n79 - Band n40+ Band n41 or n79 - Band n41+ Band n79 |
| R4-2015182 | ZTE Wistron Telecom AB | Observation 1: For UL Tx switching in Rel-16, one of the two Tx chains can remain unchanged during UL Tx switching operation. Proposal 1: RAN4 specify one set of requirements applicable for both modes of 2Tx switching. Both modes are option 1 and option 2. Proposal 2: Switching period is configurable at either carrier #1 or carrier #2. Proposal 3: When specifying the length of switching period for 2Tx switching, TAE between two Tx chains should be taken into account. |
| R4-2015197 | China Telecom | Proposal 1: Allow different capabilities for length of switching period for 2Tx switching between carrier 1 and carrier 2. Proposal 2: Semi-statically configure the switching period on one of the two uplink carriers for 2Tx switching between carrier 1 and carrier 2. Proposal 3: Define different capabilities for UEs with and without DL interruption for 2Tx switching between carrier 1 and carrier 2. |
| R4-2015262 | Xiaomi | Observation 1: there may be multiple kinds of switching periods between different cases for UL TX switching period in release 17. Proposal 1: For the UL Tx switching enhancement in R17, it should be decided whether it is enough to report only one switching period or not for all cases as shown in table 2. Table 2: Tx switching cases in Rel 17 based on uplink CA band combination Number of Tx chains in WID (carrier 1 + carrier 2). Case 1; 1T+1T. Case 2; 0T+2T. Case 3; 2T+0T. |
| R4-2015283 | Huawei, HiSilicon | Proposal 1: Specify 35us, 140us and 210us as the options from which a UE choose to report its capability of switching period for Tx switching between 2Tx carriers in Rel-17, which is to reuse the values defined in Rel-16 for 1Tx – 2Tx switching. Proposal 2: Reuse Rel-16 mechanism on location of the switching periods in Rel-17 for UE Tx switching between 2Tx carriers. |
| R4-2015325 | vivo | Observation: The basic requirements impact and structure for Rel-17 switching is similar to Rel-16 Tx switching. Proposal 1: The Rel-16 number of the location of the UL switching period requirement can be kept or at least used as baseline for Rel-17. Proposal 2: The Rel-16 requirements for DL reception interruption, including the interruption length, signalling scheme, applicability etc. could be reused or at least used as baseline for Rel-17. Proposal 3: Further consider power class stability and alignment in Rel-17 based on Rel-16 solution. At least consider further refinement of power boost in inter-band case and the impact of introduction of HPUE in Rel-17 UL CA. Proposal 4: Further consider the UL-MIMO and transmission rank related clarifications in Rel-17 based on Rel-16. |
| R4-2015355 | OPPO | Observation 1: In Rel-16, the “Tx” in cases means Tx chain ability which can be further configured by NW for Tx transmission, but this interpretation seems have no impact to RAN4 requirement definition. Observation 2: In Rel-17, two general scenarios are included, i.e. switch between 2Tx at low band and 2Tx at high band, switch between 1CC at low band and 2 contiguous CC at high band Observation 3: Switching between case1/2, between case2/3, among case1/2/3 are also listed in WID, however, there is no much difference among these scenarios in terms of switching time requirements and can be considered together. Observation 4: The UE architecture for Tx switching from Rel-16 to Rel-17 is minor, and the switching time for Rel-16 can be reused. Proposal 1: It is proposed to reuse the Rel-16 time mask for Tx switching scenarios in Rel-17. |

## Open issues summary

Open issues for 2Tx-2Tx switching between carrier 1 and carrier 2 are summarized below:

### Sub-topic 1-1: Switching time mask related requirements

Issue 1-1-1: Length of switching period

- Proposals

  - Option 1: Reuse Rel-16 values for UL CA and SUL, i.e., report {35us, 140 us, or 210us} per pair of UL bands per band combination, and apply the same set of values for switching between different cases in the WID (QC, CMCC, Huawei, HiSilicon, vivo, OPPO)

  - Option 2: TAE between two Tx chains should be taken into account for 2Tx switching (ZTE)

  - Option 3: Decide whether it is enough to report only one switching period or not for all cases. (Xiaomi)

- Recommended WF

  - Given no substantial technical issues identified, can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | Based on our observation, discrepancy between two Tx chains plays no impact in Rel-16 Tx switching since one of the Tx chains can keep unchanged during the switching, but in Rel-17 Tx switching, this is not the case. Our preference is Option 2. However, if the majority view goes for Option 1, we can accept that, but it should be clarified whether or not TAE is included in the switching period. |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| CATT | Support option 1. |
| Qualcomm | Support option 1. |
| Xiaomi | According to the objectives for the Tx switching enhancement, there are three cases allowed in release 17, thereby three potential kind of switching periods between different cases may be existed. However, in order to simplify the signaling, we also prefer to have one value for all kind of switching periods. Thus, option 1 is also OK for us. |
| OPPO | Option 1 |

Issue 1-1-2: Location of switching period

- Proposals

  - Option 1: Reuse Rel-16 agreement for UL CA and SUL, i.e., semi-statically configure the switching period on one of the two uplink carriers (CMCC, ZTE, CTC, Huawei, HiSilicon, OPPO, vivo)

- Recommended WF

  - Can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | We agree on Moderator’s recommendation |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| CATT | We support the moderator’s recommendation. |
| Qualcomm | Support option 1 |
| Xiaomi | OK with Moderator’s recommendation |
| OPPO | Option 1 |

Issue 1-1-3: Transient period

- Proposals

  - Option 1: Reuse Rel-16 agreement for UL CA and SUL, i.e., 2x10 us transient period in addition to the switching period (CMCC, OPPO)

- Recommended WF

  - Can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | Option 1 is fine. |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| CATT | Ok with the moderator’s recommendation. |
| Qualcomm | Support option 1 |
| Xiaomi | OK with Moderator’s recommendation |
| OPPO | Option 1 |

Issue 1-1-4: UL outage due to switching

- Proposals

  - Option 1: Reuse Rel-16 agreement, i.e., UL outage due to switching is applicable to both carrier 1 and carrier 2

    - Note: the Rel-16 agreement can be found in slide #3 of R4-1913041.

- Recommended WF

  - Can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | Option 1 is fine. |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| CATT | Fine with moderator’s recommendation. |

### Sub-topic 1-2: Applicability of DL interruption

Issue 1-2: Applicability of DL interruption

- Proposals

  - Option 1 (CTC, vivo): Reuse Rel-16 agreement, i.e.,

    - For SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

    - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

      - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

- Recommended WF

  - Can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | Fine with Option 1. |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| Qualcomm | Fine with recommended WF |
| Xiaomi | OK with Moderator’s recommendation |
| OPPO | Option 1 |

### Sub-topic 1-3: Other aspcets

Issue 1-3-1: Power boosting for PC3 UL CA

- Background: Rel-16 agreement on power boosting for PC3 UL CA (RP-201365)

  - ….

  - In the CR, the power boosting for carrier 2 with 2Tx is only applied to PC3 CA, i.e., not applied to PC2 CA (the same per BC power class, i.e. PC3 in this case is applied regardless of transmission in Case 1 or Case 2).

- Proposals

  - Proposal 1: Further consider power class stability and alignment in Rel-17 based on Rel-16 solution. At least consider further refinement of power boost in inter-band case and the impact of introduction of HPUE in Rel-17 UL CA (vivo)

- Recommended WF

  - Encourage more companies’ feedback on:

    - With PC2 introduced for inter-band UL CA in Rel-17, is power boosting still needed for PC3 UL CA with Rel-17 2Tx-2Tx switching?

| Company | Comments |
| --- | --- |
| ZTE | Still needed as a generic feature. For a band combination, if only PC3 is defined for the band combination, but one of the component CC supports PC2, then the situation is similar. |
| CMCC | In our view, PC2 inter-band UL CA is a better solution than power boost. And the SAR solution for SUL and PC2 UL CA are captured in Rel-17 WI. But the power boost approach is not captured based on our understanding. Do companies think PC3 UL CA with PC2 supported in one CC is typical? |
| China Telecom | We would like to share our understanding on power boosting: 1) For EN-DC Tx switching, power boosting is not supported in Rel-16, since EN-DC PC2 is already specified in Rel-16. 2) For TDD+TDD CA Tx switching, power boosting cannot enable 26+26dBm PC2, since boosting is only supported for carrier 2 in case 2. 3) For FDD+TDD CA Tx switching, if one UE support power boosting (i.e., be enable to transmit 26dBm on carrier 2 with 2Tx), the UE shall also be able to support at least 23+23dBm PC2. Moreover, with PC2 solution, UE can transmit 26dBm in both case 1 and case 2 but not only in case 2. Therefore, with the same hardware architecture, the UE can directly report PC2 instead of PC3 + power boosting, and PC2 is our favorite solution. |
| CATT | PC2 solution will be the general solution in Rel-17. Whether power boosting can be removed from Rel-17 spec needs more consideration. |
| OPPO | Still needed for 2T PC3 UE but maybe not many UE would support 23+23 but report PC3. |

Issue 1-3-2: 2-layer MIMO support

- Background: Rel-16 description on 2-layer MIMO support for carrier 2 in TS 38.101-1:

  - ….both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2…

- Proposals

  - Proposal 1: Further consider the UL-MIMO and transmission rank related clarifications in Rel-17 based on Rel-16. (vivo)

- Recommended WF

  - Encourage more companies’ feedback on:

    - For carrier 1 in 2Tx-2Tx switching, is it mandatory or optional to support 2-layer PUSCH transmission?

    - For carrier 2 in 2Tx-2Tx switching, can we reuse the above agreement in Rel-16, i.e., mandatory to support 2-layer PUSCH transmission?

| Company | Comments |
| --- | --- |
| ZTE | In 2Tx-2Tx switching, we tend to agree on aligning both carriers, i.e., mandatory to support 2-layer PUSCH Tx for both Carrier #1 and Carrier #2. |
| CMCC | Both carrier 1 and carrier 2 should mandatory support 2-layer PUSCH transmission in 2Tx-2Tx switching. |
| China Telecom | Mandating 2-layer PUSCH transmission in both carriers is preferred. Meanwhile, for carrier 1, we are not sure if separate discussion for TDD band and FDD/SUL band are needed. |
| CATT | It would be better to mandate 2-layer PUSCH transmission in 2 Tx-2 Rx switching on both carriers. |
| Qualcomm | Prefer mandating 2-layer support for both carriers for 2Tx-2Tx case. This would follow R16 agreement |
| OPPO | Ok with supporting 2-layer for carrier 1 and carrier 2. |

## Companies views’ collection for 1st round

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Tx switching between band A and band B

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014717 | Qualcomm Incorporated | (The same proposals for band B with single carrier or two contiguous aggregated carriers) Observation: No new time masks are needed compared to the Rel-16 version of the switched tx feature. Proposal: Rel-17 version of switched tx can re-use switching period capabilities from Rel-16. |
| R4-2014739 | CMCC | (The same proposals for band B with single carrier or two contiguous aggregated carriers) Observation 1: time mask requirement including switching period is band agnostic. Proposal 1: R16 time mask requirements could still apply for 2Tx (1 carrier)-> 2Tx (1 carrier) switching scenario. Proposal 2: to enhance UL capacity, it is suggested all the following band combinations could support SUL cases 1,2,3 and related switching scenarios in R17. - Band n80 + Band n41 or n79 - Band n81 + Band n41 or n79 - Band n95+ Band n41 or n79 - Band n97+ Band n41 or n79 - Band n98+ Band n41 or n79 Proposal 3: to enhance UL capacity, it is suggested all the following band combinations could support CA cases 1,2,3 and related switching scenarios in R17. - Band n3 + Band n41 or n79 - Band n8 + Band n41 or n79 - Band n39+ Band n41 or n79 - Band n40+ Band n41 or n79 - Band n41+ Band n79 |
| R4-2015198 | China Telecom | (The same proposals for band B with single carrier or two contiguous aggregated carriers) Proposal 1: Allow different capabilities for length of switching period for Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B. Proposal 2: Semi-statically configure the switching period on one of the two uplink carriers for Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B. Proposal 3: Define different capabilities for UEs with and without DL interruption for Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B. |
| R4-2015325 | vivo | (The same proposals for band B with single carrier or two contiguous aggregated carriers) Observation: The basic requirements impact and structure for Rel-17 switching is similar to Rel-16 Tx switching. Proposal 1: The Rel-16 number of the location of the UL switching period requirement can be kept or at least used as baseline for Rel-17. Proposal 2: The Rel-16 requirements for DL reception interruption, including the interruption length, signalling scheme, applicability etc. could be reused or at least used as baseline for Rel-17. Proposal 3: Further consider power class stability and alignment in Rel-17 based on Rel-16 solution. At least consider further refinement of power boost in inter-band case and the impact of introduction of HPUE in Rel-17 UL CA. Proposal 4: Further consider the UL-MIMO and transmission rank related clarifications in Rel-17 based on Rel-16. |
| R4-2015355 | OPPO | (The same proposals for band B with single carrier or two contiguous aggregated carriers) Observation 1: In Rel-16, the “Tx” in cases means Tx chain ability which can be further configured by NW for Tx transmission, but this interpretation seems have no impact to RAN4 requirement definition. Observation 2: In Rel-17, two general scenarios are included, i.e. switch between 2Tx at low band and 2Tx at high band, switch between 1CC at low band and 2 contiguous CC at high band Observation 3: Switching between case1/2, between case2/3, among case1/2/3 are also listed in WID, however, there is no much difference among these scenarios in terms of switching time requirements and can be considered together. Observation 4: The UE architecture for Tx switching from Rel-16 to Rel-17 is minor, and the switching time for Rel-16 can be reused. Proposal 1: It is proposed to reuse the Rel-16 time mask for Tx switching scenarios in Rel-17. |

## Open issues summary

Open issues for 1Tx-2Tx and 2Tx-2Tx switching between band A and band B (with 2 contiguous aggregated carriers on band B) are summarized below:

### Sub-topic 2-1: Switching time mask related requirements

Issue 2-1: Switching time mask related requirements

- Proposals

  - Option 1: For switching time mask related requirements for inter-band SUL and CA (including the length of switching period, location of switching period, transient period and uplink outage due to switching), the same agreements are applied for the scenarios with either one carrier or two contiguous aggregated carriers on band B (QC, CMCC, CTC, vivo, OPPO)

- Recommended WF

  - Can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | As commented aforementioned, it should be clarified whether or not TAE is included in the switching period under this option. |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| CATT | Support option 1 proposed by moderator. |
| Qualcomm | Support option 1 |
| Xiaomi | OK with option 1 |
| OPPO | Option 1 ok. |

### Sub-topic 2-2: Applicability of DL interruption

Issue 2-2: Applicability of DL interruption

- Proposals

  - Option 1 (CTC, vivo): The same agreements are applied for the scenarios with either one carrier or two contiguous aggregated carriers on band B, i.e.,

    - For SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

    - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

      - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

- Recommended WF

  - Can we agree option 1?

| Company | Comments |
| --- | --- |
| ZTE | Fine with Option 1. |
| CMCC | Support option 1 |
| China Telecom | Support option 1 |
| CATT | Fine with the moderator recommendation. |
| Qualcomm | Support option 1 |
| Xiaomi | OK with option 1. |
| OPPO | Option 1 ok. |

### Sub-topic 2-3: Other aspcets

Issue 2-3-1: Power boosting for PC3 UL CA

- Background: Rel-16 agreement on power boosting for PC3 UL CA (RP-201365)

  - ….

  - In the CR, the power boosting for carrier 2 with 2Tx is only applied to PC3 CA, i.e., not applied to PC2 CA (the same per BC power class, i.e. PC3 in this case is applied regardless of transmission in Case 1 or Case 2).

- Proposals

  - Proposal 1: Further consider power class stability and alignment in Rel-17 based on Rel-16 solution. At least consider further refinement of power boost in inter-band case and the impact of introduction of HPUE in Rel-17 UL CA (vivo)

- Recommended WF

  - Encourage more companies’ feedback on:

    - With PC2 introduced for inter-band UL CA in Rel-17, is power boosting still needed for PC3 UL CA with Rel-17 1Tx-2Tx and 2Tx-2Tx switching between two uplink bands?

| Company | Comments |
| --- | --- |
| ZTE | Same comments aforementioned |
| CMCC | In our view, PC2 inter-band UL CA is a better solution than power boost. And the SAR solution for SUL and PC2 UL CA are captured in Rel-17 WI. But the power boost approach is not captured based on our understanding. Do companies think PC3 UL CA with PC2 supported in one CC is typical? |
| China Telecom | Same comments aforementioned |
| CATT | Same comments aforementioned. |
| OPPO | Still needed for 2T PC3 UE but maybe not many UE would support 23+23 but report PC3. |

Issue 2-3-2: 2-layer MIMO support

- Background: Rel-16 description on the UL-MIMO support for carrier 2 in TS 38.101-1:

  - ….both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2…

- Proposals

  - Proposal 1: Further consider the UL-MIMO and transmission rank related clarifications in Rel-17 based on Rel-16. (vivo)

- Recommended WF

  - Encourage more companies’ feedback on:

    - In 2Tx-2Tx switching, for carrier 1 on band A, is it mandatory or optional to support 2-layer PUSCH transmission?

    - In both 1Tx-2Tx and 2Tx-2Tx switching, for the two contiguous carriers on band B, is it mandatory or optional to support 2-layer PUSCH transmission?

| Company | Comments |
| --- | --- |
| ZTE | Same comments aforementioned |
| CMCC | Both carrier 1 and carrier 2 should mandatory support 2-layer PUSCH transmission in 2Tx-2Tx switching. Carrier 2 should mandatory support 2-layer PUSCH transmission in 1Tx-2Tx switching |
| China Telecom | For 1Tx-2Tx switching, mandating 2-layer PUSCH transmission in the two carriers on band B is preferred. For 2Tx-2Tx switching, mandating 2-layer PUSCH transmission in all of the three carriers on band A+B is preferred. Meanwhile, for carrier 1, we are not sure if separate discussion for TDD band and FDD/SUL band are needed. |
| CATT | It would be better to mandate 2-layer PUSCH transmission in 2 Tx-2 Rx switching on both carriers. |
| Qualcomm | We prefer to mandate 2-layer PUSCH transmission in 2Tx-2Tx switching on both 2Tx carriers. |
| OPPO | Ok with supporting 2-layer for carrier 1 and carrier 2. |

## Companies views’ collection for 1st round

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |