3GPP TSG-RAN WG4 Meeting # 98-e 	                         R4-2100807

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	9.33.2

Source: 	CMCC

Title: 	TP on DL interruption applicability of CA_n3-n41-n79 for 37.xxx

Document for:	Approval

# Introduction

TP on DL interruption applicability for CA_n3-n41-n79 for 37.xxx is provided in this paper.

# Text Proposal

< Start of Text Proposal>

- 6.1	Inter-band uplink CA

- 6.1.x	CA_n3-n41-n79

- 6.1.x.1	Configurations

<Editor’s note: the CA configurations and the UL configuration are clarified here. >

| Configuration | Uplink configuration |
| --- | --- |
| CA_n3-n41-n79 | CA n3-n41, CA n3-n79, CA n41-n49 |

- 6.1.x.2	Technical analysis

<Editor’s note: This agenda captures the technical analysis on DL interruption of this band combination>

RAN4 already agreed that no DL interruption allowed for CA_n3-n41, CA_n3-n79 and CA n41-n79 [1]. And for TDD band combination CA_n41-n79, simultaneous Rx/Tx capability is mandatory supported.

So for the uplink configurations CA n3-n41, CA n3-n79, CA n41-n49 of CA_n3-n41-n79, no DL interruption for dynamic switching between two uplink carriers should also be mandated.

- 6.1.x.3	Conclusion

<Editor’s note: This agenda captures the conclusion that if DL interruption is allowed. It’ll be good that the conclusion format aligns with the final CR. >

- Table 6.1.x.3-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 3) |
| --- | --- | --- |
| CA_n3-n41-n79 | n3, n41, n79 | No |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. NOTE 3: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

# Reference

- TS38.101-1, NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone