3GPP TSG-RAN WG4 Meeting # 98-e 	                         R4-2100808

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	9.33.2

Source: 	CMCC

Title: 	TP on DL interruption applicability of CA_n8-n39-n41 for 37.xxx

Document for:	Approval

# Introduction

TP on DL interruption applicability for CA_n8-n39-n41 for 37.xxx is provided in this paper.

# Text Proposal

< Start of Text Proposal>

- 6.1	Inter-band uplink CA

- 6.1.x	CA_n8-n39-n41

- 6.1.x.1	Configurations

<Editor’s note: the CA configurations and the UL configuration are clarified here. >

| Configuration | Uplink configuration |
| --- | --- |
| CA_n8-n39-n41 | CA n8-n39, CA n8-n41, CA n39-n41 |

- 6.1.x.2	Technical analysis

<Editor’s note: This agenda captures the technical analysis on DL interruption of this band combination>

RAN4 already agreed that no DL interruption allowed for CA_n8-n41 and CA_n39-n41 [1].

For CA_n8-n39, there was no discussion on the applicability of DL interruption. Since the frequency gap between n8 and n39 is 1GHz, the separation is large enough. So no DL interruption for uplink configuration CA_n8-n39 should be allowed.

So for the uplink configurations CA n8-n39, CA n8-n41 and CA n39-n41 of CA_n8-n39-n41, no DL interruption for dynamic switching between two uplink carriers should also be mandated.

- 6.1.x.3	Conclusion

<Editor’s note: This agenda captures the conclusion that if DL interruption is allowed. It’ll be good that the conclusion format aligns with the final CR. >

- Table 6.1.x.3-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 3) |
| --- | --- | --- |
| CA_n8-n39-n41 | n8, n39, n41 | No |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. NOTE 3: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

# Reference

- TS38.101-1, NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone