3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2105023

Electronic Meeting, Apr. 12-20, 2021

Source:	Samsung

Title:	Discussion on FR2 HST Deployment Scenario-A

Agenda item:			8.7.2.1

Document for:	Discussion

# 1. Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [1, RP-202118] (which has been further revised to [2, RP-210800]). The deployment scenarios for HST in FR2 have been further discussed in last RAN4 meeting (RAN4#98-e), with the following two scenarios agreed [5, R4-2103240]:

| Scenario | Ds (meter) | Dmin (meter) | Prioritization | Note |
| --- | --- | --- | --- | --- |
| A | 700 | 10 | Prioritised | Changed from Original Scenario-2 |
| B | 700 | 150 | Prioritised | New Scenario (same as FR1 deployment) |

In this contribution, we would like to further provide our viewpoints on deployment Scenario-A, along with the accompanying discussion paper on Scenario-B [6].

# 2. Uni-directional RRH Deployment for Scenario-A

## 2.1 Assumption for Analysis

By following the assumption agreed in WF [5, R4-2103240], the assumption for following analysis is provided in the following table, with the highlighted ones be the selected option or the alternative not captured in WF.

Table 2.1-1 Assumption for Uni-directional RRH Deployment

| Parameter | Value |
| --- | --- |
| Carrier frequency | 28 GHz |
| Ds and Dmin | Scenario-A: Ds = 700m and Dmin = 10m |
| RRH parameters |  |
| RRH height | 15 m |
| RRH Tx Power | 31 dBm |
| RRH antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] Newly updated beamforming parameters in WF. |
| RRH panel orientation | Option-1: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway) Accordingly, for Scenario-A: - Azimuth angle: 0.8 degree - Down-titling: 1.2 degree |
| Number of RRH sites per BBU | 4 |
| Number of RRH panels per RRH sites | 1 (i.e., uni-directional) |
| Number of Analog Beams per RRH | 1 or 2 Note1 |
| Propagation model | RMa LOS |
| UE parameters |  |
| UE antenna height | 5m |
| UE(PC4) antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] 5dBi per element antenna gain |
| UE panel orientation | Direction is opposite to RRH boresight direction, i.e., Scenario-A: Azimuth angle: 0.8 degree Down-titling: 1.2 degree |

For the above RRH related parameters, as far as we know, the above highlighted parameters are better to reflect current FR2 gNB implementation. For UE side parameter selection, the antenna array model is selected based on some common understanding for PC4 type UE’s antenna array model, and one analog beam per RRH is assumed to see what is the lower bound of the link-budget performance with the most limited RX beam number configuration.

## 2.2 Analyitical Evalution

#### 2.2.1 Single-Beam-per-Panel

By adopting one beam per RRH panel, i.e., the beam direction is the same as RRH boresight direction, the received power’s trends along the HST track are provided in Figure 2.2.1-1/2/3. It should be noted that only one RX beam is considered in the below analysis.

| Rx Power without UE beamformig | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.1-1 RX power w./without UE RX beamforming, Single-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.1-2 RX power w./without UE RX beamforming, Single-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.1-3 RX power w./without UE RX beamforming, Single-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

Based on the above results for single beam per RRH, even the RRH panel model [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] is chosen, it has been demonstrated that narrower analog beam (compared with other [M,N] options) can still provide a satisfactory coverage along the HST track. Specifically, with the results without UE beamforming gain, the received signal can provide around 30 dB margin above the currently specified PC4 REFSENS requirements, i.e., -94dBm with 100MHz bandwidth.

Observation 1: For uni-directional RRH deployment in Scenario-A, even with single narrow analog beam per RRH panel (i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]), there is still 30dB margin compared against PC4 REFSENS requirement.

#### 2.2.2 Dual-Beam-per-Panel

Then we want to evaluate how much enhancement we can obtained if one additional beam is implemented on each RRH panel, with other parameters not changed. Specifically, by adjusting the direction of the newly added beam, the cellular coverage in some range of HST track can be enhanced.

For instance, if the newly add beam (in below figure denoted by beam-1) is directed to the point near to its RRH, e.g., with θbeam1=9 degree and φbeam1=6 degree relative to boresight direction for [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]. The beam-2 in the below figure denotes the same beam used in single-beam-per-Panel deployment, with θbeam2=0 degree and φbeam2=0 degree, i.e., the same as boresight direction.

| Rx Power without UE beamformig | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.2-1 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.2-2 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming |
| --- | --- |

Figure 2.2.2-3 RX power w./without UE RX beamforming, Dual-Beam-per-Panel, [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

Interestingly shown here, even with one fixed RX beam used in UE side (opposite to RRH panel boresight direction, i.e., beam-2), the newly introduced beam-1 can still achieve a much better coverage around its intended coverage region, i.e., near to its RRH. In other words, even with the simplest UE RX beamforming scheme, adding additional beam per RRH panel can still give benefits to its intended region, but with the cost of more frequent beam switching at gNB side.

Observation 2: For uni-directional RRH deployment in Scenario-A, with one additional beam used per RRH panel, the cellular coverage for the beam’s intended region can be improved, even with one fixed RX beam used in UE side.

## 2.3 Beam Switching Point and Beam Dwelling Time

To facilitate the discussion for channel modeling, the coverage region by each RRH is crucial to be investigated. “Switch Point” determination for single-beam-per-panel deployments is described in section 2.3.1, and for dual-beam-per-panel deployments, the similar method is followed, “Ds_offset” value for dual-beam-per-panel deployments is omitted here.

#### 2.3.1 Switch Point Determination for HST-DPS Channel Modeling

Based on the above analysis and comparison for single- and dual-beam-per-panel for RRH side, the switching point is determined by many factors, including:

(1) RRH panel’s boresight direction;

(2) RRH panel’s beam number and beam direction;

(3) RRH panel configuration [Mg, Ng, M, N, P] for wide or narrow beams.

To further explain the “switching point” concept for uni-directional RRH deployment, it is illustrated in Figure 2.3.1-1. For UE moving towards serving beam, ”Ds_offset” is defined as the distance between the switching point to RRH-n and the location of RRH-n. The numerical results of the location of ”switching point” is provided in Table 2.3.1-1.

Figure 2.3.1-1. Illustration of DPS switching point for HST-DPS Alt-1: UE Moving towards Serving Beam

Table 2.3.1-1 Beam Switching Point for Single-Beam-per-Panel, Alt-1: UE moving towards serving beam

|  | RRH antenna array model |  |  |
| --- | --- | --- | --- |
|  | [Mg, Ng, M, N, P] =[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 16, 2] |
| Ds_offset (meter) of Switching Point (Refer to Figure 2.3-1) | 40 + Ds | 47 + Ds | 81 + Ds |

Similarly, for Alt-2: UE Moving Away from Serving Beam, ”Ds_offset” is defined as the distance between the location of RRH-n to the switching point to RRH-n, as illustrated in Figure2.3.1-2.

Figure 2.3.1-2. Illustration of DPS switching point for Alt-2: UE Moving Away from Serving Beam

Here we provide the numerical results accordingly.

Table 2.3.1-2 Beam Switch Point for Single-Beam-per-Panel, Alt-2: UE Moving Away from Serving Beam

|  | RRH Antenna Array Model |  |  |
| --- | --- | --- | --- |
|  | [Mg, Ng, M, N, P] =[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 16, 2] |
| Ds_offset (meter) (refer to Figure 2.3-2) | 40 | 47 | 81 |

#### 2.3.2 Beam Dwelling Time

Accordingly, the beam dwelling time based on the above beam coverage analysis is provided.

Table 2.3.2-1 Beam Dwelling Time for Single-Beam-per-RRH, Uni-Directional RRH Deployment

| UE maximum speed | Beam Dwelling Time (s) |
| --- | --- |
| 260km/h | 9.7 |
| 350km/h | 7.2 |

Table 2.3.2-2 Beam Dwelling Time for Dual-Beam-per-Panel, Uni-Directional RRH Deployment

|  |  | RRH Antenna Array Model |  |  |
| --- | --- | --- | --- | --- |
|  |  | [Mg, Ng, M, N, P] =[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P] =[1, 1, 8, 16, 2] |
| RRH beam1 dwelling distance(m) |  | 55 | 44 | 93 |
| RRH beam1 dwelling time(s) | speed = 260km/h | 0.76 | 0.61 | 1.29 |
|  | speed = 350km/h | 0.57 | 0.45 | 0.96 |
| RRH beam2 dwelling distance(m) |  | 646 | 656 | 607 |
| RRH beam2 dwelling time(s) | speed = 260km/h | 8.94 | 9.08 | 8.40 |
|  | speed = 350km/h | 6.64 | 6.75 | 6.24 |

Observation 3: For uni-directional RRH deployment for Scenario-A, the smallest beam dwelling time can be in the range of [0.45, 0.96] seconds for UE maximum speed of 350kmph.

# 3. Bi-directional RRH Deployment for Scenario-A

## 3.1 Two Schemes for Bi-directional Deployment

As analyzed in our discussion in last RAN4 meeting, for bi-directional RRH deployment, the cellular coverage could be a problem for the region near RRH, i.e., the nearest RRH can’t direct its beam into that region and thus making a coverage hole under each RRH site.

To solve this issue of coverage hole under RRH site, we consider two possible schemes.

Observation 4: To solve the issue of coverage hole for bi-directional deployment, there are two possible schemes:

- Scheme-1: Connecting to 2nd-Nearest RRH;

- Scheme-2: Connecting to Nearest RRH except Coverage Hole.

#### 3.1.1 Scheme-1: Connecting to 2nd-Nearest RRH

As illustrated as below, if UE always connecting to 2nd-nearest RRH, the coverage hole can be solved, while it also has the obvious shortcoming, i.e., it will always lose the chance to enjoy the nearest RRH for better coverage.

If the Scheme-1 is used, each RRH is intended to cover the half of Ds far from its own site, it is straightforward to adopt Option-1 for RRH panel orientation, i.e., RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway).

Figure 3.1.1-1 Scheme-1: Connecting to 2nd-Nearest RRH

#### 3.1.2 Scheme-2: Connecting to Nearest RRH except Coverage Hole

Another bi-directional deployment scheme is to connect to the nearest RRH except the region around each RRH site, where the coverage hole is coverage by the neighboring RRH, as illustrated in following figure.

It can be shown that by adopting Scheme-2, there are three beams used to cover each Ds region, while UE need to switch two times during the same Ds region.

Figure 3.1.2-1 Scheme-2: Connecting to Nearest RRH except Coverage Hole

## 3.2 Assumption for Analysis

By following the assumption agreed in WF [5, R4-2103240], the assumption for following analysis is provided in the following table, with the highlighted ones be the selected option or the alternative not captured in WF.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 28 GHz |
| Ds and Dmin | Scenario-A: Ds = 700m and Dmin = 10m |
| RRH parameters |  |
| RRH height | 15 m |
| RRH Tx Power | 31 dBm |
| RRH antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] Newly updated beamforming parameters in WF. |
| RRH panel orientation | Option-1: RRH panel boresight pointed to the railway in the middle point between 2 RRHs Scenario-A:Azimuth angle: 1.6 degree Down-titling: 2.5 degree (used for Scheme-2, described below) Option-2: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway) Scenario-A: Azimuth angle: 0.8 degree Down-titling: 1.2 degree (used for Scheme-1, described below) |
| Number of RRH sites per BBU | 4 |
| Number of RRH panels per RRH sites | 2 (i.e., Bi-directional) |
| Number of Analog Beams per RRH | 1 |
| Propagation model | RMa LOS |
| UE parameters |  |
| UE antenna height | 5m |
| UE(PC4) antenna array model | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] 5dBi per element antenna gain |
| UE panel orientation | Direction is opposite to RRH boresight direction, i.e., Scenario-A:Azimuth angle: 1.6 degree Down-titling: 2.5 degree |

## 3.3 Analyitical Evalution

#### 3.3.1 Scheme-1: Connecting to 2nd-Nearest RRH

By applying the scheme-1 illustrated in Section 3.1.1, the received power’s trends along the HST track are provided in Figure 3.3.1-1/2/3.

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.1-1 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.1-2 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-1) |
| --- | --- |

Figure 3.3.1-3 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-1, 
[Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

Because the coverage is provided by the “far-tail” of the boresight beam in Scheme-1, the RX power margin is similar as uni-directional deployment (Section 2.2.1), while the bi-directional scheme-1 lose the chance to enjoy the nearest RRH for better coverage.

Observation 5: If Scheme-1 for bi-directional deployment is adopted for Scenario-A, there is no benefit compared with uni-directional counterpart.

#### 3.3.2 Scheme-2: Connecting to Nearest RRH except Coverage Hole

As described in Section 3.1.2 for Scheme-2, i.e., UE connects to the nearest RRH except coverage hole region, where the coverage hole is coverage by the neighboring RRH. Accordingly, if one beam per RRH panel is assumed, the RX signal power is provided as below.

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-2) |  |
| --- | --- | --- |

Figure 3.3.2-1 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-2, 
[Mg, Ng, M, N, P]=[1, 1, 4, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-2) |  |
| --- | --- | --- |

Figure 3.3.2-2 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-2, 
[Mg, Ng, M, N, P]=[1, 1, 8, 8, 2]

| Rx Power without UE beamforming | Rx Power with UE beamforming (Scheme-2) |  |
| --- | --- | --- |

Figure 3.3.2-3 RX power w./without UE RX beamforming, Single-Beam-per-Panel, Scheme-2, 
[Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]

Based on the numerical results above, Scheme-2 for bi-directional deployment can be used for solve the coverage hole issue at the expense of one additional TX beam switching within each Ds.

Observation 6: Scheme-2 for bi-directional deployment can be used for solve the coverage-hole issue, at the expense of 3 TX beam switching within each Ds.

## 3.4 Beam Dwelling Time

Similar to uni-directional RRH deployment, the coverage region and accordingly beam dwelling time by each RRH is provided as below.

Table 3.4-1 Beam Dwelling Time for Single-Beam-per-Panel for bi-directional deployment

|  |  |  | RRH Antenna Array Model |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | [Mg, Ng, M, N, P]=[1, 1, 4, 8, 2] | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] | [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2] |
| Single-beam-per-Panel Bi-directional RRH scheme1 | dwelling distance(m) |  | 350 | 350 | 350 |
|  | dwelling time(s) | speed = 260km/h | 4.85 | 4.85 | 4.85 |
|  |  | speed = 350km/h | 3.6 | 3.6 | 3.6 |
| Single-beam-per-Panel Bi-directional RRH scheme2 | RRH1P1beam1 near the RRH1 dwelling distance(m) |  | 311 | 301 | 267 |
|  | RRH1P1beam1 near the RRH1 dwelling time(s) | speed = 260km/h | 4.31 | 4.17 | 3.70 |
|  |  | speed = 350km/h | 3.2 | 3.1 | 2.75 |
|  | RRH2P1beam1 near the RRH2 dwelling distance(m) |  | 311 | 303 | 267 |
|  | RRH2P1beam1 near the RRH1 dwelling time(s) | speed = 260km/h | 4.31 | 4.19 | 3.70 |
|  |  | speed = 350km/h | 3.2 | 3.1 | 2.75 |
|  | RRH1P1beam1 near the RRH2 dwelling distance(m) |  | 78 | 96 | 194 |
|  | RRH1P1beam1 near the RRH2 dwelling time(s) | speed = 260km/h | 1.08 | 1.33 | 2.69 |
|  |  | speed = 350km/h | 0.80 | 0.99 | 1.99 |

Observation 7: For bi-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.80, 1.99] seconds for UE maximum speed of 350kmph.

# 3. Conclusion

In this contribution, we further provided our discussion and viewpoint on Scenario-A for high speed train deployment in FR2. The following observations and proposals are provided accordingly:

Observation 1: For uni-directional RRH deployment in Scenario-A, even with single narrow analog beam per RRH panel (i.e., [Mg, Ng, M, N, P]=[1, 1, 8, 16, 2]), there is still 30dB margin compared against PC4 REFSENS requirement.

Observation 2: For uni-directional RRH deployment in Scenario-A, with one additional beam used per RRH panel, the cellular coverage for the beam’s intended region can be improved, even with one fixed RX beam used in UE side.

Observation 3: For uni-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.45, 0.96] seconds for UE maximum speed of 350kmph.

Observation 4: To solve the issue of coverage hole for bi-directional deployment, there are two possible schemes:

- Scheme-1: Connecting to 2nd-Nearest RRH;

- Scheme-2: Connecting to Nearest RRH except Coverage Hole.

Observation 5: If Scheme-1 for bi-directional deployment is adopted for Scenario-A, there is no benefit compared with uni-directional counterpart.

Observation 6: Scheme-2 for bi-directional deployment can be used for solve the coverage-hole issue, at the expense of 3 TX beam switching within each Ds.

Observation 7: For bi-directional RRH deployment for Scenario-A, the beam dwelling time can be in the range of [0.80, 1.99] seconds for UE maximum speed of 350kmph.

# 4. Reference

[1] RP-202118, “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia.

[2] RP-210800, “Revised WID on NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung,

[3] TR 36.878, “Study on performance enhancements for high speed scenario in LTE”, 3GPP.

[4] R4-2100915, “Further discussion on high speed train deployment scenario in FR2”, Samsung.

[5] R4-2103240, “Way forward on Deployment Scenario and UE RF Requirement for FR2 HST”, Samsung.

[6] R4-2105024, “Discussion on FR2 HST Deployment Scenario-B”, Samsung.