3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2105028

Electronic Meeting, Apr. 12-20, 2021

Source:	Samsung

Title:	Feasibility evaluation for maximum supported speed for Rel-17 FR2 HST from demodulation perspective

Agenda item:			8.7.5.1

Document for:	Discussion

Introduction

During the last RAN4 meeting, the general issue for NR FR2 HST were discussion, including the deployment scenarios, channel modelling and the maximum supported speed from both DL and UL perspective. The related agreement was captured in the WF [1].  In order to foliated the feasibility checking of maximum supported speed, the initial simulation assumption was agreed as

- Numerology considered for maximum supported speed: - For FR2 HST evaluations and possible performance requirements definition: - Only consider 120 kHz SCS as baseline assumption. - FFS the maximum supported speed from DL demodulation perspective: - Reference signal(s) to be used for frequency offset tracking: - Option-1: TRS (4 symbol interval) - Option-2: DMRS (1+1+1) - Option-3: TRS + PT-RS (1 and 2 symbol interval) - FFS the maximum supported speed from UL demodulation perspective: - Reference signal(s) to be used for frequency offset tracking: - Option-1: DM-RS (1+1+1) only - Option-2: PT-RS (1 or 2 or 4) + DMRS - Option-3: PT-RS (1 or 2 or 4) - FFS under bi- and uni-directional deployment. - FFS the possible higher supported speed for uni-directional deployment. - FFS Frequency error of +/-0.1ppm should be taken into consideration for both UL and DL. - RAN4 shall check the maximum supportable speed from demodulation perspective and accordingly the possible enhancement - Candidate maximum speed values to be evaluated - 260km/h - 350km - Other options if identified

In this contribution, the initial simulation results are provided to analysis the maximum supported speed from the both DL an UL demodulation perspective.

Simulation Assumption & Common Parameters

Deployment scenarios

The following agreement and conclusion were made on FR2 HST deployment scenario and related aspects, captured in the approved WF [1].

Table 2-1: Deployment scenarios

| Scenario | Ds (meter) | Dmin (meter) | maximum speed(km/h) | Note |
| --- | --- | --- | --- | --- |
| A | 700 | 10 | Option 1: 260km/h Option 2: 350km/h | Other options are not precluded if identified |
| B | 700 | 150 | Option 1: 260km/h Option 2: 350km/h | Other options are not precluded if identified |

Doppler value based on UE velocity

Based on the WF [1] in the last meeting, two kinds of UE velocity are proposed for the feasibility checking for maximum supportable speed. The maximum Doppler value can be calculated without considering 0.1 ppm CFO as following as, with targeting Carrier Frequency as 28GHz and 30 GHz, respectively.

Table 2-2: Maximimum Doppler value based on UE velocity

| Velocity | Carrier Frequency | Maximum Doppler (UL) | Maximum Doppler(DL) |
| --- | --- | --- | --- |
| 260km/h | 28GHz | 13481Hz | 6741 Hz |
| 350km/h | 28GHz | 18148Hz | 9074Hz |
| 260km/h | 30GHz | 14444Hz | 7222Hz |
| 350km/h | 30GHz | 19444Hz | 9722Hz |

Maximum supported Doppler values in DL and UL based on RS configuration

As the objective provided in WF [1], the maximum supported Doppler shift for both UL and DL and maximum supported UE speed will be investigated and determined based on operating frequency, velocity and the Rel-15/16 NR design limitations for all UL/DL physical channels, while the feasibility of maximum supported speeds of up to a maximum of 350km/h will be investigated as required in WID.

DL

For DL, both TRS , DMRS and PRRS are considered for frequencyoffset tracking. TRS was introduced and assumed as a baseline resources for frequency offset tracking, we apply the TRS for baseline assumption in FR2 for ananlysis the maximum Doppler capability as follows

Table 2-3: Maximimum Doppler estimation capability based on 120 kHz TRS  assumption

| RS configuration | Supported Maximum Doppler (DL) | Supported Maximum Doppler (DL) with considering 0.1ppm@28GHz | Supported Maximum Doppler (DL) with considering 0.1ppm@30GHz |
| --- | --- | --- | --- |
| TRS (4 symbols gap) | 14000Hz | 11200Hz | 11000Hz |

Observation 1: It’s feasible to support up to 350km/h with 120kHz TRS configured for frequency offset tracking from DL perspective.

UL

For UL, both DMRS and PTRS are considered for frequency offset tracking. Based on the different configuration for RS (DMRS or PTRS), the maximum Doppler estimation capability can be calculated as follow:

Table 2-4: Maximimum Doppler estimation capability based on different RS configuration  with 120kHz SCS

| RS configuration | Number of DMRS (Position) | PTRS density in time domain | Supported Maximum Doppler (UL) | Supported Maximum Doppler (UL) with considering 0.1ppm@28GHz | Supported Maximum Doppler (UL) with considering 0.1ppm@30GHz |
| --- | --- | --- | --- | --- | --- |
| DMRS+PTRS (Type B with 9 data symbols) | 1 | LPT-RS=1 | 56000Hz | 53200Hz | 53000Hz |
|  |  | LPT-RS=2 | 28000Hz | 25200Hz | 25000Hz |
|  |  | LPT-RS=4 | 14000Hz | 11200Hz | 11000Hz |
|  | 2(0,6) | LPT-RS=1 | 56000Hz | 53200Hz | 53000Hz |
|  |  | LPT-RS=2 | 28000Hz | 25200Hz | 25000Hz |
| DMRS only (Type B with 9 data symbols) | 3(0, 3,6) | N.A | 18667Hz | 15867Hz | 15667Hz |

Based on table, with only DMRS configured, the maximum Doppler value with 3 DMRS symbols can be estimated is 18667Hz. It is not feasible to support 350km/h with targeting 30GHz under this configuration.

Observation 2:  It is not feasible to support maximum 350km/h velocity with targeting 30GHz carrier frequency under only DMRS configuration for UL .

Observation 3: It’s feasible to support up to 350km/h with combined DMRS + PTRS (120kHz) used for frequency offset tacking for UL.

Simulation Results

In this section, the initial simulation results are provided for both PUSCH and PDSCH based on different scenarios, RS configuraion and UE velocity.

With different train moving direction related to the served RRH beam direction, it will only impact on the sign of Doppler shift. From the demodulation perspective, there is no different receiver process foreseen. For simplicity, we assume the train moving direction is moving towards served RRH beam direction for simulation.

The conducted simulation based on the proposed channel model signal tap channel mode,  bi-directional and uni-directional proposed in [4]

PUSCH simulation results

For PUSCH, the channel model with bi- directional and uni- directional are selected for evelation with the following  RS configuraion as

- 1 DMRS +1 PTRS

- 2 DMRS+ 1PTRS

- 3 DMRS only

Scenario A (Ds = 700m , Dmin = 10m)

Figure 4-1 PUSCH requiremnt with bidirectional and un-directional scenario, 350km/h@30GHz

Figure 4-2 PUSCH requiremnt with bidirectional and un-directional scenario, 260km/h@30GHz

Figure 4-3 PUSCH requiremnt with bidirectional and un-directional scenario, 350km/h@28GHz

Based on above simulation, the following observations can be obtained.

Observation 4: Similar performance can be achieved for both bi-directional and un-directional deployment scenario.

Observation 5:  With 1 DMRS+PTRS (L=1, K=2) configuration, better performance can be achieved in terms of maximum throughput.

Observation 6:  With only DMRS (1+1+1) configuration, the achievable throughput is very lower, it is not feasible to support 350km/h @ 30GHz carrier frequency.

Observation 7: From UL demodulation perspective, it is feasible to support the maximum speed with 350km/h, even with high carrier frequency up to 30GHz with PTRS configured.

Scenario B (Ds = 700m , Dmin = 150m) (to be updated)

PDSCH simulation results

For PUSCH, the channel model with single tap, un-directional DPS and bi-direction DPS channel model are selected for evelation

Detailed simulation assumption is below:

Table 4-1 performance simulation assumption

| channel profile | antenna configuration | layer number | modulation and code rate scheme (refer to Table 5.1.3.1-1 in [2]) | reference |
| --- | --- | --- | --- | --- |
| Single Tap channel | 1x2 | 1 | MCS13,MCS17,MCS19 | Table 5.2.2.2.1-3 in 5.2.2.2.1 in [3] |
| Uni-directional DPS Channel | 2x2 | 1 or 2 | MCS13,MCS17 | Table 5.2.2.2.10-3 in 5.2.2.2.10 in [3] |
| Bi-directional DPS Channel | 2x2 | 1 or 2 | MCS13,MCS17 | Table 5.2.2.2.10-3 in 5.2.2.2.10 in [3] |

Table 4-2 maximum throughput of simulation cases

| simulate MCS | maximum throughput(Mbps) |
| --- | --- |
| MCS13 layer1 | 86.8864 |
| MCS17 layer1 | 118.054 |
| MCS19layer1 | 138.496 |
| MCS13 layer2 | 173.722 |
| MCS17 layer2 | 236.006 |

SingleTap Channel Simulation Results

| v = 260km/h | v = 350km/h |
| --- | --- |

Figure4-5 single tap channel simulation results

Observation8: For rank1 transmission, it is feasible to support UE speed up to 350km/h with single tap channel

Observation9: For rank1 transmission with 260km/h, the maximum throughput for MCS 17 and MCS19 can be achieved around 10 dB SNR.

Observation10: For rank1 transmission with 350km/h, the maximum throughput for MCS 13 can be achieved around 4 dB SNR. The maximum throughput for MCS 17 can be achieved around 12 dB SNR.

Uni-directional DPS Channel Simulation Results

| v = 260km/h | v = 350km/h |
| --- | --- |

Figure 4-2 Uni-directional DPS simulation results

Observation11:  Similar performance can be archived for scenario A (Dmin=10) and scenario B (Dmin=150) in uni-directional scenario with DPS transmission scheme.

Observation12: For rank1 transmission, it is feasible to support UE speed up to 350km/h in uni-directional scenario with DPS transmission scheme

Observation13: For rank1 transmission with 260km/h UE speed, the maximum throughput for MCS 17 can be achieved round 10 dB SNR

Observation14: For rank2 transmission with 260km/h UE speed, the maximum throughput of MCS 17 can be achieved around 15 dB

Observation15: For rank1 transmission with 350km/h UE speed, the maximum throughput for MCS 17 can be achieved around 18 dB

Observation16: For rank2 transmission with 350km/h UE speed, the maximum throughput for MCS 17 cannot be achieved, the SNR of 70% maximum throughput is about 12 dB , the maximum throughput of MCS13 can be achieved around 12 dB

Bi-directional DPS Channel Simulation Results

| v = 260km/h | v = 350km/h |
| --- | --- |

Figure 4-3 Bi-directional DPS simulation results

Observation17:  Similar performance can be archived for scenario A (Dmin=10) and scenario B (Dmin=150) in bi-directional scenario with DPS transmission scheme.

Observation18: For rank1 transmission, it is feasible to support UE speed up to 350km/h in bi-directional scenario with DPS transmission scheme

Observation19: For rank1 transmission with 260km/h UE speed, the maximum throughput for MCS 17 can be achieved round 10 dB SNR

Observation20: For rank2 transmission with 260km/h UE speed, the maximum throughput of MCS 17 can be achieved around 12 SNR

Observation21: For rank1 transmission with 350km/h UE speed, the maximum throughput for MCS 17 can be achieved around 19 dB SNR

Observation22: For rank2 transmission with 350km/h UE speed, the maximum throughput of MCS 17 cannot achieved, the SNR of 70% maximum throughput is about 12 dB, the maximum throughput of MCS13 can be achieved around 12 dB SNR

Conclusion

In this contribution, the initial simulation results for maximum supported speed from UL and DL demodulation perspective were provided.

First of all, we bring theoretical analysis for supporting maximum velocity based on different RS configuration for frequency offset tracking.

Theoretical analysis for maximum supported velocity with different RS configuration

- Observation 1: It’s feasible to support up to 350km/h with 120kHz TRS configured for frequency offset tracking from DL perspective.

- Observation 2:  It is not feasible to support maximum 350km/h velocity with targeting 30GHz carrier frequency under only DMRS configuration for UL .

- Observation 3: It’s feasible to support up to 350km/h with combined DMRS + PTRS (120kHz) used for frequency offset tacking for UL.

Then we bring evaluation results for both uplink and downlink with different channel modelling/deployment scenarios.

UL performance evaluation

- Observation 4: Similar performance can be achieved for both bi-directional and un-directional deployment scenario.

- Observation 5:  With 1 DMRS+PTRS (L=1, K=2) configuration, better performance can be achieved in terms of maximum throughput.

- Observation 6:  With only DMRS (1+1+1) configuration, the achievable throughput is very lower, it is not feasible to support 350km/h @ 30GHz carrier frequency.

- Observation 7: From UL demodulation perspective, it is feasible to support the maximum speed with 350km/h, even with high carrier frequency up to 30GHz with PTRS configured.

DL demodulation –Single Tap

- Observation 8: For rank1 transmission, it is feasible to support UE speed up to 350km/h with single tap channel

- Observation 9: For rank1 transmission with 260km/h, the maximum throughput for MCS 17 and MCS19 can be achieved around 10 dB SNR.

- Observation 10: For rank1 transmission with 350km/h, the maximum throughput for MCS 13 can be achieved around 4 dB SNR. The maximum throughput for MCS 17 can be achieved around 12 dB SNR.

DL demodulation - Uni-direction RRH deployment with DPS scheme

- Observation 11:  Similar performance can be archived for scenario A (Dmin=10) and scenario B (Dmin=150) in uni-directional scenario with DPS transmission scheme.

- Observation 12: For rank1 transmission, it is feasible to support UE speed up to 350km/h in uni-directional scenario with DPS transmission scheme

- Observation 13: For rank1 transmission with 260km/h UE speed, the maximum throughput for MCS 17 can be achieved round 10 dB SNR

- Observation 14: For rank2 transmission with 260km/h UE speed, the maximum throughput of MCS 17 can be achieved around 15 dB

- Observation 15: For rank1 transmission with 350km/h UE speed, the maximum throughput for MCS 17 can be achieved around 18 dB

- Observation 16: For rank2 transmission with 350km/h UE speed, the maximum throughput for MCS 17 cannot be achieved, the SNR of 70% maximum throughput is about 12 dB , the maximum throughput of MCS13 can be achieved around 12 dB

DL demodulation- Bi-direction RRH deployment with DPS scheme

- Observation 17:  Similar performance can be archived for scenario A (Dmin=10) and scenario B (Dmin=150) in bi-directional scenario with DPS transmission scheme.

- Observation 18: For rank1 transmission, it is feasible to support UE speed up to 350km/h in bi-directional scenario with DPS transmission scheme

- Observation 19: For rank1 transmission with 260km/h UE speed, the maximum throughput for MCS 17 can be achieved round 10 dB SNR

- Observation 20: For rank2 transmission with 260km/h UE speed, the maximum throughput of MCS 17 can be achieved around 12 SNR

- Observation 21: For rank1 transmission with 350km/h UE speed, the maximum throughput for MCS 17 can be achieved around 19 dB SNR

- Observation 22: For rank2 transmission with 350km/h UE speed, the maximum throughput of MCS 17 cannot achieved, the SNR of 70% maximum throughput is about 12 dB, the maximum throughput of MCS 13 can be achieved around 12 dB SNR

To summary, based on our evaluation results and theoretical analysis over different deployment scenarios for DL and UL, the initial conclusion as following:

Initial conclusion: It’s feasible to support UE velocity up to 350km/h under various deployment scenarios with 120kHz PTRS configured in uplink and 120kHz TRS configured in downlink.

Reference

[1] R4-2103240, “Way forward on Deployment Scenario and UE RF Requirement for FR2 HST”, Samsung.

[2] 3GPP TS 38.214 V16.0.0, “NR Physical layer procedures for data”.

[3] 3GPP TS 38.101-4 V16.3.0, “NR User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”.

[4] R4-2105025, “Channel modeling for FR2 HST and TP to TR 38.854”, Samsung

Appendix

PUSCH simulation assumpion

Table 7-1: PUSCH simulation assumpion

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna layout |  | 1T2R, ULA Low Correlation |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | TBD |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 8 or 9 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
|  | Bandwidth | 100 MHz |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled, or KPT-RS=2 if PT-RS is enabled |
|  | Time density (LPT-RS) | Disabled, or LPT-RS = 1 or 2 or 4 if PT-RS is enabled |
| MCS |  | MCS16 |
| Propagation channel |  | Uni-directional: Derived based on agreed deployment scenario. Bi-directional: Single-Tap channel in TS38.104 but updated parameters |

PDSCH simulation assumpion

Table 7-2:  PDSCH simulation assumpion

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| CBW and SCS |  |  | 120kHz, 100MHz (66PRB) |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 3D1S1U (FR2.120-1) S=10D:2G:2U |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  | CSI-RS periodicity | Slots | 80 (10ms) |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | Option 1: 1 Option 2: 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Propagation channel |  |  | Uni-directional: Derived based on agreed deployment scenario. Bi-directional: Single-Tap channel in TS38.101-4 B.3 but updated parameters. |
| Antenna configuration |  |  | 2x2 ULA Low Correlation |
| Number of MIMO layers |  |  | 1 |
| MCS |  |  | Option 1: 16QAM 0.5 (MCS 13 with Table 1) Other options are not excluded |