3GPP TSG-RAN WG4 Meeting # 98-bis-e 									            R4-2105479

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			9.3

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [98-bis-e][142] FS_NR_600MHz_ext

Document for:	Information

# Introduction

This email discussion is divided into three topics

- Reply LS from APT/AWG-27

- Coexistence with other services, AI 9.3.3

- Frequency arrangement, AI 9.3.4

It may be useful to recall the SID objectives:

“Specifically, this study item includes the following objectives:

- Regulatory study of the frequency range around 600MHz in Region 3

- Co-existence study for the frequency range of 612-652/663-703 MHz such as with DTV (if needed)

- Study potential frequency arrangements and conclude the possible implications (such as insertion loss, transmitter and receiver characteristics for both BS and UE, system limitations such as channel bandwidths, etc.) of different duplex filter implementations.

- Consider options B1 and B2 from AWG LS, but other options are not precluded.

- Answer the request from AWG regarding the technical feasibility of option B1 and B2, respectively. Further options are not precluded and may be included in LS to AWG.

NOTE: Since regulatory study of frequency range around 600MHz is for Region 3, the SI outcome will not impact any requirements defined for US 600MHz band.”

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: a short-list of filter configurations for B1 and B2 (or other arrangement) for further study

- 2nd round: agreement on two WF capturing conclusions that allowing progress on the studies of frequency arrangements B1, B2 and B2a (n71 + nX as proposed in R4-2104891)

# Topic #1: Reply LS from APT/AWG-27

A letter of Mr. Masanori Kondo, Secretary General, Asia-Pacific Telecommunity (APT) regarding ‘Reply Liaison Statement to 3GPP RAN, 3GPP RAN4 by the 27th Meeting of the APT Wireless Group (AWG-27)’ for appropriate action. The LS is for information and comment.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2105105 | APT/AWG-27 | Title: FREQUENCY ARRANGEMENTS FOR IMT IN THE BAND 470 – 703 MHz The main part of the LS copied below for convenience: “AWG would like to make the following comments and considerations specifically to the RAN4 on their current direction. - Options B1 and B2 - AWG primary priority is to determine the technical feasibility of options B1 and B2, as requested in our liaison statement to 3GPP and to understand the best options for implementation into terminal devices and network equipment, considering the substantial technical development ongoing. AWG kindly asks 3GPP to consider B1 from a technical implementation perspective. For option B2 the question from AWG is whether this is at all feasible to implement in a single duplexer that would then contain the 3GPP band 71/n71, OR if a second duplexer is needed, what is the best size of the second duplexer to ensure efficient performance aspects in the implementation in particular in the terminal devices. AWG will later consider the regulatory aspects such as licensing when the response from 3GPP is conveyed. - AWG request that the 3GPP prioritize to complete the technical feasibility analysis of option B1 and B2 before studying other options in detail. If option B2 can be implemented using a single duplexer this should be considered a variant of B2 but with a different filter implementation. This variant should also be considered before considering other options. - Economies of scale of Band n71 - Option B2 has two overlapping duplexers (or one duplexer if that is possible). One of these implementation options is similar to that of Band n71. This can be considered as a starting point to evaluate option B2 possible to create a large ecosystem for both Region 3 and Region 2. However, other filter arrangements such as the variation of option B2 as described in point 1b above, and are independent from Band N71, may also be considered. - Two bands: Band 71 + Band nX - The two-band approach is not the current priority of AWG. If there are severe difficulties with the option B1 or B2 as suggested by AWG that is found by the 3GPP study, it is of interest to consider this onwards, but AWG requests feedbacks on the technical feasibility of B1 and B2 first. - Base stations and handsets will have to support both bands, and ENDC band combinations available for Band n71, may not be available for bands allowed in region 3. - Requirements to support Band n71. - In doing the feasibility studies for options B1 and B2. The constraints to meet the requirements of n71 must include flexibility to cater for differences in regulatory requirements in markets in R1, R2 and R3.It would be desirable if some economies of scale in duplexer design can be taken advantage of.” |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1	Reply LS from APT/AWG-27

Sub-topic description: the APT/AWG-27 LS for’ information and comments’. One issue is listed below: whether to reply at this meeting or later (the items of the LS covered by the topics below).

Open issues and candidate options before e-meeting:

Issue 1-1-1: Reply LS to APT/AWG

- Proposals

  - Option 1: Reply at this meeting

  - Option 2: Reply after RAN4#100-e (August) according to the SID

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by m

| Company | Comments |
| --- | --- |
| Nokia | Sub topic 1-1-1: Option 2. It is not necessary to start drafting reply LS this meeting, as the next AWG is September. We can compile the technical information and decisions in RAN4 August meeting. |
| Spark | At this stage there is no need to send a LS to AWG as they are not meeting until September. It is expected that in the RAN 4 meetings later this year more information will be available on the feasibility of options B1 and B2 and we should present this to AWG with the advantages and advantages of each. We could indicate a preference for an option if both are feasible but leave the decision to AWG. |
| Ericsson | The additional comments from APT on their expectations are very clear and should be used to guide RAN4 work. We don’t think we should rush answering with another LS as long as there is no clear outcomes of the SI. |
| Huawei | Issue 1-1-1: According to the LS, AWG expects 3GPP feedback before the AWG-28 meeting, scheduled in September 2021. Therefore it is preferred to focus on the RAN4 analyses now and postpone the LS out till August: Option 2. |
| Apple | Issue 1-1-1: prefer to continue the work according to the approved work plan (i.e. Option 2). |
| CATT | According to the AWG meeting schedule, there is no urgency to reply the LS in this meeting. it is proposed to continue the technical discussion. |
| Telstra | Telstra notes the strong direction given by APT and agrees this should prioritise the work in RAN4. There is no need for a response to be drafted during this meeting for all the reasons stated by the companies above. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Tentative agreements: Option 2 (Reply after RAN4#100-e (August) according to the SID) Candidate options: Recommendations for 2nd round: no discussion, no action needed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: Coexistence with other services

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104717 | Nokia, Nokia Shanghai Bell, CBN | Title: Coexistence for APT 600 MHz Observation 1: Option B1 and B2 can coexist with the broadcast service below 610 and 605 MHz, respectively, assuming the minimum guard-band of 7 MHz. Observation 2: Option B2 may require vacating one more TV channel depending on TV channel raster. Proposal 1: No specific BS spurious emission requirement to protect the broadcast service is considered in this study item. Observation 3: The coexistence requirement with radio astronomy are out of scope of 3GPP. Proposal 2: No specific BS spurious emission requirement to protect the radio astronomy service is considered in this study item. Proposal 3: UE coexistence with band 28 shall be based on the protection level of -50 dBm/MHz. Proposal 4: For the protection of own downlink band, NS and A-MPR solution is further discussed. |
| R4-2104931 | ZTE Corporation | Title: Coexistence study for extended 600 MHz NR frequency band Observation 1: For option B1, only the DTV service below 605MHz can coexist with extended 600MHz. For option B2, the DTV service below 610MHz can coexist with extended 600MHz. This difference may cause Option B1 and Option B2 to encounter different coexistence restrictions. For option B1, this restriction may cause the DTV system to concede an additional channel. Observation 2: For option B2, DTV system with various channel rasters(6/8MHz) will not exceed the coexistence boundary of 610MHz. |
| R4-2105094 | Xiaomi | Title: Further discussion on frequency arrangement for extended 600MHz NR Band Proposal 1: Still insist on a high priority of Option B1. Proposal 2: An asymmetric band could be considered as well, i.e., UL: 663MHz-703MHz, DL: 617MHz-652MHz. |
| R4-2107348 | Qualcomm Incorporated | Title: Filtering for extended 600 MHz band Conclusions [excerpt]: […]. A single filter is highly preferred over a dual filter solution, if feasible. The key aspect explored in this contribution is whether the single filter supporting the extended 600 MHz frequency range could still be used by the UE to meet Band 71/n71 requirements. […] |

## Open issues summary

### Sub-topic 2-1	BS coexistence with RAS

Sub-topic description: BS coexistence with RAS, considerations of methods other than geographical separation

Open issues and candidate options before e-meeting:

Issue 2-1-1: Frequency separation between the Extended 600 MHz band and RAS (BS)

- Proposals

  - Option 1: Consider frequency separation to RAS as discussed in R4-2106891. State why.

  - Option 2: Do not consider frequency separation to RAS as discussed in R4-2105094. State why not.

- Recommended WF

  - TBA

Issue 2-1-2:  Spurious emissions limit for protection of RAS (BS)

- Proposals

  - Option 1: No limit as proposed in R4-2104717. State why.

  - Option 2: Specify a limit (3GPP only).

  - Option 3: No limit, but consider methods for BS coexistence with RAS

  - Option 4: Reuse recommendations based on FCC (TR 36.755)

- Recommended WF

  - TBA

### Sub-topic 2-2 Coexistence with Broadcast Services

Sub-topic description: coexistence with Broadcast services, both BS and UE

Open issues and candidate options before e-meeting:

Issue 2-2-1: Frequency separation to Broadcast (DTV) for BS

- Proposals

  - Option 1: Consider/reuse the FCC frequency separation (7 MHz) for protection of Broadcast as discussed in R4-2104717 and R4-2104931

  - Option 2: Other separation or method for coexistence (state what)

- Recommended WF

  - TBA

Issue 2-2-2:  BS spurious emissions limit for protection of Broadcast

- Proposals

  - Option 1: No limit as proposed in R4-2104717. State why.

  - Option 2: Specify a limit (3GPP only).

- Recommended WF

  - TBA

Issue 2-2-3: Blocking requirement for UE for protection against Broadcast (DTV)

- Proposals

  - Option 1: needed for interferers below 608 MHz (or other interferer range, state which)

  - Option 2: needed for protection from US CH36 (602-608 MHz) for operations in Band n71, if applicable, reuse n71 blocking requirement

  - Option 3: the n71 blocking requirement for CH36 protection not feasible for B1/B2 or other, state for which frequency arrangement

  - Option 4: not needed as discussed in R4-2107348

  - Option 45: other

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by m

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Skyworks | Issue 2-2-3: As explained in our contribution R4-2104817, the option B1 has serious issues in allowing enough attenuation of the TV channel 36. The same issue is also visible from simulations in R4-2107348. Potential UE receiver blocking needs further analysis before B1 can be claimed to be reusable for n71. For B2 however if split duplexer is used then n71 performance stays untouched. For Aisa, the nearest TV channel should be known to decide if the same blocking issue may arise. This is not only a matter of requirement on 3GPP side and it is unclear if it is suggested that n71 blocking requirements should change. |
| Nokia | Issue 2-1-1: Frequency separation between the Extended 600 MHz band and RAS (BS) Neither option. Coexistence with RAS is up to each administration; no detail is available to RAN4. It may be treated as regional specific requirement later if some inputs are available from administrations. Issue 2-1-2: Spurious emissions limit for protection of RAS (BS) Option 1: Coexistence with RAS is up to each administration; no detail is available to RAN4. It may be treated as regional specific requirement later if some inputs are available from administrations. The protection mechanism may not be based on BS spurious emission limit like in the FCC case. Issue 2-2-1: Frequency separation to Broadcast (DTV) for BS Option 1: Reusing the same assumption as band 71 is proposed. Issue 2-2-2: BS spurious emissions limit for protection of Broadcast Option 1: Reusing the same assumption as band 71 is proposed. Issue 2-2-3: Blocking requirement for UE for protection against Broadcast (DTV) Option 5: For B2, we are fine with Option 2, i.e., the same in-band and out-of blocking requirement as band 71 is applied for the lower side of duplex. For B1, the Option 1 would be required, i.e., the in-band/out-of-band blocking frequency is shifted downward by 5 MHz, so that the same guard band size is assumed. CH36 would need to be vacated in B1 option. Coexistence with CH36 is not possible for B1. |
| Qualcomm | Issue 2-2-3: With B1, the blocking protection against Ch36 may not be met in all circumstances. It is up to local regulators to decide whether the potential, if at all, blocking interference depending on the circumstances in their country is significant enough to warrant the allocation of prime spectrum as guard band. It is not necessary or appropriate to define UE blocking requirements at this time since that is out of scope of the SI. RAN4 is not defining a new band and its requirements within the scope of this SI. |
| Spark | Issue 2-1-1: Frequency separation between the Extended 600 MHz band and RAS (BS) There are two foot notes 5.305 and 5.307 in the radio regulations about RAS in China and India respectively. These footnotes can only be removed by a WRC and by people authorised to speak on behalf of their respective Administrations. Therefore, it is best to leave this issue to the individual Administration. Issue 2-1-2: Spurious emissions limit for protection of RAS (BS) This issue should also be left to individual Administration. |
| Ericsson | Sub topic 2-1: Note that this topic was already addressed in RAN4#98-e meeting. Issue 2-1-1: None: According to RR, RAS are operated in India and China. National Regulator shall protect those services which could be based on frequency separation or most likely on distance separation. Issue 2-1-2: Option 1: This will depend on National Regulation and should be considered on a country basis (India and China). Sub topic 2-2: Note that this topic was already addressed in RAN4#98-e meeting. Issue 2-2-1: Option 2: there are various DTV systems in Region 3, not only US one. It was already agreed to leave this aspect up to each national regulator, as captured in the agreed TP to TR (R4-2103267) Digital television has various channel allocations depending on country allocations in Region 3. IMT and DTV coexistence may require additional consideration on a per-country basis, leading to potential need for a guard bands. Issue 2-2-2: Option 1 for the reasons given for issue 2-2-1 |
| ZTE | Issue 2-1-1: Option 2. In fact, the RAS system are mostly deployed in the remote areas. For instance, China’s Five-hundred-meter Aperture Spherical radio Telescope(FAST) is located in a depression in a mountainous area, so it is not difficult to achieve geographic isolation from IMT system. Therefore, there is no need for frequency separation between RAS and IMT system. In addition, this issue requires the efforts of administration rather than SDOs. Issue 2-1-2: Option 1. The reason is same as above. Issue 2-2-1: Option 1. The implementation method of the APT 600 MHz band and the US 600 MHz band are highly similar, so it is reasonable and efficient to reuse existing experience to solve this issue. Issue 2-2-2: Option 1, As discussed in R4-2104931, there is a 7MHz frequency separation between the broadcast and IMT systems, and this frequency separation could provide adequate protection for Broadcast. This fact is validated by FCC. Issue 2-2-3: Option 5. For B2, there is no potential coexistence issue with Broadcast, so there is no need to impose restriction on UE. As for B1, in many countries, it is difficult to see the possibility that B1 can coexist with the Broadcast, so a delicate balance are needed to ensure that B1 will not interfere with the Broadcasting, which may require further study. |
| Huawei | Sub topic 2-1: BS coexistence with RAS Issue 2-1-1: referring to the band 71 studies, co-existence with RAS can be achieved by distance separation. Taking decision on the band plan based on location/deployment specific conditions may not be optimal from APT point of view. Issue 2-1-2: despite the fact that we are not supposed to define requirements in SI, any country-specific (i.e. due to raster variations) TV protection requirements may be introduced as regional requirements. Therefore, keeping the general BS spurious emission as the assumption for the SI discussion is ok. Sub topic 2-2: Coexistence with Broadcast Services Issue 2-2-1: this sounds like reasonable assumption, but we would like to have more time to double-check the related analysis for the n71. Option 1 as tentative agreement (more analysis to be allowed for next meeting). Issue 2-2-2: similar as 2-1-2: despite the fact that we are not supposed to define requirements in SI, any country-specific (i.e. due to raster variations) TV protection requirements may be introduced as regional requirements. Therefore, keeping the general BS spurious emission as the assumption for the SI discussion is ok. Issue 2-2-3: Huawei: Option5: For B1, propose to continue discussion on the implications of Rx blocking performance due the nearby DTV channels. For B2: consider n71 requirements as the starting point for the lower bound. |
| Xiaomi | Issue 2-1-1: Option 2. It can be achieved by geographic separation, since the RAS system are deployed in remote area mostly. Issue 2-1-2: Option 1 |
| CATT | Issue 2-1-1: Frequency separation between the Extended 600 MHz band and RAS (BS) RAS system is mostly deployed in the remote areas. Usually geographic sepration can provide enough isolation. We propose leave it to each administration. Issue 2-1-2: Spurious emissions limit for protection of RAS (BS) Same answer as for Issue 2-1-1. Issue 2-2-1: Frequency separation to Broadcast (DTV) for BS Frequency separation might be a valid solution. But we propose to leave this up to each administration. Issue 2-2-2: BS spurious emissions limit for protection of Broadcast Same answer as for Issue 2-2-1. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | Tentative agreements: Issue 2-1-1: none of the options, protection of RAS up to each Administration Issue 2-1-2: no BS spurious emissions limit for protection of RAS Candidate options: Recommendations for 2nd round: no further discussions, but the conclusions above should be captured in a WF and subsequently in the TR WF on ‘Coexistence with other services’ (see 4.1) |
| Sub-topic #2-2 | Tentative agreements: Issue 2-2-1: no consensus, possibly discuss the frequency separation considered feasible for each frequency arrangement, or if this should be left for Administrations Issue 2-2-2: no specific limits, keep general BS spurious emissions limits Issue 2-2-3: no consensus, possibly list items for further study in a WF e.g. rejection at a specific frequency separation for frequency arrangements (without discussing any specific blocking requirement for protection from DTV) Candidate options: Recommendations for 2nd round: a WF containing conclusions on 2-2-2 above and items for further study (2-2-1 and 2-2-3) WF on ‘Coexistence with other services’ (see 4.1) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

Comments on the WF on ‘Coexistence with other services’ on the reflector or below

### WF comments collection

Comments on the WF on ‘Coexistence with other services’

| CR/TP number | Comments collection |
| --- | --- |
| R4-2105421 | Company A |
|  | Company B |

# Topic #3: Frequency arrangement for Extended 600 MHz NR band

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104495 | Spark NZ Ltd | Title: Option B2 for Extended 600MHz NR band This contribution presents two variants of options B2 that could be further studied and are presented to RAN 4 for further study. |
| R4-2104718 | Nokia, Nokia Shanghai Bell | Title: Frequency arrangements for APT 600 MHz Observation 1: The ecosystem of band n71 should be maximally reused for APT. Observation 2: The passband bandwidth extension is not practically feasible without significant degradation from band n71 due to its larger insertion loss. Observation 3: Option B2 is more harmonized with band n71 in terms of UE implementation and duplexer performance. Observation 4: Option B2 is a natural extension of n71 and is not harmful to n71 ecosystem. Proposal 1: Option B1 shall be discouraged for APT/AWG to proceed. Observation 5: The upper duplexer passband bandwidth can be up to UE implementation as far as UE can support any channel bandwidth in any carrier frequency within the band and can meet the same requirement as band n71 for the entire frequency range. Proposal 2: UE RF requirement such as MOP and REFSENS shall be the same as n71. Proposal 3: For the protection of own downlink band, NS and A-MPR solution is further studied. Observation 5: Single duplexer implementation is not excluded as far as the requirement baseline based on dual duplexer assumption is fulfilled. Proposal 4: Other duplexer implementation than B2 is not precluded but the frequency arrangement and RF requirement baseline should be based on Option B2. |
| R4-2104817 | Skyworks Solutions Inc. | Title: Discussion on Extended 600MHz Band Implementation Options Proposal: - To enable n71 eco-system reuse together with guaranteed n71 performance a split duplexer approach is selected - Degradation of the DL upper 5MHz band protection and blocking performance needs to be accommodated. - Both B2 (two 2x35MHz duplexer) and B2- with 2x35MHz + 2x25MHz solutions are accommodated with additional BW constraint in the upper duplexer. |
| R4-2104891 | Apple | Title: TP on band plan for 600 MHz Proposal 1: It is proposed to approve the text proposal provided in this contribution. |
| R4-2105094 | Xiaomi | Title: Further discussion on frequency arrangement for extended 600MHz NR Band Proposal 1: Still insist on a high priority of Option B1. Proposal 2: An asymmetric band could be considered as well, i.e., UL: 663MHz-703MHz, DL: 617MHz-652MHz. |
| R4-2106593 | ZTE Corporation | Title: Discussions on Option B1 and B2 for extended 600MHz Observation 1: there should be no issues between extended 600MHz and n28; Observation 2: both Option B1 and B2 should be feasible from BS perspective; Observation 3: it might be not easy to support 30MHz with single duplexer from UE perspective; Observation 4: to reuse asymmetric UL 20MHz/DL 35MHz bandwidth configuration in n71 UE side for extended 600MHz; |
| R4-2106891 | Ericsson | Title: APT 600 MHz band – frequency arrangements Proposal1: Consider frequency arrangement option B2 for the new 600MHz band. Proposal 2: the bandwidth support and performance requirements should be based on a 2 x 30 MHz split duplexer with due allowance to facilitate implementation of a single 40 MHz filter. |
| R4-2107301 | Huawei, HiSilicon | Title: Feasibility analysis of 600MHz duplexer Observation 1: From the evaluation, it is observed that full band duplexer with 11MHz gap in B1 and dual-duplexer in B2 are both implementation feasible, while full band duplexer with 6MHz gap is not feasible for the current filter implementation. Observation 2: For Option B1, a full band duplexer could have identical performance as existing B71/n71 duplexer while it may have worse Rx blocking issue as the Rx frequency range is closer to DTV CH36. Observation 3: Option B2 needs to pay more attention to the co-existence issue for the duplexer design. The existing B71/n71 duplexer cannot be reused. Proposal 1: It is proposed to focus on the band plan option B1 based on the duplexer feasibility analysis. |
| R4-2107348 | Qualcomm Incorporated | Title: Filtering for extended 600 MHz band Conclusions [excerpt]: […]. A single filter is highly preferred over a dual filter solution, if feasible. The key aspect explored in this contribution is whether the single filter supporting the extended 600 MHz frequency range could still be used by the UE to meet Band 71/n71 requirements. […] |

## Open issues summary

### Sub-topic 3-1	Filter configuration and requirements for B1

Sub-topic description: filter configuration for B1 (SI conclusions) and associated requirements

Open issues and candidate options before e-meeting:

Issue 3-1-1: Filter configuration for B1

- Proposals

  - Option 1: Single 2 x 40 MHz duplexer

  - Option 2: Split duplexer, at least one of the duplexers with 35 MHz passband

  - Option 3: Split duplexer, other passband sizes

- Recommended WF

  - TBA

Issue 3-1-2: Band coexistence requirements for B1 (with other 3GPP bands)

- Proposals

  - Option 1: reuse the (3GPP) band coexistence requirements applicable for B71/n71

  - Option 2: other

- Recommended WF

  - TBA

Issue 3-1-3: Protection of own DL

- Proposals

  - Option 1: use the standard requirement (-50 dBm/MHz) and implied blocking requirement (‘standard’ TX rejection at RX)

  - Option 2: use NS signalling and A-MPR for protection of own DL

  - Option 3: other

- Recommended WF

  - TBA

Issue 3-1-4: MOP and REFSENS

- Proposals

  - Option 1: same as B71/n71

  - Option 2: other

- Recommended WF

  - TBA

### Sub-topic 3-2  Filter configuration and requirements for B2

Sub-topic description: filter configuration for B2 (for SI conclusions) and associated requirements

Open issues and candidate options before e-meeting:

Issue 3-2-1: Filter configuration for B2

- Proposals

  - Option 1: Consider single 2 x 40 MHz duplexer (any feasible technology if applicable)

  - Option 2: Split duplexer, at least one of the duplexers with 35 MHz passband

  - Option 3: Split duplexer, other passband sizes

- Recommended WF

  - TBA

Issue 3-2-2: Band coexistence requirements for B2 (with other 3GPP bands)

- Proposals

  - Option 1: reuse the (3GPP) band coexistence requirements applicable for B71/n71

  - Option 2: other

- Recommended WF

  - TBA

Issue 3-2-3: Protection of own DL

- Proposals

  - Option 1: use the standard requirement (-50 dBm/MHz) and implied blocking requirement (‘standard’ TX rejection at RX)

  - Option 2: use NS signalling and A-MPR for protection of own DL

  - Option 3: other

- Recommended WF

  - TBA

Issue 3-2-4: MOP and REFSENS

- Proposals

  - Option 1: same as B71/n71

  - Option 2: other

- Recommended WF

  - TBA

### Sub-topic 3-3  Other band arrangements

Sub-topic description: one proposal is to use band 71/n71 for covering the frequency range 663-698MHz for UL and 617-652MHz for DL in the APT region and specify a new band covering at least the additional 2x 5MHz spectrum proposed in the SI.

Open issues and candidate options before e-meeting:

Issue 3-3-1: Option B2a

- Proposals

  - Option 1: Consider arrangement B2a as proposed in R4-2104891

  - Option 2: Only B1 and B2

  - Option 3: other

- Recommended WF

  - TBA

### Sub-topic 3-4  Maximum channel bandwidth

Sub-topic description: the filter configuration assumed for the minimum requirements will determine the maximum channel bandwidth supported.

Open issues and candidate options before e-meeting:

Issue 3-4-1: Maximum channel bandwidth supported (any frequency arrangement)

- Proposals

  - Option 1: 40 MHz

  - Option 2: 35 MHz

  - Option 3: 30 MHz

  - Option 4: 20 MHz

  - Option 5: other

- Recommended WF

  - TBA

Issue 3-4-2: Asymmetric channel bandwidth

- Proposals

  - Option 1: Support asymmetric bandwidth as discussed in R4-2106593

  - Option 2: No, only symmetric channel bandwidths

- Recommended WF

  - TBA

### Sub-topic 3-5  Leverage Band 71/n71 for the Extended 600 MHz band?

Sub-topic description: use B71/n71 to leverage the Extended 600 MHz? Either from an implementation perspective and/or for network operations.

Open issues and candidate options before e-meeting:

Issue 3-5-1:

- Proposals

  - Option 1: use B71/n71, state how

  - Option 2: do not use B71/n71 as leverage, state why not

- Recommended WF

  - TBA

### Sub-topic 3-6  Band arrangement(s) for SI conclusion

Sub-topic description: what to capture in the SID conclusion?

Open issues and candidate options before e-meeting:

Issue 3-6-1: frequency arrangement(s) for the SI conclusion

- Proposals

  - Option 1: include B1, B2 and any other arrangement studied as stated in the SID objective

  - Option 2: conclusion with preferred frequency arrangement

- Recommended WF

  - TBA

### Sub-topic 3-7  Preferred band arrangement

Sub-topic description: can consensus on one frequency arrangement be achieved during the study phase? (Note that this is not listed as a SID objective.) Find a poll below.

Open issues and candidate options before e-meeting:

Issue 3-7-1: Preferred frequency arrangement

- Proposals

  - Option 1: B1

  - Option 2: B2

  - Option 3: other

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

Example 2

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Skyworks | Issue 3-1-1: In our view if B1 is studied it should be for full duplexer only as split duplexer approach can't reuse the band n71 filter as is. With current analysis our preference is B2 anyhow. Issue 3-1-2: In the case of full duplexer B1, band 71/n71 coexistence requirement should applyIssue 3-1-3: for B1 -50dBm/MHz can be targeted as the duplex gap is 11MHz as for n71, it is not possible to relax band 71 own DL protectionIssue 3-1-4: Use Same MOP than b71/n71 is fine but same REFSENS cannot be concluded depending on other requirements that may affect the filter IL and desense from UL. Note that n71 is up to 20MHz channel BW so far. Others: |
| Nokia | Issue 3-1-1: Filter configuration for B1 Option 1. Issue 3-1-2: Band coexistence requirements for B1 (with other 3GPP bands) Option 2: The coexisting 3GPP bands are based on the bands used in APT region. (i.e., US 700 MHz bands are not needed). Issue 3-1-3: Protection of own DL Option 1 is ok if it is feasible. Otherwise, we propose option 2. Issue 3-1-4: MOP and REFSENS Option 1 |
| Qualcomm | Issue 3-1-1: Option 1, single filter for B1 Issue 3-1-2: Agree with Nokia that n71 coexistence is not applicable for Region 3 Issue 3-1-3: Option 1 Issue 3-1-4: It’s not necessary to define refsens and MOP for this SI since we are not defining a band. But we believe that refsens analogous to B71/n71 should be achievable. |
| Spark | Issue 3-1-1: Filter configuration for B1 We are obliged to study the technical feasibility of option B1 and B2. Therefore, to suggest below as is the case in a submission to this meeting is premature: Reusing band 71/n71 as is without adding the 5 MHz below and above seems to be the only alternative to a single duplexer solution This seems like a proposal to suggest band n71 instead of studying option B1. We don’t support adding this text before a quantitative study of B1 is completed Also we cannot “ insist” on B1 without analysing both options B1 and B2 respectively. |
| Ericsson | Issue 3-1-1: Option 2 preferrable but we should also study the Option 1 for the B1 plan. This would require at least one 35 MHz filter in a split-duplexer arrangement for support of n71 requirements but is on the other hand an easier option for a full-band 40 MHz option. Option 1 appears challenging even if some companies are mentioning this would be feasible but its benefits (performance and cost) as still FFS. Also, option 1 does not allow compliance with Band 71/n71 requirements and may lead to UE fragmentation in the 600 MHz range. Issue 3-1-2: Option1 should be ok, removing bands which are not Region 3. Issue 3-1-3: Option 1 Issue 3-1-4: Option 1. |
| ZTE | Issue 3-1-1: Option 2 is more preferred, for option 1, we are also open for further discussion on its feasibility and performance/cost. Issue 3-1-2: coexistence should reply on region 3 coexistence system instead of region 1. Issue 3-1-4: Option 1. |
| Huawei | Issue 3-1-1: full band duplexer with 11MHz gap in B1 is seen as implementation feasible; option 1 Issue 3-1-2: 71/n71 analyses can be used as the baseline, with the region specifics accounted. Issue 3-1-3: Prefer Option 1. OK to support -50dBm/MHz at B28 DL frequency range. Issue 3-1-4: not sure if this needs to be decided in SI. Option 1 as baseline, but more analysis needed. |
| Xiaomi | Issue 3-1-1: Option 1, single filter for B1 Issue 3-1-2: Agree with Nokia that n71 coexistence is not applicable for Region 3 Issue 3-1-3: Option 1 Issue 3-1-4: Option 2, leave it to WI stage |
| Apple | Issue 3-1-1: Filter configuration for B1 Our preference is Option 3 (single duplexer solution with 2x30 MHz); we believe Option 2 (single duplexer with 2x35 MHz) is feasible with performance degradation. Issue 3-1-2: Band coexistence requirements for B1 (with other 3GPP bands) Option 1: reuse 71 coexistence requirements; n71 Rx should be protected from emissions by B1 (if this band plan is adopted) Issue 3-1-3: Protection of own DL Option 1: Reuse -50 dBm level Issue 3-1-4: MOP and REFSENS Option 2: We recommend concluding the band plan as the first step; in the case of B1, MOP and REFSENS performance depend on the duplexer solution. |

Sub topic 3-2

| Company | Comments |
| --- | --- |
| Skyworks | Issue 3-2-1: Option 1 is not feasible without severe performance issues with a 6MHz duplex gap. Option 2 with split duplexer with one duplexer being n71 duplexer Issue 3-2-2: option 1 band coexistence requirements for n71 (for the n71 frequncy range as there is overlap with n85) then n28 coexistence for Asia Issue 3-2-3: -50dBm/MHz is used for n71 frequency range but the upper 5MHz of B2 DL need relaxation, this can use NS signaling for this part, Issue 3-2-4: MOP and REFSENS can be reused for the n71 frequency range, the upper 5MHz may depend on upper duplexer BW choice for REFSENS but MOP should be the same |
| Nokia | Issue 3-2-1: Filter configuration for B2 We support Option 2. However, Option 1 is also fine as far as band 71/n71 requirements can be fulfilled with a single 2x40 MHz duplexer. Issue 3-2-2: Band coexistence requirements for B2 (with other 3GPP bands) Option 1 is ok if it is feasible. Otherwise, we propose option 2. Issue 3-2-3: Protection of own DL Option 1 is ok if it is feasible. Otherwise, we propose option 2. Issue 3-2-4: MOP and REFSENS Option 1. |
| Qualcomm | Issue 3-2-1: Our preference is Option 1 a single filter, but the B2 arrangement will be challenging and some compromises and tradeoffs may need to be considered and/or the use of more advanced technologies. We haven’t had the opportunity to study this filter in detail yet, so aren’t ready to dismiss or adopt any option yet. Issue 3-2-2: Option 2. Reusing n71 coexistence is not correct for Region 3 Issue 3-2-3: Option 3. The -50 dBm/MHz protection cannot be met with such a small separation. On the other hand, UE-UE coexistence is a statistical phenomenon depending on the physical separation between UE’s, their Tx and Rx power levels, etc. Applying NS and A-MPR suggests that power backoff will always be taken, which is overkill. A better solution is to relax the coexistence requirement over some portion of the Rx band. Issue 3-2-4: Option 2. Depending on the filter design, it may be possible to preserve the insertion loss over the portion of the band overlapping B71/n71 at the expense of the extended portion. But further detailed study is required to make that assessment. |
| Spark | Issue 3-2-1: Filter configuration for B2 We are obliged to study the technical feasibility of option B1 and B2. Therefore, to suggest below as is the case in a submission to this meeting is premature: “We propose to re use band 71/n71 to cover most of the frequency range and specify a new band using the additional spectrum available in APT countries” This is akin to band 71 plus band nX that is not an option preferred by APT. The intention of this study item is to study options B1 and B2. We don’t support adding this text before a quantitative study of B2 is completed |
| Ericsson | Issue 3-2-1: Option 2 or Option 3 with 2 x 30 MHz split duplexer for the B2 plan. . Requirements based on a 2 x 30 MHz split-duplexer arrangement with duplexers shifted in frequency might facilitate compliance with the standard (or slightly relaxed) requirements on attenuation at RX (DL protection) and attenuation at TX (blocking). Issue 3-2-2: Option1 should be ok, confirmed by R4-2104817. Issue 3-2-3: Due to the DL/UL proximity, some relaxation (e.g. -40dBm/MHz) should be considered, at least in the 5 MHz by the gap. Issue 3-2-4: Option 1 |
| ZTE | Issue 3-2-1: Option 2 Issue 3-2-2: Option1 . Issue 3-2-4: Option 1 |
| Huawei | Issue 3-2-1: dual duplexer Issue 3-2-2: 71/n71 analyses can be used as the baseline, with the region specifics accounted. Issue 3-2-3: Prefer Option 1 Issue 3-2-4: not sure if this needs to be decided in SI. Some REFSENS degradation may happen for the frequency close to the low edge of Dup 2 Tx. More study needed. |
| Apple | Issue 3-2-1: Filter configuration for B2 Our preference is Option 3 (split duplexer solution with 2x30 MHz); we believe Option 2 (split duplexer with 2x35 MHz) is feasible with performance degradation. Issue 3-2-2: Band coexistence requirements for B2 (with other 3GPP bands) Option 1: reuse 71 coexistence requirements; n71 Rx should be protected from emissions by B2 (if this band plan is adopted) Issue 3-2-3: Protection of own DL Option 1: Reuse -50 dBm level Issue 3-2-4: MOP and REFSENS Option 2: We recommend concluding the band plan as the first step; in the case of B1, MOP and REFSENS performance depend on the duplexer solution. |

Sub topic 3-3

| Company | Comments |
| --- | --- |
| Skyworks | Issue 3-3-1: B2a arrangement should be considered with potentially further discussion on the BW in realation with the split duplexer approach.. |
| Nokia | Issue 3-3-1: Option B2a Option 2, as recommended by APT/AWG LS. |
| Qualcomm | Issue 3-3-1. Option 2. A new upper band means splitting the spectrum over two bands and the UE might not support Band 71/n71 at all so the frequency range is not covered. Also, adding CA and DC combinations would be more complicated since they need to be added for both bands. |
| Ericsson | Issue 3-3-1: Option 2, APT confirms in the last received LS they are looking for feedback on those 2 options, asking 3GPP to prioritize the feasability analysis for both of them. The benefits of B2a are unclear as B71/n71 is not necessarily implemented with a 35 MHz duplexer. |
| ZTE | Option 2 should be prioritized to reply APT LS. |
| Huawei | Issue 3-3-1: Considering the stage of the discussion, it is seen as premature to proceed with the TP to TR – let’s first have conclusion on the band arrangement. APT LS requests to consider and study B1 and B2 first, before other options are considered: option 2 |
| Spark | Other band arrangements: issue 3-3-1 The request from AWG is very clear to consider options B1 and B2 and any consideration of band 71 + band nx is not a priority. Therefore at this stage any text proposals to this effect in the TP are not warranted |
| Apple | Issue 3-3-1: Option B2a Option 1: We would like our proposal to be considered as one of three possible solutions, alongside with B1 and B2 |
| Telstra | Issue 3-3-1: Option 2: We agree that the primary focus within the TU allocated to this study should be the study of options as directed by APT |

Sub topic 3-4

| Company | Comments |
| --- | --- |
| Skyworks | Issue 3-4-1: with split duplexer 40MHz cannot be supported, in any case like already agreed for n71 the UL BW should be limited to 20MHz to avoid significant de-sense. As of today we suggest 35MHz DL and 20MHZ UL in the band n71 frequency range and further discussion needed for other ranges. Issue 3-4-2: As discussed above asymmetric BW support is needed with UL limited to 20MHz. |
| Nokia | Issue 3-4-1: Maximum channel bandwidth supported (any frequency arrangement Option 5: First we would need to agree the filter configuration. The maximum channel bandwidth is equal to or smaller than the filter passband bandwidth. Issue 3-4-2: Asymmetric channel bandwidth Option 2: It is premature to discuss this issue. We expect UE supports band 71 so asymmetric bandwidth would be supported, but it is up to market demand if such asymmetric configuration is included in the new band. |
| Qualcomm | Issues 3-4-1: Wider bandwidths offer a single operator better efficiency, capacity, and performance. But if the spectrum is divided among multiple operators and spectrum sharing is not available, then 40 MHz channels will not be available. In the absence of information, we prefer a 40 MHz maximum bandwidth, but we need a better understanding of filter feasibility and the likelihood of a single operator owning 40 MHz or spectrum sharing to more meaningfully answer this question. Issue 3-4-2: Asymmetric UL/DL channel bandwidth will probably be a feature of this band. As observed in Band 71/n71, with the narrow Tx/Rx separation, discrete spruious products from the transmitter will land in the Rx band degrading the performance with wide UL and DL bandwidths. The exact details need further study. |
| Ericsson | Issue 3-4-1: Option 3 to guarantee better performance. Issue 3-4-2: Option 2, there is no good reason really for considering asymmetric bandwidth for the time being. |
| ZTE | Issue 3-4-1: prefer to start with DL 35MHz and UL 20MHz \ Issue 3-4-2: this is aligned with existing band n71 |
| Huawei | Issue 3-4-1: Prefer Option 1. It can be also considered to look at multiple channel values and capture related feasibility analyses / challenges in the TR. Issue 3-4-2: this was not discussed so far. More analysis needed. |
| Apple | Issue 3-4-1: Maximum channel bandwidth supported (any frequency arrangement) Option 3: 30 MHz with all three alternative band plans (B1, B2, and B2a) Issue 3-4-2: Asymmetric channel bandwidth Option 2: we prefer to focus on symmetric CBW |

Sub topic 3-5

| Company | Comments |
| --- | --- |
| Skyworks | Issue 3-5-1: in any case band n71 should be leveraged for the UE at least for the corresponding frequency range to be able to claim the economy of scale. |
| Nokia | Issue 3-5-1: Option 1. |
| Qualcomm | Issue 3-5-1: Option 1 should be a goal, but not a requirement. If possible, it would be nice to be able to fully leverage B71/n71, but we see this as a “nice-to-have” rather than a “must-have”. The spectrum plans for one part of the world should not be forcibly constrained to those of another. There are many instances of “regional bands” in the 3GPP specifications, though it would be nice to avoid that if possible. |
| Ericsson | Issue 3-5-1: Option 1 preferable regardless of band arrangement, UE fragmentation is avoided and may allow use of existing UEs for initial deployment of the extended 600 MHz band. |
| ZTE | Issue 3-5-1: Option 1. |
| Huawei | Issue 3-5-1: We have different spectrum arrangement in Region 3 as compared to Region 1. There seems to be no obvious objection to reuse n71 conclusions (where possible). Still, it is not clear if we need to have firm decision on such general statement – analysis needs to be done case by case anyways. |
| Spark | Leverage band n 71 It is not clear what is meant by this, AWG does not support band n 71 plus nX. On the other hand reuse of band n71 duplexer in the extended APT 600 band such as the case is with option B2 is another way to leverage band n71 and is worth studying further. |
| Apple | Issue 3-5-1: Leverage Band 71/n71 for the Extended 600 MHz band? Option 1: as we described in our paper, the use of n71 to partially cover this spectrum addresses key time to market concerns, as already available commercial UEs can be used in the network today. Future expansion of spectrum availability, according to B2a for example, will take time to integrate into both networks and devices. This path is a straightforward realization of the available spectrum in two phases. On the other hand, the lack of timely solutions presented by both B1 and B2 are quite evident. Furthermore, B1 has the potential to impact UE performance within n71, making this optiona particularly undesirable for UEs which strive to maintain global coverage of supported bands. The new dual duplexer definition of the B2 band plan is technically possible but has the disadvantage of not using the economy of scale of existing band 71/n71 phones. |
| Telstra | Issue: 3-5-1: While not clear what is fully meant by “use B71/n71”, it should be an underlying principle during this study that fragmentation of the ecosystem around the 600MHz spectrum needs to be avoided. |

Sub topic 3-6

| Company | Comments |
| --- | --- |
| Nokia | Issue 3-6-1: frequency arrangement(s) for the SI conclusion Option 2: One harmonized band should be pursued in 3GPP. |
| Qualcomm | Option 1. The regulators and regional administrations should decide what is best for their country. RAN4 can provide feedback on technical feasibility as asked and may define a band after the regulators have decided on the arrangement. However, it is not within 3GPP’s authority or obligation to decide a frequency arrangement or band plan for any country. |
| Ericsson | Issue 3-6-1: preferrably option 2. But option 1 might also be acceptable (SID objective anyway) if no consensus on one option to select. The TR should at least include a study of both B1 and B2 each with its assumed UE filter configuration. |
| ZTE | This SID could capture any potential implementation solutions. |
| Huawei | Issue 3-6-1: it is proposed to capture feasibility analysis for both B1 and B2 in TR – we are obliged to follow SID objective. On top of this, the preferred frequency arrangement can be derived, if possible. |
| Spark | The TR should study options B1 and B2 and their required filter arrangements |
| Xiaomi | Issue 3-6-1: frequency arrangement(s) for the SI conclusion Option1 |
| Apple | Issue 3-6-1: According to the SID, the objective of the study is to “Answer the request from AWG regarding the technical feasibility of option B1 and B2, respectively. Further options are not precluded and may be included in LS to AWG.” Given the introduction of option B2a this meeting, our understanding is that this study shall provide the technical feasibility of all three options as a response to AWG. We don’t see a specific objective to define a single preferred frequency arrangement in the SID. Thus, our preference is Option 1, including B2a |
| Telstra | Issue 3-6-1: Allocated TU for this study needs to prioritise the study around the technical feasibility of B1 and B2 options as directed by the LS from AWG. |

Sub topic 3-7

| Company | Comments |
| --- | --- |
| Nokia | Issue 3-7-1: Preferred frequency arrangement We prefer B2 as it is more harmonized with band 71 ecosystem. |
| Qualcomm | Issue 3-7-1: As stated above in subtopic 3-6, we do not feel it is appropriate for 3GPP to proffer and band arrangement unless specifically asked to, especially if it suggests to let some spectrum lie fallow. Our preference is to provide a technical feasibility assessment for options B1 and B2 and possibly any variants thereof that we may study. |
| Ericsson | Issue 3-7-1: option 2 and re-use band 71/n71 eco-system. For B2 the operator can allow legacy n71 UEs in the network by using multiple FBI, this is not possible for B1 since its duplex spacing is different from that of n71. |
| ZTE | Issue 3-7-1: option 2. |
| Huawei | Issue 3-7-1: It is proposed to capture pros and cons of B1 as well as B2 in the TR (other options are subject to 3-6-1). For the preferred option: option 1 It is suggested to continue the technical discussion and to defer issue 3-7-1 to the final meeting of the SI – no need to have such decision this meeting. |
| Spark | Once the study of B1 and B2 is completed and their pros and cons documented, we should send a LS to AWG but the preferred arrangement of B1 or B2 may be decided by APT administrations based on our technical advice. |
| Xiaomi | Issue 3-7-1: Preferred frequency arrangement Option1: B1 |
| Apple | Issue 3-7-1: Preferred frequency arrangement According to the SID, the objective of the study is to “Answer the request from AWG regarding the technical feasibility of option B1 and B2, respectively. Further options are not precluded and may be included in LS to AWG.” Given the introduction of option B2a this meeting, our understanding is that this study shall provide the technical feasibility of all three options as a response to AWG. We don’t see a specific objective to define a single preferred frequency arrangement in the SID. Of course, our preferred arrangement is B2a, as proposed in our contribution. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Tentative agreements: Issue 3-1-1: full-band and split duplexer (discuss passband sizes/gaps and capture in a WF) Issue 3-1-2: depends on 3-1-1, discuss for each filter configuration (next meeting) Issue 3-1-3: see 3-1-2 Issue 3-1-4: see 3-1-2. Candidate options: Recommendations for 2nd round: further discussions, capture agreed UE filter configurations for study of B1 in a WF “Frequency arrangements for further study and corresponding UE filter configurations”. |
| Sub-topic#3-2 | Tentative agreements: Issue 3-2-1: full-band [possibly] and split duplexer (discuss passband size/gaps and capture in a WF) Issue 3-2-2: depends on 3-1-1, discuss for each filter configuration (next meeting) Issue 3-2-3: see 3-2-2 Issue 3-2-4: see 3-2-2. Candidate options: Recommendations for 2nd round: further discussions, capture agreed UE filter configurations for study of B2 in a WF “Frequency arrangements for further study and corresponding UE filter configurations”. |
| Sub-topic#3-3 | Tentative agreements: Study B2a as proposed in R4-2104891 in addition to B1 and B2. Candidate options: Recommendations for 2nd round: capture the agreement in a WF “Frequency arrangements for further study and corresponding UE filter configurations”. |
| Sub-topic#3-4 | Tentative agreements: No conclusions. Discuss this for each frequency arrangement and UE filter configuration. Candidate options: Recommendations for 2nd round: no further discussions |
| Sub-topic#3-5 | Tentative agreements: For each frequency arrangement, capture in the TR possibilities for avoiding fragmentation of the ecosystem around the 600MHz spectrum (e.g. leverage of B71/n71) Candidate options: Recommendations for 2nd round: no further discussions. |
| Sub-topic#3-6 | Tentative agreements: Option 1 (based on the poll in 3-7-1). RAN4 does not recommend in the TR and the AWG reply any specific frequency arrangement amongst those studied. Candidate options: Recommendations for 2nd round: capture this agreement in a WF “Frequency arrangements for further study and corresponding UE filter configurations”. |
| Sub-topic#3-7 | Tentative agreements: No consensus (no majority view) Candidate options: Recommendations for 2nd round: no discussions. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

Further discussion on sub-topics 3-1 and 3-2 (except requirements). Capture agreed filter configuration for B1 and B2 in a WF ““Frequency arrangements for further study and corresponding UE filter configurations”. Filter configuration for B2a as discussed in R4-2104891.

### Sub-topic 3-1	Filter configuration and requirements for B1

Issue 3-1-1: Filter configuration for B1

- Proposals

  - Option 1: Single 2 x 40 MHz duplexer

  - Option 2: Split duplexer, at least one of the duplexers with 35 MHz passband

  - Option 3: Split duplexer, other passband sizes

- Recommended WF

  - TBA

### Sub-topic 3-2  Filter configuration and requirements for B2

Issue 3-2-1: Filter configuration for B2

- Proposals

  - Option 1: Consider single 2 x 40 MHz duplexer (any feasible technology if applicable)

  - Option 2: Split duplexer, at least one of the duplexers with 35 MHz passband

  - Option 3: Split duplexer, other passband sizes

## Companies views’ collection for 2nd round

### Open issues

| Company | Comments |
| --- | --- |
| Spark | Sub topic 3-3 Option B2a is band n71 plus band nX. We have been requested by AWG to study B1 and B2. They have said that the two band approach is not the current priority: We should focus on the technical feasibility of options B1 and B2 first and depending on this look at B2a if required. Sub topic 3-5 It wasn’t clear in the Moderator discussion summary what is meant by “ leverage band 71/n71 for the extended band” We take it that this mean re using band 71 duplexer in option B2 ( two duplexers in the same band) as this could also allow band 71 support …. Others: |
| Ericsson | Issue 3-1-1: option1 , that’s AWG’s request. Issue 3-2-1: Option 2, aligned with AWG’s request and will enable re-use of band 71 UE eco-system. The upper filter should not be too narrow, the maximum channel BW should not depend on the carrier frequency in the band |
| Skyworks | Although the way forward leaves many options open for implementation, the eband n71 ecosystem reuse claim should mean that the new filter arrangement should be reusable for n71 with negligible impact to band n71 performance. With our current feasibility study based on real filters measurement that are scaled to the desired frequency range, we see only this possible with the B2 split duplexer approach. Even there to minimize the additional cost vs n71, the second duplexer could use a smaller BW (25MHz). unless we have further requirements which can drive a redesign it is difficult to see how we could state feasibility of the solutions any further. We hope there can be clear criteria for the filter design elaborated for next meeting. |

### WF comments collection

Comments on the WF on ‘Frequency arrangements for further study and corresponding UE filter configurations’ on the reflector or below

| CR/TP number | Comments collection |
| --- | --- |
| R4-2105422 | Company A |
|  | Company B |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on “Coexistence with Other Services” | Moderator (Ericsson) | See details in 2.4.1 |
| WF on “Frequency arrangements for further study and corresponding UE filter configurations” | Qualcomm | See details in 3.4.1 |
| New version of the TR (baseline for next meeting) | Rapporteur | The TR was not updated with the TP in R4-2103267 agreed at RAN4#98-e. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2104495 | Option B2 for Extended 600MHz NR band | Spark NZ Ltd. | Noted |  |
| R4-2104717 | Coexistence for APT 600 MHz | Nokia, Nokia Shanghai Bell, CBN | Noted |  |
| R4-2104718 | Frequency arrangements for APT 600 MHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2104931 | Coexistence study for extended 600 MHz NR frequency band | ZTE Corporation | Noted |  |
| R4-2104817 | Discussion on Extended 600MHz Band Implementation Options | Skyworks Solutions Inc. | Noted |  |
| R4-2104891 | TP on band plan for 600 MHz | Apple | Noted |  |
| R4-2105094 | Further discussion on frequency arrangement for extended 600MHz NR Band | Xiaomi | Noted |  |
| R4-2105105 | [LS on] FREQUENCY ARRANGEMENTS FOR IMT IN THE BAND 470 – 703 MHZ | APT/AWG-27 | Noted |  |
| R4-2106593 | Discussions on Option B1 and B2 for extended 600MHz | ZTE Corporation | Noted |  |
| R4-2106891 | APT 600 MHz band – frequency arrangements | Ericsson | Noted |  |
| R4-2107301 | Feasibility analysis of 600MHz duplexer | Huawei, HiSilicon | Noted |  |
| R4-2107348 | Filtering for extended 600 MHz band | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2105421 | WF on Coexistence with Other Services | Ericsson (moderator) | Agreeable |  |
| R4-2105422 | WF on “Frequency arrangements for further study and corresponding UE filter configurations” | Qualcomm Incorporated | Return to | GTW Apr 20th |
| R4-2105423 | TR 38.xxx v 0.2.0, “APT 600 MHz Band for NR” | Rapporteur (Spark NZ) | (e-mail approval) | New version of the TR (was not updated with the TP in R4-2103267 agreed at RAN4#98-e) |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents