3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2105838

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			12.1

Source:			Samsung

Title:	WF on FeMIMO L1/L2-Centric Inter-Cell Mobility

Document for:		Approval

0	Introduction

This contribution contains the WF for [228] Rel-17 FeMIMO L1/L2-Centric Inter-Cell Mobility issues in RAN4#98-bis-e w.r.t. RAN4 understandings for further study and how to reply the RAN1 LS R1-2102248.

Way Forward on Rel-17 FeMIMO L1/L2-Centric Mobility

RAN4 understanding on FeMIMO L1/L2-Centric inter-cell mobility

Operations in L1/L2-Centric inter-cell mobility that RAN4 needs to evaluate

Based on RAN1/RAN2 agreement, RAN4 may need to consider and evaluate the impact of at least the followings from RAN4 perspective

- Measurement on NSC and reporting of the measurement; and

- Operations for indicating a TCI state referring to RSs on the NSC; and

- FFS: Transmission schemes for enabling inter-cell multi-TRP operations

  - Note: Transmission schemes are discussed in the following items.

Identified impacts on RAN4 for further discussion

For L1/L2-Centric inter-cell mobility, RAN4 shall discuss on the impacts with regard to following issues:

- UE L1-measurement on NSC for beam measurements and reporting;

  - Whether to enhance UE capability due to L1 measurements on NSC

- Requirement for TCI state switching considering TCI related operations pertained to RS(s) on NSC;

- UE BM capability and timing requirement for

  - UE simultaneous“reception and transmission”or simultaneous reception only from multiple SC, after L1/L2-Centric inter-cell mobility operation , if RAN1 supports this feature;

  - UE CA scenario, if RAN1 considers the case that UE is configured with multi CCs;

- Other issues are not precluded.

RAN4 assumptions for L1/L2-Centric inter-cell mobility scenarios

RAN4 considers the feature of L1/L2-Centric inter-cell mobility under the assumptions that:

- Focus on both FR2 and FR1

  - Both SC and NSC are in the same FR;

- Study on non-collocated and collocated SC and NSC case

- SC and NSC are

  - in the same frequency layer

    - The SSBs of NSC have the same center frequency and SCS as the SSBs of the serving cell (RAN1 agreement)

  - not in the same frequency layer, if supported by RAN1

    - CA scenarios (e.g. intra-band and inter-band), if defined and supported by RAN1

    - Other inter-frequency case, if any supported by RAN1

- Other options are not precluded.

Clarification on L1/L2-Centric inter-cell mobility and transmission schemes for enabling inter-cell multi-TRP operations (For information purpose)

RAN4 needs to first align the understanding of L1/L2-Centric inter-cell mobility and transmission schemes for enabling inter-cell multi-TRP operations with RAN1 before RAN4 further discusses any related issues.

- Note1: In RAN4, some schemes are discussed in this meeting, which may need further alignment. For example, schemes based on different companies’ understanding, such as Simultaneous Transmission case, Multi-TRP case, Regular CA case, and Multi-TRP operation after mobility operation might be considered as intra-band CA scenario indicated by RAN1 in LS R1-2102248. Other options/interpretations are not precluded.

- Note2: RAN4 shall not give preference on any of transmission schemes. RAN4 could only study on the RAN4 impact in the context of possible schemes.

RAN4 Reply to RAN1 LS R1-2102248

What to include in the reply LS to RAN1

Identified RAN4 impacts in the contexts of RAN1 LS Q5 and Q6 that includes but not limited to

- Impacts due to measuring inter-frequency and/or inter-band NSC L1 resources

  - Use of MG

  - Different SMTCs between SC and NSC

  - Reference timing for measuring NSC L1 resources (if SSB based search is needed)

  - Other issues if any

- Impacts due to TCI state switching for NSC

And RAN4 asks RAN1 to clarify, if needed,

- Which of the following(s) and what else are under consideration by RAN1 for enabling inter-cell multi-TRP operations

  - Simultaneous transmission

  - Multi-TRP

  - Regular CA

  - Other options if any