# 3GPP TSG-RAN WG4 Meeting # 98bis-e                                                        R4-2106376

# Electronic Meeting, Apr. 12-Apr. 20, 2021

Source:	ZTE Corporation

Title:	TP for TR 36.717-03-02_CA_1A-3A-20A-38A

Agenda Item:	10.5.3

Document for:	Approval

- Introduction

This contribution provides a text proposal on LTE CA_1A-3A-20A-38A with the UL CA configuration of CA_1A-20A and CA_3A-20A for TR36.717-03-02.

- Reference

[1] TR36.717-03-02, LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17, v0.2.0

- Text Proposal

----- Start of TP -----

  - 7.x			LTE-A inter-band CA: Band 1 and Band 3 and Band 20 and Band 38 with 2 bands UL

    - 7.x.1	List of specific combination issues

      - 7.x.1.1	Channel bandwidths per operating band for CA

Table 7.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-20A-38A | CA_1A-20A CA_3A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 7.x.1.2	Co-existence studies for LTE-A inter-band UL CA_1A-20A and DL CA_1A-3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.x.1.2-1

Table 7.x.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2496 | 2586 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4566 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 | 2000 | 812 | 882 |

Based on Table 7.x.1.2-1:

-There is no harmonics and IMDs products fall into own Rx of Band 3.

-3rd harmonic of band 20 may fall into own Rx of Band 38, and  no IMDs products fall into own Rx of Band 38.

      - 7.x.1.3	Co-existence studies for LTE-A inter-band UL CA_3A-20A and DL CA_1A-3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.x.1.3-1

Table 7.x.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 832 | 862 |
| 2nd order harmonics frequency range (MHz) | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd order harmonics frequency range (MHz) | 3420 | 3570 | 1664 | 1724 |
| 3rd order harmonics frequency range (MHz) | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd order harmonics frequency range (MHz) | 5130 | 5355 | 2496 | 2586 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 953 | 848 | 2542 | 2647 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 | 2738 | 121 | 14 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 | 4432 | 3374 | 3509 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1696 | 1906 | 5084 | 5294 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 | 4523 | 711 | 876 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 | 6217 | 4206 | 4371 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 | 1543 | 6308 | 5978 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 | 5233 | 7672 | 8002 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 834 | 1074 | 3691 | 3406 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6156 | 6794 | 7079 |

Based on Table 7.x.1.3-1:

-There is no harmonics and IMDs products fall into own Rx of Band 1.

-3rd harmonic of band 20 may fall into own Rx of Band 38 and 2rd IMDs products fall into own Rx of Band 38.

      - 7.x.1.4	MSD

According to the co-existence study, some MSD issue needs to be considered. However, for harmonic MSD, it have already been captured in 1UL/2DL, and for IMD MSD, it also have been captured in the corresponding fallback 2UL/3DL configuration, which means there is no need to consider additional MSD requirements for this combination.

      - 7.x.1.5	∆TIB and ∆RIB values

The same requirements of fallback combination can be applied.

----- End of TP -----