3GPP TSG-RAN WG4 Meeting#98-bis-e	 R4-2106583

E-meeting, Apr 12th – Apr 20th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation analysis for HST in FR2

Agenda item:	8.7.4.1

Document for:	Discussion

Introduction

The discussion about the work item for high-speed operation in FR2 has been going on for a couple of meetings [1]. In the last meeting, we provided some initial simulation results related to deployment scenarios and RRM requirements [2]. In this contribution, we provide an extended set of results and share a full list of simulation parameters used in our simulations. We expect that these parameters can be widely used for evaluating the system-level performance of CPE operation in FR2 HST environment.

In the contribution, we show results for:

- Handover rate

- Ping pong handover rate

- Beam switch rate

- Inter-cell mobility failure rate (RLF + HOF)

- Beam failure indication rate

- Time-of-stay in cell

- Time-of-outage due to low SINR

- Time-of-outage percentage per call (including handovers)

- SINR distributions

Simulation assumptions

We have performed fully dynamic system-level simulations with train speed 350 km/h in both uni- and bi-directional Scenarios A and B using non-SFN and non-DPS deployment. We assume each BBU has only one RRH creating a challenging mobility scenario to test the requirements. In addition to updating D_min and D_s values according to the latest agreements, we have added simulation settings for DRX with long cycles 40 and 80 ms compared to our previous paper [2]. We have also modified simulation parameters for DRX disabled case to comply with FR2 minimum requirements [3] for measurement periods, cell detection delay, and RLM/BFD monitoring assumptions. Table 1 has a detailed list of system simulation parameters.

Table 1: Detailed system-level parameter settings

| Simulation parameter | Value |
| --- | --- |
| Number of sites (separate gNBs) | 8 |
| Inter-site distance (ISD, D_s) | 700 m |
| RRH distance to track (D_min) | 10, 150 m (Scenario A, B) |
| RRH height (D_RRH_Height) | 15 m |
| CPE height (D_CPE_Height) | 5 m |
| Carrier frequency | 28 GHz |
| Bandwidth | 50 MHz |
| Subcarrier spacing | 120 KHz |
| Propagation and channel model | TR 38.901 RMa with LOS only [4] |
| RRH antenna panel | [Mg, Ng, M, N, P] = [1, 1, 8, 8, 2] In uni-directional case panel is pointing towards the track at the x-axis where the next site is situated (ISD away) In bi-directional case panel is pointing towards the track at the x-axis at ISD/2 away |
| SSB beams per RRH | 1 beam: Pointing into the boresight of the RRH antenna panel 2 beams: One beam is pointing into the boresight and the other beam is pointing 20 degrees towards the track from boresight 4 beams: One beam is pointing into the boresight and the other beams are pointing 20, 40, 60 degrees towards the track from boresight |
| Train speed | 350 km/h |
| CPE antenna panel | [Mg, Ng, M, N, P] = [1, 1 or 2, 4, 4, 2] In uni-directional case where RRHs point east CPE has one antenna panel pointing west In bi-directional case CPE has two antenna panels pointing to 180 degrees opposite directions (west-east) MPUE assumption: only one panel can be used at a time for measurements |
| Traffic | DL Full Buffer |
| Inter-cell interference | Only one train with one CPE is simulated meaning there is no inter-cell interference |
| DRX | DRX disabled (DRX 0), 40, 80 ms cycles |
| Handover assumptions | Event A3 with SS-RSRP Offset: 3 dB Time-to-trigger: 80 ms |
| RRC measurement period L1 RSRP measurement period | DRX 0: 480 ms DRX 40: 1440 ms DRX 80: 2880 ms |
| Cell detection delay | DRX 0: 600 ms DRX 40: 1440 ms DRX 80: 2880 ms |
| RLM assumptions | TEvaluate_out_CSI-RS: 600, 3600, 7200 ms (DRX 0, 40, 80) TEvaluate_in_CSI-RS: 300, 1800, 3600 ms (DRX 0, 40, 80) N310: 2 samples N311: 2 samples Qout threshold SINR: -8 dB Qin threshold SINR: -6 dB |
| BFD assumptions | TEvaluate_BFD_CSI-RS: 300, 1800, 3600 ms (DRX 0, 40, 80) |
| Simulation length | 60 seconds (80 drops of 60 seconds simulated, and statistics samples are gathered from all drops) |

RAN4 to consider using parameters listed above as a reference for system-level evaluations in HST FR2.

Simulation results and observations

Figure 1 shows the successful handover rate normalized per CPE per second. We observe that in bi-directional deployment, handover rates are typically over twice the rates in uni-directional scenario. This is caused by double the number of cells in the scenario and more potential for ping-pong handovers, as seen in Figure 2, when there are more overlapping areas between coverage areas of the cells. Another observation is that adding DRX to be used decreases handover rate, particularly in bi-directional deployment. This is caused by significantly longer measurement filtering as seen in Table 1. The effect of Scenario A vs. B on handover rates is smaller than DRX settings or RRH direction settings. Ping-pongs are almost absent when DRX cycle is increased to 80 ms. A ping-pong is observed when there are two handovers back-and-forth between same cells during one second.

Figure 1 Handover rate                                                 Figure 2 Ping-pong handover rate

Figure 3 shows successful beam switch rate normalized per CPE per second. We have simulated Scenario A only with one beam per RRH so beam switches are obviously missing in that scenario as well as with option for Scenario B setting one beam per RRH. In the cases with 2-4 beams per RRH we see that beam switches are much more common in bi-directional scenario than uni-directional. Also, longer L1 RSRP filtering used with DRX caused beam switch rate to drop significantly.

Figure 3 Beam switch rate

Figure 4 and Figure 5 show inter-cell mobility failure rates ((RLF+HOF)/(RLF+HOF+HO) * 100%) and beam failure indications rates respectively. We observe that generally both of the failure rates are very low in the simulated scenarios as inter-cell failure rates stays under 1% in all cases and beam failure indications (not even leading to failures) are happening in the maximum rate of 1 per 200 seconds per CPE. Without DRX effects the failure rates are even lower than these and particularly the failures are absent in uni-directional case. It must be noted from these results that the failure events are so rare that detailed comparison between different cases would require very high number of simulation samples. Nevertheless, the current results can be used to observe generally low level of failures or indications in scenarios without inter-cell interference.

Figure 4 Inter-cell mobility failure rate (RLF + HOF)                     Figure 5 Beam failure indication rate

Figure 6 shows average time-of-stay (ToS) in a cell. As expected, ToS follows the inverse pattern compared to handover rate so uni-directional deployment has much longer ToS than bi-directional deployment. Also, DRX increases the average ToS particularly in bi-directional deployment.

Figure 6 Time-of-stay in cell

Time-of-outage (ToO) is an important metric to analyze the mobility performance because it also considers periods of low signal quality (SINR) that are observed prior to detection radio link problem or beam failure indication. These specified detections can be delayed by using longer filtering based on minimum requirements with DRX particularly in FR2, but ToO metrics capture those situations of low SINR as well. Figure 7 shows how long ToO periods are caused by low SINR of below -8 dB at 95-percentile of CDF. We observe that ToO caused by low SINR happens at significant rate only in bi-directional scenarios and particularly in Scenario A. In Scenario B having more beams than 1 improves the coverage at site locations towards the track in a way that the longest periods of ToO due to low SINR diminish. However, in all cases the 95-percentile of CDF of ToO length is below 100 ms. These rather short periods of low SINR can lead into not observing many mobility failures as seen in Figure 4 and Figure 5 although DRX increases the length of ToO condition. Figure 8 shows the total ToO percentage per call, which includes on top of low SINR periods also handover execution time, which is set to 40 ms per each handover in these simulations. We see larger percentage of calls in ToO in bi-directional deployment compared to uni-directional mainly due to larger number of handovers, but also due to the longer low SINR periods. DRX does not have significant effect on total ToO, because while low SINR period can be longer, the number of handovers drops with DRX.

Figure 7 Time-of-outage due to low SINR       Figure 8 Time-of-outage percentage per call (including handovers)

Figure 9 group shows SINR distributions for Scenario A with 1 beam per RRH and Scenario B with 1, 2 and 4 beams per RRH. We observe that the delays in measurements caused by DRX can lead into lower SINR level particularly in bi-directional deployment. However, as we have seen from results presented earlier in this paper, the SINR is typically still good enough for not triggering mobility failures and even in worst situations there is usually a fast recovery.

Figure 9 SINR distributions

When using current requirements for FR2, no significant mobility performance degradation can be observed in our simulations when DRX is disabled, train speed is up to 350 km/h and there is no inter-cell interference in the frequency band.

Although DRX with 40-80 ms long cycles can cause additional delays to mobility based on minimum requirements, the mobility failure rates stay low.

Conclusion

In this paper we provided system-level mobility performance results for NR HST in FR2. Based on the simulation results we observe the following:

- When using current requirements for FR2, no significant mobility performance degradation can be observed in our simulations when DRX is disabled, train speed is up to 350 km/h and there is no inter-cell interference in the frequency band.

- Although DRX with 40-80 ms long cycles can cause additional delays to mobility based on minimum requirements, the mobility failure rates stay low.

- References

[1] R4-2103679, WF on Rel-17 NR HST FR2 enhancements, RRM requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting #98-e, Electronic Meeting, Jan. 25th – Feb. 5th, 2021.

[2] R4-2102093, Simulation results for HST in FR2, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting#98-e, Electronic Meeting, Jan. 25th – Feb. 5th, 2021.

[3] 3GPP TS 38.133 V17.0.0, NR Requirements for support of radio resource management (Release 17)

[4] 3GPP TR 38.901 V16.1.0, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)