3GPP TSG RAN WG4 Meeting #98-bis-e	R4-2106625

Electronic Meeting, April 12 – 20, 2021

Agenda item:			5.5.1.2

Source:	vivo

Title:	Discussion on PRS RSRP measurement requirements

Document for:	Discussion

- Introduction

In the RAN4#98-e meeting, the measurement period requirements for PRS-RSRP when configured with RSTD or UE Rx-Tx were discussed. It was agreed if UE is configured with PRS-RSRP measurement for different positioning methods, then measurement period requirements should be met independently for different measurements.

- Scenario #2: UE being configured to measure PRS-RSRP for different positioning methods - If PRS-RSRP measurement is separately configured for each positioning method, the requirements shall be met independently for each positioning method.

However, if PRS-RSRP is configured additionaly for RSTD or UE Rx-Tx measurements, then measurement period for PRS-RSRP measurement for AoD needs further study.

- FFS Scenario #1: UE being configured to measure PRS-RSRP for DL-TDOA (or Multi-RTT) - UE behavior: - Option 1 - UE continues PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when - PRS-RSRP is configured together with RSTD/UE Rx-Tx, and - the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (without PRS-RSRP) - Option 2 - UE does not continue PRS-RSRP measurement - Applicable PRS-RSRP measurement period requirements - Option 1: - The minimum measurement period is extended to that for RSTD/UE Rx-Tx - Option 2: - The minimum measurement period is linked with the accuracy and thus does not need to be extended, but the UE shall continue measuring so the PRS-RSRP matches RSTD/UE Rx-Tx - Option 3 - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT, the UE stops PRS-RSRP measurement

In this contribution, we provide our views on PRS-RSRP measurement period for Scenario #1.

# Discussion

There is also open issue for RSTD measurement for Scenario #1 that PRS-RSRP is configured for DL-TDOA but not other positioning methods.

For scenario #1, the PRS-RSRP could be measured on the same PRS resources as for RSTD measurement at the same time. From measurement period requirements perspective PRS-RSRP measurement requirements would not require longer measurement period than RSTD measurement as both are based on 4 samples and vice versa. So RSTD measurement period shall not be impacted by PRS-RSRP measurement. Correspondingly PRS-RSRP measurement period shall not be impacted RSTD measurement.

For PRS-RRSP measurement and RSTD/UE Rx-Tx measurement, accuracy requirements should also be met. The accuracy requirements are defined by assuming 4 samples is used. So, in general RSTD/UE Rx-Tx measurement and PRS-RSRP measurement are supposed to be measured during the same measurement period from requirements perspective. How UE would actually meet the requirements is up to UE implementation. UE behaviour related to UE implementation would not be specified. Even if it is specified, it cannot be verified as long as UE can meet the requirements.

Proposal 1: UE bahviour is not defined when PRS-RSRP is configured additionally to RSTD or UE Rx-Tx measurement.

Proposal 2: Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT.

# Summary

In this contribution we provide our views on PRS-RSRP measurement period for Scenario #1. Based on analysis following proposals are present.

Proposal 1: UE bahviour is not defined when PRS-RSRP is configured additionally to RSTD or UE Rx-Tx measurement.

Proposal 2: Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT.

- References

- R4-2104076	WF on UE PRS measurement requirements, Huawei, HiSilicon