3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106801

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation results on NR V2X power imbalance test

Source: 	Huawei, HiSilicon

Agenda item:	5.2.1.3.1

Document for:	Discussion

Background

At last meeting, simulation assumptions and test design for V2X power imbalance were discussed. In this paper, we provide our simulation results based on simulation assumptions derived from [1]:

- Methodology for power imbalance test: - The target requirement ICS = -27 dBc - Select SINR2 from simulation results for SNR@10% BLER point based on simulation assumptions in Table 2.1-2. - Select SNR2 such that SNR2 = SINR2+5dB - Compute SNR1 = 30.35dB from the relation: SINR2 = SNR2 – 10*log10(10^((SNR1 + ICS)/10)+1).

Simulation results

According to the methodology agreed at the last meeting, we provide our simulation results for SINR2 in Figure 2-1 and Table 2-1. The corresponding simulation assumptions and FRC are shown in the appendix.

Figure 2-1 Simulation results for power imbalance test.

Table 2-1: Simulation results for NR V2X power imbalance test

| Test metric | Ideal results | Impairment results |
| --- | --- | --- |
| SINR2 (SNR @ 10% BLER based on Table5-2 ) | -2.53 | -1.03 |
| SNR2(SINR2+5dB) | 2.47 | 3.97 |

Conclusion

In this contribution, we provide our simulation results for V2X power imbalance test

Reference

[1]   R4-2103993 Updated simulation assumptions for NR V2X multiple link test cases, Huawei, HiSilicon

Appendix

Table 5-1: Test assumptions

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | PRB index: 0~9 |
|  | PSSCH RMC |  | Refer to Table 2.1-2 |
|  | PSCCH allocation |  | 10 RBs and 3 OFDM symbols (including first symbol used for AGC) |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 Low |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | PRB index: 30~39 |
|  | PSSCH RMC |  | Refer to Table 2.1-2 |
|  | PSCCH RB allocation |  | 10 RBs and 3 OFDM symbols (including first symbol used for AGC) |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 Low |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |

Table 5-2: Simulation assumptions and FRC for derivation of SINR2

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Synchronization source |  |  | GNSS |
| Channel bandwidth |  | MHz | 20 |
| Allocated resource blocks |  | RB | 10 |
| Subcarrier spacing |  | kHz | 30 |
| CP-OFDM symbols for slot with PSFCH (Note 2) |  |  | 10 |
| CP-OFDM symbols for slot without PSFCH (Note3) |  |  | 13 |
| DMRS symbols for slot with PSFCH |  |  | 2 |
| DMRS symbols for slot without PSFCH |  |  | 3 |
| Modulation order |  |  | 2 |
| MCS index |  |  | 4 (308/1024) |
| SCI format 2-A configuration | Payloads | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 3.5 |
| Transport Block Size for slot with PSFCH |  | Bits | 208 |
| Transport Block Size for slot without PSFCH |  | Bits | 432 |
| Transport block CRC |  | Bits | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 744 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 1464 |
| PSFCH resource periodicity |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 2 |
| Performance metric |  |  | SNR@10% BLER of PSSCH |
| Note 1: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. The first OFDM symbol of a PSFCH is duplicated as described in clause 8.3.4.2.2 of TS 38.211 Note 2: Guard OFDM symbols as described in clause 8.2.1 of TS 38.211 and OFDM symbols used for PSFCH are not included. Note 3: Last OFDM symbol used for guard symbol as described in clause 8.2.1 of TS 38.211 is not included. Note 4: 10 RBs and 3 OFDM symbols including first symbol for AGC are used for PSCCH transmission. |  |  |  |