3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106803

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Discussions and simulation results on NR V2X soft buffer test

Source: 	Huawei, HiSilicon

Agenda item:	5.2.1.3.2

Document for:	Discussion

Background

At last meeting, simulation assumptions and test design for NR V2X soft buffer test were determined but some issues are still open [1]. In this paper, we provide our simulation results and discussions.

Simulation results

We give our simulation results in Figure 2-1 and Table 2-1 and corresponding simulation assumptions are shown in appendix.

Figure 2-1: Simulation results for soft buffer test with n64

Table 2-1: Summary of simulation results for soft buffer test with n64

| Test metric | Ideal requirement | Impairment requirement |
| --- | --- | --- |
| SNR(dB)@5%BLER with 0.1PPM CFO and 12Ts CTO | 9.10 | 10.60 |
| SNR(dB)@5%BLER without CFO and CTO | 8.17 | 9.67 |

Discussions

From simulation results in Table 2-1, 0.93 dB performance gap caused by ICI can be observed between the tests with/without 0.1PPM CFO and 12Ts CTO

Observation 1: 0.93 dB performance gap caused by ICI can be observed between the test with/without 0.1PPM CFO and 12Ts CTO.

From our understanding, RAN4’s intention is to define the minimum requirements for every case and from our simulation results it can be see that ICI has great influence on this test. Therefore we prefer to define the requirements for the worst case. i.e. 0.1PPM CFO and 12Ts CTO at RX side. What’s more, according to the WF [2] in RAN4#96-e meeting, RAN 4 has agreed to consider 0.1PPM CFO and 12Ts CTO at RX side for simulation alignment.

Proposal 1: Define requirements for soft buffer test considering 0.1PPM CFO and 12Ts CTO at RX side.

Conclusion

In this contribution, we provide our simulation results and discussions for V2X soft buffer test. The proposals and observations are:

Observation 1: 0.93 dB performance gap caused by ICI can be observed between the test with/without 0.1PPM CFO and 12Ts CTO.

Proposal 1: Define requirements for soft buffer test considering 0.1PPM CFO and 12Ts CTO at RX side.

Reference

[1]   R4-2103993 Updated simulation assumptions for NR V2X multiple link test cases. Huawei, HiSilicon

[2]   R4-2012758 WF on work scope and general assumptions for NR V2X demodulation performance, LG Electronics

Appendix

- According to the agreement of RAN4#98-e meeting, RAN4 will only define one set of performance requirements based on the results of harq-RxProcessSidelink = n64. Interested companies are encouraged to provide simulation results in next meeting based on the simulation assumptions given in Table 2.2-1 and Table 2.2-2.

Table 2.2-1： Simulation assumptions for soft buffer test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Bandwidth |  | MHz | 20 |
| SCS |  | kHz | 30 |
| Subchannel size |  | RBs | 10 |
| Number of symbols for PSCCH |  | Symbol | 2 |
| Number of RBs for PSCCH |  | RBs | 10 |
| Active Sidelink UE(s) |  |  | 64 |
| Sidelink UE i, 0 ≤ i ≤ 63 | Sidelink Transmissions |  | First 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit PSCCH + PSSCH in the same slot as the first two UEs but in the third subchannel. |
|  | Time gap between initial transmission and retransmission | Slots | Min(31, N) (Note 5) |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| Timing offset from receiving side (Note 3) |  | s | Option 1: 0 Option 2: 12Ts |
| Frequency offset from receiving side (Note 4) |  | Hz | Option 1: 0 Option 2: 600 |
| Propagation condition |  |  | AWGN |
| Test metric |  |  | SNR@5% BLER of PSSCH |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Timing offset from receiving side is with respect to GNSS reference timing. Note 4: Frequency offset from receiving side is with respect to GNSS reference frequency. Note 5: N is equal to harq-RxProcessSidelink. |  |  |  |

Table 2.2-2： FRC for PSSCH

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Test |  |  | SCH_Test1 |
| Synchronization source |  |  | GNSS |
| Channel bandwidth |  | MHz | 20 |
| Allocated resource blocks |  | RB | 10 |
| Subcarrier spacing |  | kHz | 30 |
| CP-OFDM symbols (Note 1) |  |  | 10 |
| DMRS symbols |  |  | 2 |
| MCS index |  |  | MCS 28 |
| SCI format 2-A configuration | Payloads | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 2.5 |
| Transport Block Size for slots |  | Bits | 6528 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits for slots |  | Bits | 2544 |
| PSFCH resource periodicity |  | Slot | 1 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: OFDM symbols are for PSCCH/PSSCH transmission not including guard symbols and PSFCH symbols. Note 2: 10 RBs and 3 OFDM symbols including first symbol for AGC are used for PSCCH transmission. |  |  |  |