3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106999

Electronic Meeting, 12 – 20 April, 2021

Title: 	Discussion on remaining issues for PRS-RSRP measurement requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.5.1.2

Document for:	Discussion

Introduction

RRM requirements for PRS-RSRP measurements have been discussed in RAN4#98-e, and the conclusions are captured in the WF [1]. Based on our understanding, there is still some remaining open issues:

- Measurement period when configured for different positioning methods

In this paper we will provide our views on the remaining open issues on PRS-RSRP measurement requirements.

Discussion

- Measurement period when configured with RSTD or UE Rx-Tx - FFS Scenario #1: UE being configured to measure PRS-RSRP for DL-TDOA (or Multi-RTT) - UE behavior: - Option 1 - UE continues PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when - PRS-RSRP is configured together with RSTD/UE Rx-Tx, and - the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (without PRS-RSRP) - Option 2 - UE does not continue PRS-RSRP measurement - Applicable PRS-RSRP measurement period requirements - Option 1: - The minimum measurement period is extended to that for RSTD/UE Rx-Tx - Option 2: - The minimum measurement period is linked with the accuracy and thus does not need to be extended, but the UE shall continue measuring so the PRS-RSRP matches RSTD/UE Rx-Tx - Option 3 - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT, the UE stops PRS-RSRP measurement

When UE is configured with PRS-RSRP measurement for DL-TDOA, the PRS-RSRP is measured on the same set of PRS resources as RSTD. Based on the current requirements the measurement period for RSTD and PRS-RSRP are same.

  - For UE behavior: neither option 1 nor option 2 applies, as UE would complete the two measurements at the same time.

  - For applicable requirements: the current requirements in clause 9.9.3 apply

Proposal: Requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT.

Conclusions

In this paper we provided our views on remaining issues in PRS-RSRP measurement requirements.

Proposal: Requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT.

Reference

- R4-2104076, WF on UE PRS measurement requirements, Huawei, HiSilicon