3GPP TSG-RAN WG4 #98-bis-e	R4- 2107076

Electronic meeting, 12 April – 20 April 2021

Source: 	Ericsson

Title:  	Band plan for NR operation for 52.6-71GHz

Agenda Item:	8.12.2

Document for:	Discussion

# Discussion

According to WID on Extending current NR operation to 71GHz [1], one of the objectives of the study in RAN4 is to specify new band(s) for the frequency range 52.6-71GHz.

A detailed and thorough global spectrum situation analysis for this range has been captured in TR 38.807 [2]. Table 4.2.1-1 in [2] provides an overall summary of the current licensing situation for 52.6GHz-71GHz range for various countries/regions. Symbol 'U' represents unlicensed spectrum.

Table 4.2.1-1: Current Licensing situation for various countries/regions between 52.6GHz and 71GHz

| Region | Country/Region | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-71 |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 2 | USA |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U (Mobile) |  |  |  |  |  |  |

It should be note that local licensing is also allowed in Europe/CEPT in 57-71 GHz range.

In 2015, the international telecommunication union (ITU) proposed 66–71 GHz spectrum range as one of the candidate bands for possible identification for IMT in WRC-19.  The WRC-19 concluded that the frequency range should have an IMT identification to enable 5G deployments. The IMT allocation in turn will facilitate the global harmonization and licensed usage.

From the discussions above it can be concluded that there are demands and possibilities for both licensed and unlicenced usage in this range consequently we propose to specify the following bands considering other ranges for licensed usage are not precluded at this stage:

- 57 – 71 GHz for unlicensed usage

- 66 – 71 GHz for licensed usage

# Proposals

Based on the discussion in clause 1 we propose the following bands considering other ranges for licensed usage are not precluded at this stage:

Proposal 1: Specify 57 – 71 GHz band for unlicensed usage

Proposal 2: Specify 66 – 71 GHz band for licensed usage

# References

RP-202925 “Revised WID on Extending current NR operation to 71GHz”, CMCC

TR 38.807, “	Study on requirements for NR beyond 52.6 GHz”, Rel-16