3GPP TSG-RAN WG4 Meeting #98-bis-e		                                 R4-2107270

Electronic Meeting, 12 April – 20 April 2021

# Source:	Thales

# Title:	On the S-band NTN coexistence scenarios and simulation parameters

# Type:	discussion

# Document for:	discussion

# Agenda Item:	8.8.2. Coexistence aspects

Release	Rel-17

# Introduction

In RAN4#98e in has been agreed:

- Include S-band, L-band as exemplary bands for FR1

  - Using S-band frequency range i.e. 2GHz for co-existence simulation in FR1

- At least one of above bands RF requirements completed, then Rel-17 NTN WI, RF requirements for FR1 can be considered as completed.

- Baseline/Starting point of simulation assumptions.

  - Use Satellite and UE parameters as well as network deployment assumptions in TR 38.821 as the starting point for FR1 coexistence study.

  - ITU-R studies would also be used as inputs.

- NTN UE parameters

  - Handheld UEs for FR1.

- TN UE parameters

  - The existing RF requirements (i.e. ACS and ACLR for both BS and UE) of TN in the spec (i.e. TS 38.104 and 38.101) shall be reused when doing the coexistence study between NTN and TN.

Moreover, it has been agreed (RAN4#98e agreement):

TN-NTN coexistence scenarios in adjacent bands

| No. | Frq. | TN | TN scenario | NTN |
| --- | --- | --- | --- | --- |
| 1 | 2GHz | NR | Rural | GEO |
| 2 | 2GHz | NR | Rural | LEO 600km |
| 3 | 2GHz | NR | Rural | LEO 1200km |
| 4 | 2GHz | NR | Urban macro | GEO |
| 5 | 2GHz | NR | Urban macro | LEO 600km |
| 6 | 2GHz | NR | Urban macro | LEO 1200km |
| 7 | 2GHz | NR | Rural | HAPS |
| 8 | 2GHz | NR | Urban macro | HAPS |

NTN-NTN coexistence scenarios in adjacent bands

| No. | Frq. | NTN | NTN |
| --- | --- | --- | --- |
| 1 | 2GHz | GEO | GEO |
| 2 | 2GHz | GEO | LEO 600km |
| 3 | 2GHz | LEO 600km | LEO 600km |
| 4 | 2GHz | LEO 1200km | LEO 1200km |
| 5 | 2GHz | HAPS | HAPS |

RAN4 further agreed to take following scenarios for initial simulation alignment purpose in Q2 2021:

- TN deployment: NR only with Rural, Urban Macro

- NTN deployment: GEO, LEO-600, LEO-1200, HAPS

- Satellite Set: Set1

The goal of this contribution is therefore to further clarify coexistence scenarios and simulation parameters for S-band in FR1.

# S-band coexistence scenarios

R4-2103998 (revised from R4-2103965) on FR1 simulation assumption for NTN co-existence study (RAN4#98e) shows in Table 2.1-2 the following aggressor and victim combination list:

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL |  |
| 2 | TN with NTN | TN UL | NTN UL |  |
| 3 | TN with NTN | NTN DL | TN DL |  |
| 4 | TN with NTN | NTN UL | TN UL |  |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. |
| 7, 8 | NTN with NTN | NTN DL | NTN DL | LEO-LEO or GEO-GEO |
|  |  | NTN UL | NTN UL | LEO-LEO or GEO-GEO |

As already explained in contribution R4-2015915, and showed on Figure 1, the identified MSS band (1980-2010 MHz and 2170-2200 MHz) are already part of 3GPP band 65 and band n65. According to TR 38.819, the 3GPP bands nearby to band n65, when addressing co-existence aspects are bands 1 and 34 as shown in Figure 2. Band 1 is allocated to MS in both Region 1 and 3, while Band 34 is allocated in Region 3.

Figure 1: 3GPP bands nearby the S band allocated to Mobile Satellite Services

As an exemplary band for RAN4 KPI evaluation, we suggest considering the MSS band which is the upper band part of the band n65 e.g. 1980-2010 MHz for UL (which is a common part for all regions) and 2170-2200 MHz for DL (which is a common part for all regions). Note that 2000-2020 MHz for UL and 2180-2200 MHz bands for DL are allocated by FCC for MS and MSS use in US (and Canada).

In Figure 2 below, is represented a more complete set of NTN MSS S-band potential coexistence scenarios with TN in adjacent bands. Some of coexistence scenarios could be grouped together and RAN4 could consider only the most representative ones.

Figure 2: 3GPP bands that could be considered for adjacent channel coexistence with MSS NTN

As showed on Figure 2, some of the networks neighbor with MSS band are FDD and some of them TDD.

Moreover, taking into account potential coexistence scenarios in S-band, the previous table (with the aggressor and victim combination list) should be further revised into:

Table 1. Aggressor and victim scenarios for NTN-NTN/TN co-existence

| No. | Combination | Aggressor | Victim | Comment | 5G bands potentially considered for coexistence in adjacent channels with MSS S-Band (1980-2010 MHz for UL; 2170-2200 MHz for DL) |
| --- | --- | --- | --- | --- | --- |
| 1. (i1) | TN with NTN | TN DL | NTN DL | Coexistence with FDD band. | n1, n65; b23; b4, b10, n66 |
| 2. (i2) | TN with NTN | TN UL | NTN UL | Coexistence with FDD band. | n1, n65; b23; (n2, n25); n70 |
| 3. (i3) | TN with NTN | NTN DL | TN DL | Coexistence with FDD band. | n1, n65; b23; b4, b10, n66, see note |
| 4. (i4) | TN with NTN | NTN UL | TN UL | Coexistence with FDD band. | n1, n65; b23; (n2, n25); n70, see note Remark: This scenario should be deprioritized since NTN-TN UL coexistence in adjacent bands is similar to already existent TN-TN UL coexistence in adjacent bands |
| 5. (i5) | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. | n34, (n39) |
| 6. (i6) | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. | n34, (n39) |
| 7. | NTN with NTN | NTN DL | NTN DL | LEO-LEO or GEO-GEO or GEO-LEO600 or HAPS-HAPS | NTN MSS S-Band 5MHz with adjacent NTN MSS S-Band 5 MHz |
| 8. | NTN with NTN | NTN UL | NTN UL | LEO-LEO or GEO-GEO or GEO-LEO600 or HAPS-HAPS | NTN MSS S-Band 5MHz with adjacent NTN MSS S-Band 5 MHz |

Note: n1 and S-Band is a family of n65.

Figure 3 shows S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode (scenarios that have also been included in Table 1), where interference types i1, i2, i3 and i4 from Table 1 are represented:

Figure 3: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

We should also note that interference type i4 (NTN UL – TN UL) should not be considered for coexistence scenarios in adjacent bands since is very similar to current TN UL - TN UL coexistence.

Proposal 1: RAN4 shall remove from S-band coexistence scenarios the combination TN-NTN with FDD NTN UL to FDD TN UL.

Moreover, Figure 4 is showing S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (not all scenarios are considered because UL TN TDD is located far away from DL NTN FDD), where interference types i5 and i6 from Table 1 are represented:

Figure 4: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

On top of this RAN4 has also considered 2 additional coexistence scenarios for HAPS in adjacent bands, and a different set of combinations as expressed below (as per RAN4#98e decision):

Table 2. TN-NTN coexistence scenarios in adjacent bands

| No. | Frq. | TN | TN scenario | NTN |
| --- | --- | --- | --- | --- |
| 1 | 2GHz | NR | Rural | GEO |
| 2 | 2GHz | NR | Rural | LEO 600km |
| 3 | 2GHz | NR | Rural | LEO 1200km |
| 4 | 2GHz | NR | Urban macro | GEO |
| 5 | 2GHz | NR | Urban macro | LEO 600km |
| 6 | 2GHz | NR | Urban macro | LEO 1200km |
| 7 | 2GHz | NR | Rural | HAPS |
| 8 | 2GHz | NR | Urban macro | HAPS |

Table 3. NTN-NTN coexistence scenarios in adjacent bands

| No. | Frq. | NTN | NTN |
| --- | --- | --- | --- |
| 1 | 2GHz | GEO | GEO |
| 2 | 2GHz | GEO | LEO 600km |
| 3 | 2GHz | LEO 600km | LEO 600km |
| 4 | 2GHz | LEO 1200km | LEO 1200km |
| 5 | 2GHz | HAPS | HAPS |

RAN4 should further considering down-scoping coexistence studies from these scenarios. If we gather all the previous information into a single table, we will find that at least for FR1 S-band we would have:

| Combination | Aggressor | Victim | Number of scenarios |
| --- | --- | --- | --- |
| TN with NTN | TN DL | NTN DL | At least 8 |
| TN with NTN | TN UL | NTN UL | At least 8 |
| TN with NTN | NTN DL | TN DL | At least 8 |
| TN with NTN | NTN UL | TN UL | At least 8 |
| TN with NTN | NTN UL | TN DL | At least 8 |
| TN with NTN | TN DL | NTN UL | At least 8 |
| NTN with NTN | NTN DL | NTN DL | At least 5 |
| NTN with NTN | NTN UL | NTN UL | At least 5 |
| Total number of scenarios FR1 S-band |  |  | Around 58 |

Observation 1: For S-band there are currently around 58 scenarios to be considered for simulations required for coexistence studies in adjacent bands, and 50 if we do not consider NTN UL (aggressor) to TN UL (victim) with 8 potential combinations.

Proposal 2: RAN4 should further considering down-scoping coexistence studies from existent proposed NTN-TN and/or NTN-NTN coexistence in adjacent bands.

Proposal 3: Coexistence simulations in adjacent bands should consider a dedicated TR (similar to e.g. TR 38.803).

Moreover, NTN-NTN coexistence scenarios are largely well documented by ITU-R radio regulations.  Furthermore, the following considerations should be taken into account:

- NTN-NTN coexistence scenarios involve almost exclusively co-channel coexistence;

- Coexistence is guaranteed by spatial separation. Operators occupy the same or partly overlapping frequency ranges, but different orbital slots or orbital trajectories. There are different co-existence mechanisms between GSO and NGSO, but the overarching principle remains the same.

For these above-mentioned reasons, it does not make much sense to prioritize NTN-NTN adjacent channel coexistence. RAN4 should prioritize NTN-TN adjacent channel coexistence for S-band and/or deprioritize NTN-NTN adjacent channel coexistence for S-band.

Proposal 4: RAN4 should prioritize NTN-TN adjacent channel coexistence for S-band.

Moreover, coexistence with FDD TN scenarios in S-band for both LEO@600km and GEO should also be further prioritized.

Proposal 5: RAN4 should prioritize FDD TN adjacent channel coexistence for S-band.

Proposal 6: RAN4 should prioritize LEO@600km and GEO scenarios.

Observation 2: If prioritization decisions are made, this may further help to further reduce the number of adjacent channel coexistence scenarios for S-band to a lower number, as represented below:

| Combination | Aggressor | Victim | Number of scenarios |
| --- | --- | --- | --- |
| TN with NTN | TN DL | NTN DL | At least 6 |
| TN with NTN | TN UL | NTN UL | At least 6 |
| TN with NTN | NTN DL | TN DL | At least 6 |
| Total number of scenarios FR1 S-band |  |  | Around 18 |

Observation 3: Finally, if HAPS is excluded from Satellite coexistence scenarios analysis (since not a satellite), than for satellite coexistence cases only 12 scenarios will be relevant, which is a reasonable assumption from the simulations point of view.

| Combination | Aggressor | Victim | Number of scenarios |
| --- | --- | --- | --- |
| TN with NTN | TN DL | NTN DL | At least 4 |
| TN with NTN | TN UL | NTN UL | At least 4 |
| TN with NTN | NTN DL | TN DL | At least 4 |
| Total number of scenarios FR1 S-band |  |  | Around 12 |

# S-band updates on simulation results

Simulation results provided in R4-2101859 (with Set-1 of satellite parameters for S-band with LEO@600km) have been further updated with CDF values in DL and UL.

Figure 5: SNR CDF in DL per Cell

The intermediate simulation results show for DL that (depending on the cell position), 90% of the users are experiencing SNR values above 15 dB (please see Figure 5).

Figure 6:  SNR CDF in UL per Cell

The intermediate simulation results show for UL that (depending on the cell position), 90% of the users are experiencing SNR values above -6 dBs (please see Figure 6).

Proposal 7: RAN4 should use CDF to determine SNR values experienced by most of the users.

On the other hand, the signal strength received from LEO@600km is higher than in the case of LEO@1200km. From the point of view of the coexistence scenarios, LEO@600km therefore represents an extreme case (with higher induced interference to TN, but also higher received interference from TN). RAN4 should further prioritize worst cases for coexistence scenarios analysis, and for this reason, and with respect to the results obtained in the Figures 5 and 6 (LEO@600km, Set-1), we would like to recall again Proposal 6:

Proposal 6: RAN4 should prioritize LEO@600km and GEO scenarios.

LEO@1200km scenario does not present any material difference from LEO@600km, and it can be further de-scoped, and therefore the number of considered scenarios can be further reduced accordingly.

# Conclusions

Proposal 1: RAN4 shall remove from S-band coexistence scenarios the combination TN-NTN with FDD NTN UL to FDD TN UL.

Observation 1: For S-band there are currently around 58 scenarios to be considered for simulations required for coexistence studies in adjacent bands, and 50 if we do not consider NTN UL (aggressor) to TN UL (victim) with 8 potential combinations.

Proposal 2: RAN4 should further considering down-scoping coexistence studies from existent proposed NTN-TN and/or NTN-NTN coexistence in adjacent bands.

Proposal 3: coexistence simulations in adjacent bands should consider a dedicated TR (similar to e.g. TR 38.803).

Proposal 4: RAN4 should use CDF to determine SNR values experienced by most of the users.

Proposal 5: RAN4 should prioritize FDD TN adjacent channel coexistence for S-band.

Proposal 6: RAN4 should prioritize LEO@600km and GEO scenarios.

Proposal 7: RAN4 should use CDF to determine SNR values experienced by most of the users.

Observation 2: These decisions may further help to further reduce the number of adjacent channel coexistence scenarios for S-band to a lower number, as represented below:

| Combination | Aggressor | Victim | Number of scenarios |
| --- | --- | --- | --- |
| TN with NTN | TN DL | NTN DL | At least 6 |
| TN with NTN | TN UL | NTN UL | At least 6 |
| TN with NTN | NTN DL | TN DL | At least 6 |
| Total number of scenarios FR1 S-band |  |  | Around 18 |

Observation 3: Finally, if HAPS is excluded from Satellite coexistence scenarios analysis (since not a satellite), than for satellite coexistence cases only 12 scenarios will be relevant, which is a reasonable assumption from the simulations point of view.

| Combination | Aggressor | Victim | Number of scenarios |
| --- | --- | --- | --- |
| TN with NTN | TN DL | NTN DL | At least 4 |
| TN with NTN | TN UL | NTN UL | At least 4 |
| TN with NTN | NTN DL | TN DL | At least 4 |
| Total number of scenarios FR1 S-band |  |  | Around 12 |

END