3GPP TSG-RAN WG4 Meeting #98-bis-e                                       R4-2107308

Electronic Meeting, Apr. 12-20, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion and draft reply LS on EN-DC power class

Agenda Item:	5.8.1.2

Document for:	Approval

# Introduction

According to the RAN#88e agreement [1], new power class powerClassNRPart-r16 was introduced into RAN2 specification.

The ambiguity of EN-DC power class rooted from UE implementation architecture caused some confusion in other groups. With the progress for Rel-16, some clarification to respond the LS from RAN5 is necessary. It is noted that the solutions on EN-DC power class ambiguity for Rel-15 and Rel-16 are different, and to address the power class issue for Rel-15 spec, two options to changes the Pcmax requirement were discussed in previous RAN4 meetings.

This contribution provides further consideration on EN-DC power class, and draft LS is provided for RAN5.

# Discussion

The EN-DC power class issue is related to UE implementation.

It is understandable that for a PC2 UE with 2x23dBm implementation, considering the 1Tx from E-UTRA side, only 1Tx 23dBm could be available for the NR band in EN-DC band combination. Thus the power class reported per NR band may not reflect the UE capability accurately.

Rel-15 solution: clarification in TS 38.101-3

Unless otherwise stated, if UE indicates IE maxNumberSRS-Ports-PerResource = n2 in NR standalone operation mode,  the said UE shall meet the NR requirements for either power class 2 or power class 3 in EN-DC within FR1 if UE indicates IE maxNumberSRS-Ports-PerResource = n1 for EN-DC on this NR band.

Rel-16 solution: introduce new signaling

New signaling was introduced in RAN2 specification.

BandCombination-v1610 ::=          SEQUENCE {

bandList-v1610                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610  OPTIONAL,

ca-ParametersNR-v1610               CA-ParametersNR-v1610                  OPTIONAL,

ca-ParametersNRDC-v1610             CA-ParametersNRDC-v1610                OPTIONAL,

powerClass-v1610                    ENUMERATED {pc1dot5}                   OPTIONAL,

powerClassNRPart-r16                ENUMERATED {pc1, pc2, pc3, pc5}        OPTIONAL,

featureSetCombinationDAPS-r16       FeatureSetCombinationId                OPTIONAL,

mrdc-Parameters-v1620               MRDC-Parameters-v1620                  OPTIONAL

}

From implementation point of view, there is no big difference for a Rel-16 UE or a Rel-15 UE for this specific case.

To enable the Rel-15 in the specification, it still needs some changes on Pcmax requirement, which was already mentioned in the RAN5 LS, i.e. how to use PCMAX_L,f,c,,NR for a Rel-15.

Two alternatives were discussed in last meeting [2][3].

Alt 1: set the low bound of Pcmax based on the lower power class could be supported by UE implementation, and no UE declaration is needed

-	∆PPowerClass,NR = 3 dB for a power class 2 capable EN-DC UE in PCMAX_L,f,c,,NR if PPowerClass,NR is indicated as power class 2 or power class 1.5; otherwise ΔPPowerClass,NR = 0 dB;

Alt 2: determine the low bound of Pcmax based on UE implementation declaration

-	∆PPowerClass,NR = 3 dB if the UE indicates power class 2 by UE-NR-Capability but only complies with power class 3 for the NR part of the EN-DC band combination configured; otherwise ∆PPowerClass,NR = 0 dB;

As there is no corresponding capability defined as that in Rel-16, it’s always ambiguous for the applied requirement. The main issue is that UE may fail the test without appropriate requirements rather than the PHR reporting. Whether the UE can configure the Pcmax properly based on declaration by the UE is still ambiguous for the network, which results the value unpredictable by the network.

It is also noticed that PC1.5 is also release independent from Rel-15, for which 26+26dBm implementation is a common assumption. In other words, in EN-DC mode, it can only support a lower power class.

To address the test issue, if it’s not possible to indicate the power class clearly, the better way in our view is to use the lower possible power class to decide PCMAX_L,f,c,,NR . Therefore, our preference is Alt 1.

Observation 1: Without a power class to indicate the difference between SA and NSA for the NR band, it’s ambiguous which power class would be used for PCMAX_L,f,c,,NR , consequently, UE may fail the Pcmax test for the NR part in an EN-DC band combination.

Observation 2: The main issue of Pcmax identified by RAN5 is to address the measurement problem.

Proposal: It is proposed to adopt the method to set a lower bound for PCMAX_L,f,c,,NR if PPowerClass,NR is indicated as a higher power class rather than the default power class.

Draft reply LS is provided as below for approval.

# Draft Reply LS to and RAN5

Title:	Draft Reply LS on requirement for Power Class 2 EN-DC

Response to:	R5-202805

Release:	Rel-15

Work Item:	NR_newRAT-core

Source:	RAN4

To:	RAN5

Cc:

Contact Person:

Name:

E-mail Address:

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN5 for the LS on requirements related to Power Class 2 EN-DC. For the difference of power class handling between Rel-15 and Rel-16 specifications, RAN4 would like to provide some background information.

In Rel-16, unlike Rel-15 using clarification in the specification to allow UE to meet either PC2 or PC3 requirements, new power class was introduced for the NR part (single NR band or intra-band NR CA) of the MR-DC band combination in addition to the indicated power class for the MR-DC band combination.

Though the solutions of handling Rel-15 and Rel-16 EN-DC power class issue are different, RAN4 sees no difference from UE implemenation perspective for these two releases.

Regarding the questions raised by RAN5, the following clarifications are provided:

- Clarify the definitions of PPowerClass and PPowerClass, EN-DC, and if these parameters are identical to the UE signalled power class for NR and EN-DC respectively

Answer: In RAN4 specification, PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination, which could be different from the power class for NR band reported by the UE for the SA mode, but no specific power class is defined to distinguish the difference in Rel-15. While PPowerClass, EN-DC is the nominal UE power of the power class that the UE supports for the EN-DC combination.

- How to evaluate PCMAX_L,f,c,,NR in the scenario indicated in the Overall Description for Rel-15.

Answer: Since the UE capability of powerClassNRPart-r16 is not introduced in Rel-15, it cannot indicate PCMAX_L,f,c,,NR clearly if the power class is different for the NR band between NR SA and EN-DC combination. Two solve the issue, ∆PPowerClass,NR can be utilized to reflect the power class difference for the NR part in different cases, i.e. ∆PPowerClass,NR can be set to 3 dB for a power class 2 capable EN-DC UE in PCMAX_L,f,c,,NR if PPowerClass,NR is indicated as a higher power class than the default power class,  otherwise ΔPPowerClass,NR = 0 dB.

2. Actions:

To GCF-CAG and RAN5:

ACTION: RAN4 respectfully asks RAN5 to the above information into consideration on requirement for Power Class 2 EN-DC in Rel-15 specification.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#99-e                    May 2020	                    Online

# References

[1] RP-201392, draft LS on introducing new power class for NR band in MR-DC, Huawei, HiSilicon

[2] R4-2016482, CR for TS 38.101-3: correction of power class for EN-DC, Huawei, HiSilicon

[3] R4-2015977, Correction of Pcmax for an NR PC2 UE supporting NR PC3 for EN-DC, Ericsson