3GPP TSG-RAN WG4 Meeting #98-bis-e                                       R4-2107312 Electronic Meeting, Apr. 12-20, 2021

Source: 	Huawei, HiSilicon

Title: 	On RMR 900MHz band

Agenda Item:	7.43.1

Document for:	Approval

# Introduction

A new WI on introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio was approved in RAN#91e meeting [1].

This contribution provides our consideration on RMR 900MHz band.

# Discussion

## WI objectives

The objectives of the WI are copied as below for reference.

- Specify a new NR FDD operating band to include band numbering and core requirements to operate in Europe by considering that:

  - UE transmit: 874.4 - 880 MHz, BS transmit: 919.4 MHz – 925 MHz

  - The use of UE power class 3 (23dBm);

  - To support 5 MHz channel bandwidth;

  - To support 15kHz and 30kHz SCS;

  - To support Wide Area Base Stations;

  - To support only non-AAS Base Stations;

  - The Block-Edge-Mask (BEM) is developed on the basis that detailed coordination and cooperation agreements would not be required to be in place prior to network deployment.

During the RAN discussion, operators commented that negative impacts to existing deployed equipment in bands (n)8 and (n)1 shall be avoided. Since the RMR bands are new operating bands, and (n)8 and (n)1 are legacy bands, which have incumbent services in the network, therefore, we agree with that the deployed network should not be affected.

Observation 1: Some description in the WID are deployment related, it should be clear that the newly introduced band shall not have impact to the incumbent deployed network.

## Some issues to be discussed

The adjacent bands around RMR 900MHz are illustrated in Figure 1.

Figure 1: Operating bands around RMR 900MHz

RMR 900M is a FDD band, as the RMR DL spectrum is close to B8/n8 UL spectrum, more attention should be paid to guarantee the RMR system shall not have impact to the deployed B8/n8 network.

Some general consideration on RMR bands have been given in [2]. Based on the similar WID objectives, we have similar observations as well.

Observation 2: Requirement of BEM in the WID objective needs to be clarified.

Observation 3: It’s not clear whether some regional regulatory requirements need to be considered to define some UE band specific requirements.

Apart from the observations according to the WID, we also noticed that there are some technical conditions for GSM-R in 874.4-880.0 MHz / 919.4-925.0 MHz.

For GSM-R, the following parameters apply: GSM-R DL centre frequency fDL = 921 MHz + n×0.2 MHz where n∈Z -7≤n≤19 GSM-R UL centre frequency fUL = fDL – 45 MHz GSM-R channel bandwidth is 200 kHz

The centre frequency of GSM-R is 200kHz based, which may have impact to the selection of channel raster for RMR 900MHz band.

Observation 4: Selection of channel raster of RMR 900MHz band should consider the technical conditions in in ECC Decision (20)02.

# Conclusion

This contribution provides some consideration for RMR 900MHz band.

Observation 1: Some description in the WID are deployment related, it should be clear that the newly introduced band shall not have impact to the incumbent deployed network.

Observation 2: Requirement of BEM in the WID objective needs to be clarified.

Observation 3: It’s not clear whether some regional regulatory requirements need to be considered to define some UE band specific requirements.

Observation 4: Selection of channel raster of RMR 900MHz band should consider the technical conditions in in ECC Decision (20)02.

Proposal 1: It is proposed to have some clarification of the WID objectives and make clear of the aspects to be studied in both BS and UE sides.

Proposal 2: Make clear that the newly introduced RMR bands shall not have impact to the deployed network.

# References

[1] RP-210878, New WID on introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio, UIC

[2] R4-2107313, On RMR 1900MHz band, Huawei, HiSilicon