3GPP TSG-RAN WG4 Meeting #99-e	R4-210xxxx

Electronic Meeting, 19 - 27 May, 2021

Title:	[Draft] LS on UL MIMO coherence for Tx switching between two carriers

Release:	Release 16

Work Item:	NR_RF_FR1-Core

Source:	RAN4

To:	RAN2, RAN1

CC:

Contact person:	YANG Shan

yangshan@chinatelecom.cn

Attachments:	none

- 1 Overall description

For Tx switching between uplink carriers for inter-band UL CA, SUL and EN-DC, RAN4 reached the following agreements w.r.t. UL MIMO coherence:

- Introduce UE capability to indicate whether or not the UL MIMO coherence can be maintained for the carrier capable of two antenna connectors, when UE is configured with uplink switching with parameter uplinkTxSwitching-r16 and uplink switching is triggered by the switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 between last transmitted SRS and scheduled transmission.

- UE capability is defined as per band combination.

- If the above capability is absent, the existing per band UE capability pusch-TransCoherence is applicable to the scenario when UE is configured with uplink switching with parameter uplinkTxSwitching-r16 and uplink switching is triggered by the switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 between last transmitted SRS and scheduled transmission.

- 2 Actions

To 3GPP RAN2

ACTION: RAN4 respectfully asks RAN2 to take into consideration above RAN4 agreements on UL MIMO coherence related UE capability to introduce UE capability signalling.

To 3GPP RAN1

ACTION: 	RAN4 respectfully asks RAN1 to check if there is any RAN1 specification impact due to the above RAN4 agreements.

- 3	Dates of next TSG RAN WG4 meetings

3GPP RAN4 #100e 		16 – 27 Aug 2021		        	Electronic Meeting

3GPP RAN4 #101e		01 – 12 Nov 2021			Electronic Meeting