3GPP TSG-RAN WG4 #79AH	R4-79AH-0052

Hong Kong, China, 28 - 30 June, 2016

Source: 	Ericsson

Title:	Simulation assumption for LAA PDCCH and ePDCCH

Agenda Item:	3.2.3

Document for:	Approval

# Introduction

In this paper, simulation assumption for LAA PDCCH and ePDCCH  is provided.

# Common parameters for Scell

Table 1 provides all common parameters for Scell. Without special mention, the parameters in Table 1 will be applied for all the following sections.

Table 1: Common parameters for Scell

| Parameters | unit | Value |
| --- | --- | --- |
| MBSFN configuration |  | No MBSFN subframe is configured |
| DRS periodicity | ms | 80 |
| Frequency offset related to primary carrier | Hz | 750 |
| Time offset related to primary carrier | us | 30.26 |
| Bandwidth | MHz | 20 |
| Number of allocated resource blocks (Note 2) | PRB | 100 |
| Number of control OFDM symbols |  | 2 |
| Signal transmission model |  | Randomness model is used as baseline with the exception: The burst length is {1} subframe |

# Simulation assumption for (e)PDCCH

## PDCCH

The parameters for Scell in PDCCH test is given in Table 2.

Table 2: The parameters for PDCCH (Scells)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Propagation condition |  |  | EVA70 |
| Correlation matrix |  |  | 2x2 Low |
| Number of CCE |  |  | 4 |
| Format 2A |  | bits | 48 (not included CRC) |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

## ePDCCH

The parameters for ePDCCH are reused from Table 8.8.2.1-1 with minor modification for Scell.

Table 3: Test Parameters for Localized EPDCCH with TM9 (recap from Table 8.8.2.1-1 with minor

Modification)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 1 (Note 1) |
| EPDCCH starting symbol |  | symbols | 2 (Note 1) |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 1 |
| Beamforming Pre-Coder |  |  | Annex B.4.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| CSI-RS Reference Signal |  |  | Port 15 and 16 |
| CSI-RS reference signal resource configuration |  |  | 0 |
| CSI reference signal subframe configuration ICSI-RS |  |  | 2 |
| ZP-CSI-RS configuration bitmap |  |  | 0000010000000000 |
| ZP-CSI-RS subframe configuration IZP-CSI-RS |  |  | 2 |
| Number of EPDCCH Sets |  |  | 2 (Note 2) |
| EPDCCH Subframe Monitoring pattern subframePatternConfig-r11 |  |  | 1111111110 1111111101 1111111011 1111110111 (Note 3) |
| Number of CCE |  | CCE | 2 |
| Propagation Condition |  |  | EVA5 |
| Antenna configuration and correlation Matrix |  |  | 2 x 2 Low |
| DCI Format |  |  | DCI Format 2C |
| Number of DCI bits without CRC |  |  | 50 |
| PDSCH TM |  |  | TM9 |
| Note 1: The starting symbol for EPDCCH is signalled with epdcch-StartSymbol-r11. However, CFI is set to 1. Note 2: The first set is distributed transmission with PRB = {0, 49} and the second set is localized transmission with PRB = {0, 7, 14, 21, 28, 35, 42, 49}. ePDCCH is scheduled in the second set for all tests. Note 3: EPDCCH is scheduled in every SF. UE is required to monitor ePDCCH for UE-specific search space only in SFs configured by subframePatternConfig-r11. Legacy PDCCH is not scheduled. |  |  |  |

# Conclusion

In this paper, simulation parameters are provided for PDCCH and ePDCCH. We hope the group approves the above parameters for simulation results alignment.