3GPP TSG-RAN WG4 Meeting #79AH	R4-79AH-0083

Hong Kong, China, 28 - 30 June, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | Draft CR | rev |  | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: NB-IoT NPUSCH Format 2 Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-06-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | NB-IoT BS performance requirements for NPUSCH Format 2 are not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define NB-IoT BS performance requirements for NPUSCH Format 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No NB-IoT BS performance requirements for NPUSCH Format 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections 8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.141 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

## 8.5.2	Requirements for NPUSCH Format 2

### 8.5.2.1	DTX to ACK performance requirements

The DTX to ACK requirement is valid for any number of receive antennas, for all frame structures and for any channel bandwidth. The DTX to ACK probability denotes the probability that ACK is detected when nothing is sent on the wanted signal and the interfering signals are present.

#### 8.5.2.1.1	Minimum requirement

The DTX to ACK/NACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1% per NPUSCH Format 2 transmission. A NPUSCH transmission may take multiple subframes due to NPUSCH transmission repetition. The performance measure is defined as follows:

ProbNPUSCH DTX→ACK bits=#(false ACK bits)#(NPUSCH DTX transmissions)≤10-2

where:

●	#(false ACK bits) denotes the number of detected ACK bits for all NPUSCH DTX transmissions.

●	#(NPUSCH DTX transmissions) denotes the total number of NPUSCH DTX transmissions

### 8.5.2.2	ACK/NACK false detection requirements

The ACK false detection probability is the probability of not detecting an ACK when an ACK was sent.  The NACK false detection probability is the probability of not detecting an NACK when a NACK was sent.

#### 8.5.2.2.1	Minimum requirements

Both ACK false detection probability and  NACK false detection probability shall not exceed 1% at the SNR given in table 8.5.2.2.1-1.

Table 8.5.2.2.1-1 Minimum requirements for NPUSCH format 2

| Number of TX antenna | Number of RX antennas | Channel Bandwidth | Singe-/Multi-tone | Sub-carrier spacing | Modulation | repetition level for each NPUSCH transmission | Propagation conditions and correlation matrix (Annex B) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 180KHz | Single | 3.75KHz | BPSK | 1 | [EPA5 or ETU1] | TBD |
|  |  |  |  |  |  | 4 | [EPA5 or ETU1] | TBD |
|  |  |  |  |  |  | 16 | [EPA5 or ETU1] | TBD |
| 1 | 2 | 180KHz | Single | 15KHz | BPSK | 1 | [EPA5 or ETU1] | TBD |
|  |  |  |  |  |  | 4 | [EPA5 or ETU1] | TBD |
|  |  |  |  |  |  | 16 | [EPA5 or ETU1] | TBD |

---END OF CHANGE 1---